KERMIT-65 USERS GUIDE

C64 VERSION 1.0

BETA TEST

COMPILED BY:

ERIC LAVITSKY

17 DECEMBER 1984

CENTER FOR COMPUTER AND INFORMATION SERVICES

RUTGERS UNIVERSITY

PISCATAWAY, NEW JERSEY

Introduction                                                             Page 1

1. Introduction

1.1. Overview

KERMIT-65  is  a  program that implements the KERMIT file transfer protocol for

the Commodore 64 micro computer system. It is written in 6502 assembly language

and  should  run  on  any C64. KERMIT-65 also Converts your C64 into a powerful

computer terminal. It emulates the popular Digital Equipment  Corporation  VT52

terminal or a simple glass TTY.

C64  Kermit-65  was  translated by Dave Dermott from the original Apple version

1.0 by Antonio J. Mione. Several enhancements and bug fixes including many from

Apple version 2.1 were added by Eric Lavitsky.

C-64 Kermit-65 Capabilities At A Glance:

Local operation:                   Yes

Remote operation:                  No

Transfers text files:              Yes

Transfers binary files:            No?

Wildcard send:                     No

^X/^Y interruption:                No

Filename collision avoidance:      No

Can time out:                      No

8th-bit prefixing:                 No?

Repeat count prefixing:            No

Alternate block checks:            No

Terminal emulation:                Yes (VT52)

Communication settings:            Yes; local echo, parity

Transmit BREAK:                    Yes

IBM communication:                 Yes?

Transaction logging:               No

Session logging (raw download):    No

Raw upload:                        No

Act as server:                     No

Talk to server:                    Yes

Advanced commands for servers:     No

Local file management:             Yes

Handle file attributes:            No

Command/init files:                No

Printer control:                   No

1.2. System Requirements

What you will need:

- C64 and Disk drive

- Modem - any will do (300/1200, VIC, RS232)

- Supermon64.V1 (or any equivalent relocatable monitor)

Known Bugs, Deficiencies                                                 Page 2

2. Known Bugs, Deficiencies

KERMIT-65  is  currently  missing some features which exist in Apple KERMIT-65.

There are also some restrictions which apply.

The differences and restrictions are as follows:

- IBM mode is not supported.

- The FILE-WARNING option is not implemented. If a file already exists,

a fatal error occurs.

- The SET PARITY option is not implemented. Parity must be set with SET

RS232-REGISTERS.

2.1. HINTS

If you wish to edit BASIC programs on the mainframe system, you should  convert

them to text files on the C64 before you send them since the BASIC interpreters

store keywords as single character tokens. If the  receiving  system  does  not

handle  eight-bit ASCII or CBM ASCII characters well, KERMIT-65 should have its

FILE-BYTE-SIZE set to SEVEN-BIT. It will send standard seven-bit ASCII  to  the

receiving  KERMIT.  When  KERMIT-65  is receiving from such a system, this will

pick up the seven-bit characters and write out CBM ASCII on the disk file.

Running KERMIT                                                           Page 3

3. Running KERMIT

3.1. Booting KERMIT

Insert your working KERMIT-65 disk into the 1541 drive label side up.  Type the

following:

LOAD"KERMIT",8 <cr>

RUN <cr>

The screen will come up with the following message:

C64 Kermit-65 V1.1 40 Column Mode

Kermit-65>

You  are now in the KERMIT-65 parser which is modeled after the COMND parser of

the TOPS-20 Operating System from Digital Equipment Corporation.  You  may  now

type commands to the parser to set options or begin actions.

During  interactive  operation,  you may use the ?-prompting help feature ("?")

and recognition (ESC) features while typing commands.  A question mark typed at

any  point in a command displays the options available at that point; typing an

ESC character causes the current keyword to be completed (or default  value  to

be  supplied).  If you have not typed sufficient characters to uniquely specify

the keyword or filename (or if there is no default value) then a beep  will  be

sounded  and you may continue typing. Keywords may be abbreviated to any prefix

that is unique.

3.2. Commands in KERMIT

Commands available in version 1.1 of Kermit-65 are:

BYE         CONNECT     DOS         EXIT        FINISH      GET

QUIT        RECEIVE     SEND        SET         SHOW        STATUS

BYE

This  command  will  cause Kermit-65 to tell a server Kermit at

the other end of the connection to exit from Kermit and, if ap-

plicable, terminate its job (or process, etc.).  When Kermit-65

receives the acknowledgement that this is being done,  it  will

exit to DOS.

KERMIT-65>BYE

CONNECT

The CONNECT command will allow you to connect to a  remote  job

over the terminal line as a virtual terminal. The format of the

CONNECT command is:

KERMIT-65>CONNECT

Running KERMIT                                                           Page 4

DOS

The  DOS  command  will  invoke  a separate parser to allow the

sending of commands to the disk drive. The format is:

KERMIT-65>DOS

Commands are in standard DOS format as  decribed  in  the  disk

manual.  They include $ for directory (note block counts are in

hex!), S to scratch files. A blank line (RETURN)  will  display

the disk error status if any. Enter Q to return to KERMIT.  eg:

DOS>$KER*     -- directory of ker* files

DOS>S0:a*     -- delete files A*

DOS>Q  -- returns to KERMIT

EXIT

The EXIT command will cause Kermit to return to  BASIC  command

level. This command is the same as the QUIT command. An example

of this command is:

KERMIT-65>EXIT

FINISH

This  command  will  cause Kermit-65 to tell a server Kermit at

the other end of the connection to  exit  from  Kermit.    When

Kermit-65 receives the acknowledgement that this is being done,

Kermit-65 will prompt for another command.

KERMIT-65>FINISH

GET

This  command  can  only be used in 'user' mode.  It will cause

Kermit-65 to request that the other Kermit (which must be  run-

ning in server mode) transmit the specifed file to Kermit-65.

KERMIT-65>GET file-specification

Where "file-specification" is a valid  file  specification  for

the system on which the other Kermit is running.

QUIT

This command will cause Kermit-65 to return  to  BASIC  command

level. This is the same as the EXIT command.

KERMIT-65>QUIT

RECEIVE

The RECEIVE command is used to put Kermit-65 into  remote  mode

waiting  for  a  single  file transfer transaction.  If no file

specification is given, Kermit-65 will wait for a file transfer

initialization  sequence  from  the  other  Kermit.   If a file

Running KERMIT                                                           Page 5

specification is given, that file specification will be used to

store the received file(s), regardless of what name is supplied

by the other Kermit.

KERMIT-65>RECEIVE

or

KERMIT-65>RECEIVE file-specification

Where "file-specification" is any valid CBM DOS file specifica-

tion.

SEND

The  SEND  command  will  allow you to send a file to the other

Kermit.  The command format is:

KERMIT-65>SEND file-specification

Where   "file-specification"   is   any  normal  CBM  DOS  file

specification.

SET

The  SET  command  is used to set various parameters in Kermit.

The format of the command is:

KERMIT-65>SET <option> <suboption> ...

The options available to SET are:

DEBUGGING           EIGHT-BIT-QUOTING   ESCAPE

FILE-BYTE-SIZE      FILE-TYPE           FILE-WARNING

IBM                 LOCAL-ECHO          PARITY

RECEIVE             RS232-REGISTERS     SEND

VT52-EMULATION

A description of the options and suboptions follows.

SET DEBUGGING

The SET DEBUGGING command is used to set the debug type out  on

the user's terminal.

KERMIT-65>SET DEBUGGING keyword

Where keyword is either OFF, TERSE, or VERBOSE.  OFF causes all

debugging  output  to  be  suppressed.  TERSE directs a copy of

each packet to the screen printing any  control  characters  as

'^char'.  VERBOSE prints identifying text for each field in the

Running KERMIT                                                           Page 6

header and prints the actual data section of the packet in  the

same manner TERSE uses.

SET EIGHT-BIT-QUOTING

This command will turn on or  turn  off  the  eight-bit-quoting

feature.      If   set   ON,   it   forces   Kermit-65  to  use

eight-bit-quoting which may be handy if it must receive a  text

file  in which some or all of the characters are not in CBM as-

cii.  The command takes ON or OFF as options.

SET ESCAPE

This  command  will  set  the  escape character for the CONNECT

processing.  The command will take the hex value of the charac-

ter  to  use  as  the  escape character.  This is the character

which is used to "escape" back to  KERMIT-65  after  using  the

CONNECT  command.  It  defaults to ^ (hex 00).  It is usually a

good idea to set this character to something which is not  used

(or  at  least  not  used  very  much)  on  the system to which

KERMIT-65 is CONNECTing.

KERMIT-65>SET ESCAPE hex-character-value

Where  hex-character-value  is the ASCII value of the character

to use as the escape character (in hex).

SET FILE-BYTE-SIZE

This  command  sets  the byte size KERMIT-65 will use for files

being received or transmitted.  SEVEN-BIT bytes should be  used

for  ASCII text files which are to be used as text files on the

CBM DOS system. EIGHT-BIT bytes should be used for binary files

being transferred to and from the C64.  This allows the file to

be returned without any changes.

KERMIT-65>SET FILE-BYTE-SIZE size

Where size is SEVEN-BIT or EIGHT-BIT.

SET FILE-TYPE   This command tells Kermit-65 what type of file is being sent or

received.  It is important that this parameter is set correctly

since  it is used to determine the exact file length when send-

ing files in addition to determining special  processing  which

may  have  to  be done (i.e.  When receiving a text file, write

all characters in CBM ASCII.). The format of this command is:

KERMIT-65>SET FILE-TYPE type

Where  type  is  ASCII,  BINARY, PETASCI or SCRIPT. BINARY is a

core image (and is normally transferred in eight-bit mode). AS-

CII  is  any  file  of ASCII text. PETASCI is a CBM ASCII file.

Running KERMIT                                                           Page 7

SCRIPT is a file produced  by  the  SPEEDSCRIPT  wordprocessor,

which was published in the Gazzette! Magazine.

SET FILE-WARNING

This command determines what KERMIT-65 will do when it receives

a  file  with a file name that already exists.  If FILE-WARNING

is turned ON, KERMIT-65 will not supersede the old file.    In-

stead  it will create a file with the original file name plus a

'.0' appended to it.  It will increment  the  'version'  number

until  it  succeeds  in  creating  a  unique  file  name.    If

FILE-WARNING is turned OFF (the default), Kermit-65 will super-

sede the old file.

KERMIT-65>SET FILE-WARNING keyword

Where keyword is either ON or OFF. (This option does  not  work

in this version of Kermit).

SET IBM

The SET IBM command allows KERMIT-65 to  be  put  into  a  mode

which  will  allow  transfers  to  an  IBM  host.   This causes

KERMIT-65 to  wait  for  the  IBM  turnaround  character  (XON,

CTRL-Q)  before sending any characters to the other Kermit.  It

also forces the parity type to be mark, and turns on local echo

for the CONNECT command.

KERMIT-65>SET IBM keyword

Where keyword is either ON or OFF.

SET LOCAL-ECHO   The SET LOCAL-ECHO command specifies whether characters should

be  echoed  locally  when  CONNECTing  to  another  system.  If

LOCAL-ECHO is set to ON, any character typed  on  the  terminal

will  be  echoed  immediately to the terminal, as well as being

sent to the other system.  If LOCAL-ECHO is  set  to  OFF  (the

default), the characters typed on the terminal are only sent to

the  other  system  (which  would  normally  be   echoing   the

characters).

KERMIT-65>SET LOCAL-ECHO keyword

Where keyword is either ON or OFF.

SET PARITY

This command determines the type of parity to use on the trans-

mission line.  Kermit normally uses characters which consist of

eight  data  bits with no parity bit.  For systems or transmis-

sion media which require a specific  parity  type,  Kermit  can

send characters as seven data bits plus a parity bit.

Running KERMIT                                                           Page 8

KERMIT-65>SET PARITY keyword

Where keyword is one of:

1. NONE (default) - eight data bits and no parity bit.

2. MARK - seven data bits with the parity  bit  set  to

one.

3. SPACE  -  seven data bits with the parity bit set to

zero.

4. EVEN - seven data bits with the parity  bit  set  to

make the overall parity even.

5. ODD  -  seven  data  bits with the parity bit set to

make the overall parity odd.

Note that this command currently does not actually set the parity.    You  must

use  SET  RS232-REGISTERS to manually select communication set-

tings.

SET RECEIVE

It  is  possible  to set various parameters associated with the

receiving of the data from the remote Kermit.  SET RECEIVE will

enable you to set the various receive parameters.

The suboptions of this SET command are:

EIGHT-BIT-QUOTE-CHAR        END-OF-LINE   PACKET-LENGTH

PAD-CHAR                    PADDING       QUOTE-CHAR

TIMEOUT

SET RECEIVE EIGHT-BIT-QUOTE-CHAR

This  command sets the character to be used (when necessary) to

quote characters which have the eighth bit  (parity  bit)  set.

This  is  used  to  transfer  eight-bit bytes on a transmission

medium which only supports seven data bits.  The default  value

is 26 (ASCII "&").  Eight-bit-quoting will only be used if both

Kermit's can handle it, and the  tranmission  medium  does  not

transmit 8 data bits (as indicated by the SET PARITY command).

KERMIT-65>SET RECEIVE EIGHT-BIT-QUOTE-CHAR nn

Where nn is the ASCII value of the character (in  hex)  to  use

for quoting characters which have the eighth bit set.

SET RECEIVE END-OF-LINE

Running KERMIT                                                           Page 9

This will set the end of line character that KERMIT-65  expects

to receive from the remote Kermit.  This is the character which

terminates a packet.   The  default  value  is  0F  (ASCII  CR,

CTRL-M).

KERMIT-65>SET RECEIVE END-OF-LINE nn

Where nn is the ASCII value of the character to use for the end

of line character (in hex).

SET RECEIVE PACKET-LENGTH

This will set the receive packet length.  The  value  for  this

parameter must be between 10 and 96.  Packet lengths outside of

this range are illegal.  The default value is 5E (94 decimal).

KERMIT-65>SET RECEIVE PACKET-LENGTH nn

SET RECEIVE PAD-CHAR

This  parameter  is the padding character that is expected from

the remote Kermit.  The parameter must be a hex number  in  the

range  of  0  to  1F or 7F.  All other values are illegal.  The

default value is 0 (an ASCII NUL).

KERMIT-65>SET RECEIVE PAD-CHAR nn

Where  nn  is  the ASCII value of the character to be used as a

pad character (in hex).

SET RECEIVE PADDING

This  command  will  set  the number of padding characters that

will be expected from the other Kermit.  The default  value  is

0.

KERMIT-65>SET RECEIVE PADDING nn

Where nn is the hex number of padding characters to use.

SET RECEIVE QUOTE-CHAR

This  will set the quoting character that KERMIT-65 will expect

on incoming messages.  This is the character used to quote con-

trol characters.  The default value is 23 (ASCII "#").

KERMIT-65>SET RECEIVE QUOTE-CHAR nn

Running KERMIT                                                          Page 10

Where nn is the ASCII value of the quoting character (in hex).

SET RECEIVE TIMEOUT

This  will  set  the number of seconds after which KERMIT-65 is

requesting a timeout from the remote Kermit. This time  out  is

used  to  handle  transmission errors which totally lose a mes-

sage.  The default value is 15 seconds.

KERMIT-65>SET RECEIVE TIMEOUT nn

Where  nn  is  the  number of seconds to wait for a message (in

hex).

SET RS232-REGISTERS

This sets the 2 registers for controlling the baud rate, parity

etc. The 2 registers are treated as a 16 bit hex  integer.  The

first  2  digits  are the SECOND register and the last 2 digits

are the FIRST register. If  only  2  digits  are  entered,  the

second register is cleared. Examples:

KERMIT-65>SET  RS232-REGISTERS  0006  (or  SET  RS 06) sets the

registers to 06,00 for 300 baud, 8 bit, no parity (this is  the

default)

KERMIT-65>SET  RS232-REGISTERS A026 sets the registers to 26,A0

for 300 baud, 7 bit, parity mark.

See the C64 reference manual for a complete description of  all

the bits in the 2 registers.

SET SEND

It is possible to set various parameters  associated  with  the

sending of data to the remote Kermit.  SET SEND will enable you

to set the various SEND parameters.   These  parameters  should

not  normally be set since, as part of the transfer initializa-

tion  process,  the  two   Kermits   exchange   their   RECEIVE

parameters.    The  capability  of  setting these parameters is

provided so that the transfer initialization can  be  completed

even if the default parameters are not correct.

The suboptions of this SET command are:

EIGHT-BIT-QUOTE-CHAR        END-OF-LINE   PACKET-LENGTH

PAD-CHAR                    PADDING       QUOTE-CHAR

TIMEOUT

SET SEND EIGHT-BIT-QUOTE-CHAR

This  command sets the character to be used (when necessary) to

Running KERMIT                                                          Page 11

quote characters which have the eighth bit  (parity  bit)  set.

This  is  used  to  transfer  eight-bit bytes on a transmission

medium which only supports seven data bits.  The default  value

is 26 (ASCII "&").  Eight-bit-quoting will only be used if both

Kermit's can handle it, and the  tranmission  medium  does  not

transmit 8 data bits (as indicated by the SET PARITY command).

KERMIT-65>SET SEND EIGHT-BIT-QUOTE-CHAR nn

Where nn is the ASCII value of the character (in  hex)  to  use

for quoting characters which have the eighth bit set.

SET SEND END-OF-LINE

This will set the end of line  character  that  KERMIT-65  will

send  to  the  remote Kermit.  This is the character which ter-

minates a packet.  The default value is 0F (ASCII CR, CTRL-M).

KERMIT-65>SET SEND END-OF-LINE nn

Where nn is the ASCII value of the character to use for the end

of line character (in hex).

SET SEND PACKET-LENGTH

This  will  set  the  send  packet  length.  The value for this

parameter must be between 10 and 96.  Packet lengths outside of

this range are illegal.  The default value is 5E (94 decimal).

KERMIT-65>SET SEND PACKET-LENGTH nn

SET SEND PAD-CHAR

This parameter is the padding character that  is  sent  to  the

remote  Kermit.    The  parameter  must be an hex number in the

range of 0 to 1F or 7F.  All other values  are  illegal.    The

default value is 0 (an ASCII NUL).

KERMIT-65>SET SEND PAD-CHAR nn

Where nn is the ASCII value of the character to be  used  as  a

pad character (in hex).

SET SEND PADDING

This command will set the number  of  padding  characters  that

will be sent to the other Kermit.  The default value is 0.

Running KERMIT                                                          Page 12

KERMIT-65>SET SEND PADDING nn

Where nn is the hex number of padding characters to use.

SET SEND QUOTE-CHAR

This will set the quoting character that KERMIT-65 will send in

outgoing messages.  This is the character used to quote control

characters.  The default value is 23 (ASCII "#").

KERMIT-65>SET SEND QUOTE-CHAR nn

Where nn is the ASCII value of the quoting character (in hex).

SET SEND TIMEOUT

Since  Kermit-65  has no clock to use, it can not timeout. This

parameter, therefore is useless.

SET VT52-EMULATION

This switch determines whether or not Kermit-65 will act like a

VT52 when it is CONNECTed to  a  remote  Kermit.  The  possible

values for the switch are ON and OFF. The default is ON.

KERMIT-65>SET VT52-EMULATION OFF

SHOW

The SHOW command will allow you to show the various  parameters

that are set with the SET command.

The options available to SHOW are:

ALL             DEBUGGING       EIGHT-BIT-QUOTING

ESCAPE          FILE-BYTE-SIZE  FILE-TYPE

FILE-WARNING    IBM             LOCAL-ECHO

PARITY          RECEIVE         RS232-REGISTERS

SEND            VT52-EMULATION

A description of the options and suboptions follows.

SHOW ALL

The  SHOW  ALL  command  will cause all of the parameters to be

listed.

SHOW DEBUGGING

Display  whether debugging is off, in terse mode, or in verbose

mode.

SHOW EIGHT-BIT-QUOTING

Running KERMIT                                                          Page 13

Shows  whether  or  not Kermit-65 is using eight-bit-quoting on

file transfers.

SHOW ESCAPE

This  will  display  the  character being used to escape from a

connection to a remote Kermit.

SHOW FILE-BYTE-SIZE

This command shows whether Kermit is treating seven bits or all

eight bits as significant when writing to a file.

SHOW FILE-TYPE

Shows what type of file Kermit-65 is reading or writing.

SHOW FILE-WARNING

Shows whether or not Kermit-65 will check if a file  exists  on

receive,  and  try  to  modify the name used for the file if it

does.

SHOW IBM

Shows  whether or not Kermit-65 is being used to talk to an IBM

mainframe.

SHOW LOCAL-ECHO Shows whether or not Kermit-65 will print a copy of  characters

on  the  screen  which  are  typed  by the user and sent to the

mainframe during connect processing.

SHOW PARITY

Shows  the  type  of parity being used by Kermit-65 in order to

talk to the other Kermit.

SHOW RECEIVE

Shows the setting of a particular receive packet parameter. The

suboptions available are:

EIGHT-BIT-QUOTE-CHAR            END-OF-LINE     PACKET-LENGTH

PAD-CHAR                        PADDING         QUOTE-CHAR

TIMEOUT

These are analogous to the SET  RECEIVE  options  given

above.

SHOW RS232-REGISTERS

Shows the value of the 2 RS232 registers as a single 16 bit hex

value.

SHOW SEND

Shows the setting of a particular send  packet  parameter.  The

suboptions available are:

EIGHT-BIT-QUOTE-CHAR            END-OF-LINE     PACKET-LENGTH

PAD-CHAR                        PADDING         QUOTE-CHAR

Running KERMIT                                                          Page 14

TIMEOUT

These  are  analogous  to  the  SET  SEND options given

above.

SHOW VT52-EMULATION

Shows whether or not Kermit-65 is acting like a VT52 when talk-

ing to another Kermit during connect processing.

STATUS

The  current  status  of Kermit-65 will be displayed.  This in-

cludes the  number  of  characters  that  have  been  sent  and

received  from  the remote Kermit.  Also included is the amount

of characters which were actual data and the overhead for  both

send and receive.

KERMIT-65 SYSTEM INSTALLATION GUIDE                                     Page 15

4. KERMIT-65 SYSTEM INSTALLATION GUIDE

4.1. THE FILES

The following files should be supplied on the distribution tape:

- C64LBT.BAS - Initial bootstrap program to load C64DXL

- C64LBT.FOR - FORTRAN program at mainframe to talk to C64LBT.BAS

- C64DXL.BAS - Hex disk file loader in BASIC

- C64DXL.HEX - Assembled version of Disk Hex load program

- C64DXL.M65 - Source of program to load KERMIT-65 from C64 diskette

- C64KER.BIN  - Assembled version of KERMIT-65 (Eight-bit Binary Object

code)

- C64KER.HEX - Assembled version of KERMIT-65

- C64KER.M65 - CROSS assembler source for KERMIT-65

- CROSS.MAC - CROSS Microprocessor Assembler (Source)

- CROSS.EXE - CROSS Microprocessor Assembler (Object)

4.2. Building KERMIT-65

.R CROSS<cr>                    ! Run CROSS Microprocessor Assembler

*C64K.HEX/PTP:KIM=C64K.M65/M65  ! Generate .HEX file

This command will produce an ASCII HEX file which can be downline  loaded  onto

the  C64  using  C64LBT. If a listing is desired, one can be produced by adding

",KERMIT.LST" after the "/PTP:KIM" in the command line to CROSS.

4.3. General Procedures and Conventions

Before working with KERMIT on the C64 do the following:

1. Make sure all equipment is turned off.

2. Remove all cartridges from you C64 computer.

3. Turn on your disk drive and monitor.

4. Turn on your C64 computer.

5. Turn on your printer.

KERMIT-65 SYSTEM INSTALLATION GUIDE                                     Page 16

6. Insert your working disk into the disk drive.

7. Follow the procedures outlined below to bring up KERMIT.

4.4. Loading an assembled file

The procedure to bootstrap an assembled KERMIT object file from a mainframe  to

the C64 is as follows:

On  the  C64,  Load  and  run  SUPERMON machine language monitor or equivalent.

Preferably should be in address $C800-$CFFF.

Type in the BASIC program C64LBT.BAS on  the  C64.  This  is  a  dumb  terminal

program  with  a  simple file receive function. It is recommended that the user

save this program as it may be needed to bootstrap newer  versions  of  KERMIT,

C64HXL or C64DXL in the future.

Enter  and save the BASIC program C64DXL.BAS on the C-64.  This is a simple and

slow program to load hex object files  produced  by  CROSS.  A  faster,  better

loader written in CROSS assembler will be loaded with this.

Call and login to the mainframe on which KERMIT-65 resides.

Compile  and execute C64LBT.FOR. This will be used along with C64LBT.BAS on the

C64 to load the C64DXL program. Once C64LBT is executing on the mainframe, give

control back to the C64 and then run C64LBT.BAS on the C64. The procedure is:

RUN<cr>

<hit the F7 key>

Name of File? C64DXL.HEX

Relocate and save C64HXL. Type the following:

SYS 51181 (to enter Supermon)

.S "C64DXL.C000",08,C000,xxxx

Where xxxx is the last address +1 of the program. You will have to

figure out the last address yourself for now.

Now execute C64LBT again:

RUN<cr>

<Hit the F7 key>

Name of File? KERMIT.HEX <cr>

This will take about 35 minutes

Start executing C64DXL:

LOAD "C64DXL.C000",8,1

SYS 49152

KERMIT-65 SYSTEM INSTALLATION GUIDE                                     Page 17

Hex Object File Loader

Object File Name? KERMIT.HEX <cr>

C64DXL will display what it is receiving on the screen as well as

loading it into memory.

Whn finished, C64DXL will display the last address of the program

and return you to SUPERMON. In SUPERMON, save the Program:

.S "KERMIT",08,0801,xxxx+1

Where xxxx is the last address displayed by C64DXL.

You will now have a working version of Kermit.

4.5. Loading a new version of KERMIT-65

Once  KERMIT-65  is on the users system, it can be used to load new versions of

KERMIT-65.  There are two ways of doing this.  One will work in any  enviroment

where  the  initial  bootstrap  procedure  works,  since it depends only on the

capability of transferring text files.   The  other  method  depends  upon  the

capability of being able to obtain and transfer a binary file.

4.6. Loading from a .HEX file

Use  Kermit-65 to transfer the new version of itself over.  Make the C64 file a

Text file. WARNING: This file will take LOTS of space (about  120  sectors)  so

make sure the disk is reasonably empty.

Start executing C64DXL.

LOAD "C64DXL.C000",8,1

SYS 49152

Hex Object File Loader

Object File Name? KERMIT.HEX <cr>

When done, enter SUPERMON and save the file:

.S "KERMIT",08,0801,xxxx+1

Where xxxx is the last address diplayed by C64DXL

The new version of Kermit is now on disk.

KERMIT-65 SYSTEM INSTALLATION GUIDE                                     Page 18

4.7. Loading from the binary version

There  is  yet another way to Bootstrap a new version of KERMIT onto an C64. If

the user has an older version of KERMIT-65 which supports binary mode transfer,

and  has  access  to  a  machine with a valid copy of C64K.BIN, they can simply

transfer  C64K.BIN  using  their  version  of  KERMIT.  Be  sure  to  set   the

File-byte-size  to Eight-bit, and the File-type-mode to Binary before transfer-

ing the file since this is the actual object code.  Also be sure that the send-

ing Kermit is also set for doing a binary transfer.  No special loading or con-

version is needed.

C64LBT.BAS - BOOTSTRAP PROGRAM FOR C64HXL                               Page 19

I. C64LBT.BAS - BOOTSTRAP PROGRAM FOR C64HXL

C64LBT.FOR - MAINFRAME SIDE OF BOOTSTRAP                                Page 20

II. C64LBT.FOR - MAINFRAME SIDE OF BOOTSTRAP

Character and Control Code Definitions                                  Page 21

III. Character and Control Code Definitions

III.1. Keyboard Layout

TO SEND         TYPE

-------         ----

\               BRITISH-POUND

^\              ^BRITISH-POUND

^_              ^=

^               UP-ARROW

^^              ^UP-ARROW

^[ (ESC)        ^[ or f1

_

|

`

~

{

}

DEL (RUB)       Commodore DEL

BREAK           f3

^T              SHIFT/INST

^H (BS)         ^H, or ^T

^J (NL)         SHIFT/RETURN or ^J

^S (DC3)        ^S or HOME

^Q (DC1)        ^Q or CURSOR DOWN

^I (TAB)        ^I (ie. No separate TAB key)

Certain other keys also preform other actions:

III.2. GLASS terminal

The GLASS TTY terminal type prints all characters except ASCII 0-31  and  ASCII

127.  Most control codes are ignored.  The following, however, are recognized:

CHARACTER       EFFECT

---------       ------

^G (BEL)        sounds a beep

^H (BS)         cursor backspaces

^I (TAB)        cursor moves right to next tab stop

^J (NL)         cursor moves down one line with scrolling

^M (CR)         cursor moves to start of line

Table III-1:   Control Codes For Glass TTY

Character and Control Code Definitions                                  Page 22

III.3. VT-52 Terminal

The  VT-52  terminal prints the same characters as the GLASS TTY.  It reacts to

control characters the same as the GLASS,  except  when  ESC  (decimal  27)  is

received.  Kermit will treat the next character received specially; if it isn't

in the following table, KERMIT ignores the character; if it is  in  the  table,

the action is as indicated.

CHARACTER       EFFECT

---------       ------

^[A             cursor moves up one line

^[B             cursor moves down

^[C             cursor moves right one char.

^[D             cursor moves left one char.

^[H             cursor homes

^[I             cursor moves up one line with scrolling

^[J             clears from cursor to end of screen

^[K             clears from cursor to end of line

^[Ylc           cursor moves to line l-$20, col c-$20

^[Z             sends DEC VT-52 terminal id string

^[n             turns off reverse video

^[o             turns on reverse video

Table III-2:   Escape Sequences for VT-52

Character and Control Code Definitions                                  Page 23

VAX UNIX users (Berkeley 4.1bsd) will find that the following "termcap" entries

effectively describe this terminal program for use with the terminal  dependent

UNIX programs (eg. vi, emacs, more, rogue, etc.):

qc|c64|C64|Commodore 64:co#40:li#25:\

:up=\EA:do=\EB:cl=\EH\EJ:am:bs:\

:bw:ho=\EH:nd=\EC:so=\Eo:\

:se=\En:cm=\EY%+ %+ :\

:ce=\EK:sr=\EI:pt:

VAX  VMS  users  can  probably  get  away with the following (although some VMS

programs [eg. edt] require an auxilliary keypad, and  do  not  work  especially

well in 40 column mode):

set term /dev=vt52

set term /pag=25

set term /wid=40

TOPS-20 users can run "emacs" and the like after the following:

term vt52

term length 25

term width 40

term pause com

Table of Contents                                                        Page i

Table of Contents

1. Introduction                                                               1

1.1. Overview                                                              1

1.2. System Requirements                                                   1

2. Known Bugs, Deficiencies                                                   2

2.1. HINTS                                                                 2

3. Running KERMIT                                                             3

3.1. Booting KERMIT                                                        3

3.2. Commands in KERMIT                                                    3

4. KERMIT-65 SYSTEM INSTALLATION GUIDE                                       15

4.1. THE FILES                                                            15

4.2. Building KERMIT-65                                                   15

4.3. General Procedures and Conventions                                   15

4.4. Loading an assembled file                                            16

4.5. Loading a new version of KERMIT-65                                   17

4.6. Loading from a .HEX file                                             17

4.7. Loading from the binary version                                      18

I. C64LBT.BAS - BOOTSTRAP PROGRAM FOR C64HXL                                 19

II. C64LBT.FOR - MAINFRAME SIDE OF BOOTSTRAP                                 20

III. Character and Control Code Definitions                                  21

III.1. Keyboard Layout                                                    21

III.2. GLASS terminal                                                     21

III.3. VT-52 Terminal                                                     22

List of Tables                                                          Page ii

List of Tables

Table III-1:   Control Codes For Glass TTY                                21

Table III-2:   Escape Sequences for VT-52                                 22