SETLIN.DOC

SETLIN - An RT-11 macro program to set/alter values for the four

serial lines on the WEBSTER COMPUTER CORPRATION's PCLV11-J

(MFU) board. Users should be familiar with the PCLV11-J manual.

SETLIN overrides any initial serial lines values provided

by the MFU boot PROM.  These lines may be located at one

of two locations (176500 or 176540).  In addition the fourth line may

be defined as the console for a total of nine possible locations.  The

settable parameters for each line are the baud rate, number of characters,

parity and number of stop bits.

Invoking the SETLIN program :-

1)  Manually at the RT-11 command level by entering: SETLIN or

R SETLIN or

RUN SETLIN to

alter the default values within the SETLIN program.

2)  Via a command file to establish SETLIN's default values on

the PCLV11-J serial lines.  Enter: SETLIN DEF or

R SETLIN DEF or

RUN SETLIN DEF.

It is anticipated that this will form part of the RT-11

STARTS.COM file.

Modifying the default values in SETLIN.SAV :-

enter:	 SETLIN or R SETLIN or RUN SETLIN

receive: Enter option (HELP/MODIFY

enter:	 HELP

receive: help text

enter:	 MOD

receive: display of SETLIN.SAV's existing default values.  Operator

selects CSR line to modify and responds to prompts.  Default

answers are enclosed in [ ] and operator presses RETURN to

accept default answer.  Modifications are implemented

immediately and operator is again invited to either select

another CSR line to modify or exit.  Upon exit operator may

save the session's modifications as a new version of

SETLIN.SAV or to another .SAV file.

Setting the PCLV11-J serial lines to SETLIN.SAV's default values :-

enter:	 SETLIN DEF or R SETLIN DEF or RUN SETLIN DEF

receive: next command in command file or RT-11 dot.  The SETLIN

program does not produce any output when the default option

is invoked.