May 18, 1983

DY22V4.SLP (for V4-Autopatch-G DY.MAC) is a patch for the DY handler to

support  22-bit  addressing of the LSI-11/23 using the Micro Technology

MXV-22 floppy disk controller with TSX+ V3 and later.  The  22-bit  ad-

dress  jumper  (46-47)  must  be  installed  on  the  MXV-22 for 22-bit

addressing to work. Once 22-bit addressing is enabled on the board, the

handler  must support that mode. Thus the handler must be modified even

for the SJ monitor. Use the included command file DY22V4.COM.

Modified 17-May-83 by:

Dale J. Travis

Saturn Systems, Inc.

6875 Washington Ave S.

Suite 218

Minneapolis, MN  55345

612-944-2452