ALEDA User Documentation

Aleda was written by Dennis V.  Jensen

of  the  Ames Laboratory at Iowa State

University as part of U.S.  Department

of Energy contract W-7405-ENG-82.  The

right to use this software release  is

allowed  in  the  public  domain.  The

author,  Ames  Laboratory,  and  USDOE

assume  no  responsibility  as  to the

correctness  of  the  code   nor   the

appropriateness for such use as it may

be applied.  Any  comments  about  the

Aleda  Library  may be directed to the

author  at  258H   Development,   Ames

Laboratory, Ames, Iowa  50011.

1.0   Introduction

____________

Aleda is designed for the Fortran programmer to gain access to

serial  ports for device interfacing and for user interaction.

This library of  subroutines  provides  defaults  for  missing

parameters   wherever   possible  yet  maintains  a  versatile

programmable approach to interfacing.  The  nucleus  of  Aleda

will   detect  the  operating  system  environment  (RT-11  or

TSX-Plus) and adapt appropriately.  This is based  on  options

configured  in  a  device  table and possibly overriden by the

user's  program  at  run  time.   The  ability  to   implement

overlapped  I/O  and  computation (asynchronous processing) is

included.  Response to error conditions is also programmable.

An Aleda routine is invoked with any of the following forms:

Call Lprog ( ...  )

Ierror = Lprog ( ...  )

If ( Lprog( ...  ) ) goto <error processing>

1.1   The Functional Value of Aleda subroutines

___ __________ _____ __ _____ ___________

All Aleda routines  are  setup  as  function  subprograms  and

return  an  integer  as the functional value of the call.  The

values returned indicate the result of  processing  the  call.

These  routines  may  also  be accessed using the FORTRAN CALL

statement.  When using the CALL statement it would be wise  to

always  include  the  optional Return Code parameter to verify

that the routine terminated with no errors.

ALEDA User Documentation                 PAGE 2

Aleda Conventions

1.2   Required and Optional Parameters

________ ___ ________ __________

Aleda subprogram parameters are classified as either  required

or  optional.   Required  parameters  are  necessary  for  the

routines to carry out their specified task.  An error code  is

returned  if  a required parameter is not provided.  Generally

the required parameters are at the beginning of the  parameter

list and optional parameters are at the end.

Optional parameters are those that need not be provided in the

call  list.   They  are  optional  because  they  have default

values, or they return supplemental information  that  is  not

necessary to complete the specified operation.

Parameters are positional;  they must always occupy  the  same

position  in the call list.  Null parameters are those that do

not specify a variable or constant at  an  optional  parameter

position  in  the  list;  the delimiting commas are present or

less than the maximum number of parameters are specified.

Note:  One restriction currently exists for LGETI2 and LGETR4.

They  currently  require  that  trailing  commas exist for all

non-specified optional parameters.  This is because  they  are

written  in  Fortran  and  cannot  correctly  determine if the

parameters exist unless an actual null entry is in the  user's

call  list.   This  restriction  will be corrected in the near

future.

1.3   Return Code Parameters

______ ____ __________

Return codes are associated with most Aleda routines.  If  the

return code is NEGATIVE then this code is an Aleda ERROR CODE.

The return code value will be the same as the functional value

of  an  Aleda  routine  when  an error has occured.  After the

SUCCESSFUL COMPLETION of an Aleda  routine,  the  return  code

will   typically  be  ZERO.   For  some  Aleda  routines,  the

successful  return  code  will  be  a  count   of   characters

transfered.   For  routines  which  interact  with the user, a

return code value of ONE indicates a processing BREAK REQUEST.

For   this  case,  the  functional  value  of  an  Aleda  user

interaction routine can be non-zero without  being  an  actual

error.   (Refer to the description of each routine to know how

to interpret specific return codes.)

1.4   Error Codes

_____ _____

Aleda  indicates  that  processing  errors  have  occurred  by

returning  a  negative  functional  value  for  the  call that

detected an error.  In addition, if an  optional  Return  Code

parameter  was  provided,  it  too  will  be assigned the same

negative value.  A list of the defined error codes and a brief

description of their meaning is provided in a table at the end

of this document.

ALEDA User Documentation                 PAGE 3

Aleda Conventions

The routine LERROR will evaluate an Aleda error code and  list

at   the   console   the   error  number  and  the  associated

description.  Using optional  LERROR  parameters  a  user  can

include  in  the  error  message  text  additional information

specific to the program to help locate the exact point  and/or

cause of the error.

1.5   Control Word Parameters

_______ ____ __________

Some Aleda routines have an optional control  word  parameter.

The  value  of this parameter specifies variations in the type

of processing a particular routine will do.   To  enable  more

than  one  optional processing function it is necessary to sum

the individual control word values of each  desired  function.

(Refer to the description of each routine to know what control

word options are available.)

1.6   Logical Unit Numbers

_______ ____ _______

A connection to a serial port is established through a logical

unit  number.   Each  logical  unit  number  associates with a

device descriptor that provides all  information  and  working

storage  for  correctly  interfacing with a device or terminal

port.  There is a logical unit for each input and each  output

stream.   For  example,  the console keyboard is unit zero (0)

and the console display is unit one (1)  by  convention.   All

other  units  can  be  defined in addition to these to be such

things  as  a  device  supported  under  RT-11  using   direct

interrupts   (DLv11-types   of   interfaces),  as  a  TSX-Plus

Communication  Line  (CL)  type  of  device,  and/or   dynamic

configuration  to  adapt to either of these depending on which

system is being run.

The device table is configured in Macro-11.  The  details  for

configuring  these  tables  are  documented in the Macro files

*dev*.mac .  Typically, this can be configured for your system

once  and  used thereafter.  Some modifications can be applied

from the Aleda call list for simple changes to the  table.   A

typical  device  table  is included in the library and will be

linked  into  an  Aleda  program  if  another  table  is   not

explicitly  included in the link input files.  (See CLTABL.MAC

which  is  the  distributed  device  table  included  in   the

library.)

For  interfacing  to  devices,  the  unit  number  is   always

required.   For  interfacing  to  the user, the unit number is

never specified because units 0 and 1 are always used to  talk

to  the user's terminal.  Units 2 and 3 would typically be the

input stream and output  stream  of  the  first  device  being

interfaced.   The maximum unit number is configured internally

as  24  but  can  be  easily  changed  in  the  ALV51C  module

(DEVICES==24.).

ALEDA User Documentation                 PAGE 4

Aleda Conventions

1.7   The Macro Call Conventions

___ _____ ____ ___________

Aleda provides access  to  facilities  which  were  originally

developed in Macro-11.  Within the source files these routines

are known as Alvin.  Alvin  provides  more  capabilities  than

available   at   the   Fortran   level   but  with  associated

complexities in their use.  The source for these routines  are

well  documented.   For  brevity,  many of the details are not

included here and the user will for now have to refer  to  the

source  to  gain more information.  To aid in using Alvin, the

$Alvin macro  is  defined  in  the  Alvin.MLB  macro  library.

$Alvin defines table offsets of relevant descriptors and other

structures which are used internally.

In a future release more documentation  will  be  included  on

these extended features.

1.8   RT-11 Interrupt Considerations

_____ _________ ______________

When running under RT-11, the interrupts will be processed  by

inline service routines.  To be generally executable under all

monitors and to avoid interference with the user's program, no

.DEVICE  protection is provided automatically within Aleda.  A

device can be dynamically enabled and then disabled  when  not

in  use,  though.   And  NOABORT  can  be added to the console

device definition (see DLTDEV) to prevent control-C aborts and

to  allow  soft  error reporting.  For example, an LBREAK will

detect a control-C, then a call to  ALVEXI  will  disable  all

active  devices,  after  which  a  program  can  STOP  with no

repercussions.   Of  course,  under  the  SJ  monitor   active

interrupts  are  not an issue as a bus reset will be issued on

abort.

When an interrupt is enabled, the exact contents of the vector

and  CSR  are retained.  These are restored when the device is

disabled.  Therefore, what Aleda taketh, it giveth back to  be

as it was before the enable.

ALEDA User Documentation                 PAGE 5

Detailed Description of Aleda Device Interfacing Routines

LENABL

2.0   Description

___________

Fortran callable subroutine to enable  the  specified  logical

unit number for the transfer of characters.

2.1   Calling Sequence

_______ ________

Ierror = LENABL ( Unitnumber, [Buffer], [BufferSize],

[ReturnCode], [ControlWord], [CSR],

[Vector], [AssociatedUnit]        )

2.2   Parameter Description

_________ ___________

Unitnumber:         Required parameter.  Logical  unit  number

of device to be enabled.

Buffer:             Optional parameter.  Array which  will  be

used  as  the  circular  Buffer into which

data will  be  written  by  the  specified

logical unit.

BufferSize:         Optional parameter.  Size in bytes of  the

buffer  mentioned  above.  Buffers must be

at least 20 bytes long.

ReturnCode:         Optional parameter.  Variable which, after

the  call,  will  contain  a numeric value

indicating  the  success  or  reason   for

failure of a particular call.

ControlWord:        Optional   parameter.    Value   indicates

certain  optional processing functions are

to be performed.   No  optional  functions

are currently defined for this routine.

CSR:                Optional  parameter.    Address   of   the

Control  Status  Register of the device to

be  enabled.   (Applicable  to  RT  direct

interrupt support only.)

Vector:             Optional parameter.  Interrupt  vector  of

device  to  be enabled.  (Applicable to RT

direct interrupt support only.)

AssociatedUnit:     Currently not used.

ALEDA User Documentation                 PAGE 6

Detailed Description of Aleda Device Interfacing Routines

LDSABL

3.0   Description

___________

Fortran callable subroutine to disable the  specified  logical

unit  number.   I/O  will  be prohibited on the unit after the

disable.  All interrupt linkage will be reverted to the  state

prior to enable.

3.1   Calling Sequence

_______ ________

Ierror = LDSABL ( UnitNumber, [ReturnCode] )

3.2   Parameter Description

_________ ___________

UnitNumber:         Required parameter.  Logical  unit  number

of device to be disabled.

ReturnCode:         Optional parameter.  Variable which, after

the  call,  will  contain  a numeric value

indicating  the  success  or  reason   for

failure of a particular call.

ALEDA User Documentation                 PAGE 7

Detailed Description of Aleda Device Interfacing Routines

LGETLN

4.0   Description

___________

Fortran callable subroutine to access  the  specified  logical

unit  number  for transfer of characters to the user specified

buffer.

4.1   Calling Sequence

_______ ________

Ierror = LGETLN ( UnitNumber, Line, LineSize,

[ReturnCode], [ControlWord],

[TerminatorArray]           )

4.2   Parameter Description

_________ ___________

UnitNumber:         Required parameter.  Logical  unit  number

of the device from which characters are to

be transferred.

Line:               Required  parameter.   Array  into   which

characters from the specified logical unit

will be transferred.

LineSize:           Required parameter.  Size in bytes of  the

buffer LINE mentioned above.

ReturnCode:        Optional parameter.  If positive- number of

characters  transferred  into  LINE on the

current call.  Else- error code.

ControlWord:        Optional   parameter.    Value   indicates

certain  optional processing functions are

to be performed.

0           Do not perform any special processing.

1           Interpret  the  characters  in  the   user

specified  TerminatorArray  as end-of-line

characters.

2           Include  the   parity   bit   when   doing

character transfer.

TerminatorArray:    Optional parameter.  String of  characters

which  will  be interpreted as end-of-line

characters.  The  first  byte  of  a  this

array  is  the  number of characters which

are terminators, then each terminator byte

follows immediately in the array.

ALEDA User Documentation                 PAGE 8

Detailed Description of Aleda Device Interfacing Routines

LPUTLN

5.0   Description

___________

Fortran callable subroutine to output characters from  a  user

specified line buffer to a specified logical unit number.

5.1   Calling Sequence

_______ ________

Ierror = LPUTLN ( Unitnumber, Line, LineSize,

[ReturnCode], [ControlWord],

[TerminatorArray]           )

5.2   Parameter Description

_________ ___________

UnitNumber:         Required parameter.  Logical  unit  number

of the device to be written to.

Line:               Required parameter.   Array  to  hold  the

characters  which  will  be  output to the

device specified by the unit number.

LineSize:           Required parameter.  Size  of  the  buffer

array LINE in bytes.

ReturnCode:         Optional parameter.  Number of  characters

actually   transferred  to  the  specified

device on the current call.

ControlWord:        Optional   parameter.    Value   indicates

certain  optional processing functions are

to be performed.

TerminatorArray:    Optional parameter.  String of  characters

which  will  be interpreted as end-of-line

characters.  The  first  byte  of  a  this

array  is  the  number of characters which

are terminators, then each terminator byte

follows immediately in the array.

5.3   Control Word Value Description

_______ ____ _____ ___________

0           Do not perform any special processing.

1           Interpret  the  characters  in  the   user

specified  TerminatorArray  as end-of-line

characters.

2           Include  the   parity   bit   when   doing

character transfer.

8           Output any  terminator  encountered  along

with the other characters.

ALEDA User Documentation                 PAGE 9

Detailed Description of Aleda Device Interfacing Routines

LRECEV

6.0   Description

___________

Fortran  callable  subroutine  to  initiate  the  transfer  of

characters  from  the specified logical unit number to an user

specified circular buffer.  An LRecev is required  before  the

first LGetLN to a specified logical unit.  Subsequent LRecev's

are optional and can be used  to  accomplish  such  things  as

flushing  the current contents of the circular buffer (control

option RESET).

6.1   Calling Sequence

_______ ________

Ierror = LRECEV ( UnitNumber, [Buffer], [BufferSize],

[ReturnCode], [ControlWord]       )

6.2   Parameter Description

_________ ___________

UnitNumber:         Required parameter.  Logical  unit  number

of  the  device  on  which  the  character

transfer is to be initiated.

Buffer:             Optional parameter.  Array which  will  be

used  as  the  circular  buffer into which

data  will  be  written  by   the   device

specified by UnitNumber.

BufferSize:         Size of the circular buffer in bytes.   If

size  =  0, then the buffer is presumed to

already be initialized

ReturnCode:         Optional parameter.  Variable which, after

the  call,  will  contain a numberic value

indicating  the  success  or  reason   for

failure of a particular call.

ControlWord:        Optional   parameter.    Value   indicates

certain  optional processing functions are

to be performed.  Alvin receive  functions

PARON,   PAROFF,  SILENT,  and  RESET  are

available.   (No  echo   is   SILENT=10000

octal, buffer RESET=1000 octal)

ALEDA User Documentation                PAGE 10

Detailed Description of Aleda Device Interfacing Routines

LERROR

7.0   Description

___________

Fortran callable subroutine to output to the console a message

associated with ALEDA error return codes.

7.1   Calling Sequence

_______ ________

Ierror = LERROR ( ErrorCode, [Value], [UserLine, Count] )

7.2   Parameter Description

_________ ___________

ErrorCode:          Required parameter.   Numeric  error  code

value  returned  by one of the other ALEDA

routines.

Value:              Optional   parameter.     Numeric    value

supplied by the user which will be printed

at the console along with  the  associated

ALEDA error message.

UserLine:           Optional  parameter.    Character   string

supplied by the user which will be printed

at the console along with  the  associated

ALEDA error message.

Count:              Required parameter if  parameter  UserLine

specified,  otherwise  ignored.  Number of

characters in character string UserLine.

ALEDA User Documentation                PAGE 11

Detailed Description of Aleda Device Interfacing Routines

LCNVTD

8.0   Description

___________

Fortran callable subroutine to  convert  a  two  byte  integer

value   to   a   character  string  representing  the  decimal

equivalent of that value.  The value of the optional parameter

CONTROL WORD determines the format of the string returned.

8.1   Calling Sequence

_______ ________

Ierror = LCNVTD ( Value, Stringbuffer, [Returncode],

[ControlWord]                     )

8.2   Parameter Description

_________ ___________

Value:              Required parameter.  Integer*2 value which

the   character   string   returned   will

represent.

StringBuffer:       Required parameter.  Area into  which  the

character  string  representation of value

will be placed.   This  buffer  should  be

seven bytes long.

ReturnCode:         Optional  parameter.   Number   of   valid

characters  in the converted string.  This

value  should  be  used  in  printing  the

string   returned  because,  with  certain

formatting   options,   StringBuffer   may

contain   garbage  characters  beyond  the

first ReturnCode number of characters.

ControlWord:        Optional parameter.  Value determines  the

format of the string returned.

8.3   Control Word Values and Associated Formats

_______ ____ ______ ___ __________ _______

0 (default)    Left justify  string  in  buffer  with  no

trailing decimal point.

1,3            Right justify string in first 5  positions

of  buffer if value was positive, or first

6 positions if string has  a  minus  sign.

The string will contain leading zeroes and

will not include a decimal point.

2              Same format as ControlWord = 1, except any

leading zeroes will become blanks.

4              Left justify string and  include  trailing

decimal point.

5,7            Right justify string in first 6  positions

of  buffer if value was positive, or first

7 positions if value  was  negative.   The

last  valid character in StringBuffer will

be a decimal point  and  the  string  will

include leading zeroes.

6              Same format as ControlWord = 5, except any

ALEDA User Documentation                PAGE 12

Detailed Description of Aleda Device Interfacing Routines

leading   zeroes  in  string  will  become

blanks.

ALEDA User Documentation                PAGE 13

Detailed Description of Aleda User Interfacing Routines

LPUTST

9.0   Description

___________

Fortran callable subroutine to output a string to the  console

listing device.  This routine expects strings to be terminated

with a zero byte (null) or a backslash (\).  If the string  is

terminated with a backslash, LPUTST will add a carriage return

and a linefeed after the message has  been  transmitted.   The

current maximum string length is 256 characters.

9.1   Calling Sequence

_______ ________

Ierror = LPUTST ( String, [ReturnCode] )

9.2   Parameter Description

_________ ___________

String:             Required parameter.  Character  string  to

be  output  to the console listing device.

String must be less  than  256  characters

long  and  terminate with a null byte or a

backslash.

ReturnCode          Optional parameter.  Value  returned  will

be  the  number of characters transmitted,

if positive, or a negative error code.

ALEDA User Documentation                PAGE 14

Detailed Description of Aleda User Interfacing Routines

LGETI2

10.0  Description

___________

Fortran callable subroutine to prompt for and accept, from the

console  keyboard,  an  ascii string representing an Integer*2

value,  convert   the   value   to   its   internal   16   bit

representation,  and return the value to the caller.  Optional

default, minimum, and maximum values may be  provided.   If  a

default value is provided, it will be displayed along with the

caller supplied Prompt.  If minimum and/or maximum values  are

provided,  they will be displayed when keyboard input can't be

converted to  a  valid  value,  eg.   entering  an  alphabetic

character  in  response  to  the  prompt  will  cause an error

message and the minimum and maximum values  to  be  displayed.

Prompting  will  continue  until  a  valid  Integer*2 value is

provided.

10.1  Calling Sequence

_______ ________

Ierror = LGETI2 ( Prompt, Var, [ReturnCode], [Default],

[Minimum], [Maximum]                )

10.2   Parameter Description

_________ ___________

Prompt:             Required parameter.  User supplied  string

which   will  be  output  to  the  console

listing device to prompt of input.

Var:                Required  parameter.   Integer*2  variable

which    will    contain    the   internal

representation of  value  input  from  the

keyboard.

ReturnCode:         Optional parameter.   Value  will  be  the

number   of   characters  input  from  the

keyboard  if  there  was   no   error   in

processing,  otherwise  it will contain an

error code value.

Default:            Optional parameter.  Value  that  will  be

returned  if  a carriage return is entered

in response to the prompt for input.

Minimum:            Optional parameter.  Minimum  valid  input

value.   Entering  a valued less than this

will cause an error message and the  valid

range of input values to be displayed, and

the user will be prompted to enter another

value.

Maximum:            Optional parameter.  Maximum  valid  input

value.  Entering a value greater than this

will cause an error message and the  valid

range of input values to be displayed, and

the user will be prompted to enter another

value.

ALEDA User Documentation                PAGE 15

Detailed Description of Aleda User Interfacing Routines

LGETR4

11.0  Description

___________

Fortran callable subroutine to prompt for and accept, from the

console keyboard, an ascii string representing a Real*4 value,

convert the value to its internal 4 byte  representation,  and

return  the  value  to the caller.  Optional default, minimum,

and maximum values may be provided.  If  a  default  value  is

provided,  it will be displayed along with the caller supplied

Prompt.  If minimum and/or maximum values are  provided,  they

will  be displayed when keyboard input can't be converted to a

valid value, eg.  entering an alphabetic character in response

to  the prompt will cause an error message and the minimum and

maximum values to be displayed.  Prompting will continue until

a valid Real*4 value is provided.  Values may be in decimal or

scientific notation, but, either way, input values and  values

for parameters Default, Minimum and Maximum should all contain

a decimal point.

11.1  Calling Sequence

_______ ________

Ierror = LGETR4 ( Prompt, Var, [ReturnCode], [Default],

[Minimum], [Maximum]                )

11.2  Parameter Description

_________ ___________

Prompt:             Required parameter.  User supplied  string

which   will  be  output  to  the  console

listing device to prompt for input.

Var:                Required parameter.  Real*4 variable which

will  contain  the internal representation

of value input from the keyboard.

ReturnCode:         Optional parameter.   Value  will  be  the

number   of   characters  input  from  the

keyboard  if  there  was   no   error   in

processing,  otherwise  it will contain an

error code value.

Default:            Optional parameter.  Value  that  will  be

returned  if  a carriage return is entered

in response to the prompt for input.

Minimum:            Optional parameter.  Minimum  valid  input

value.   Entering  a valued less than this

will cause an error message and the  valid

range of input values to be displayed, and

the user will be prompted to enter another

value.

Maximum:            Optional parameter.  Maximum  valid  input

value.  Entering a value greater than this

will cause an error message and the  valid

range of input values to be displayed, and

the user will be prompted to enter another

value.

ALEDA User Documentation                PAGE 16

Detailed Description of Aleda User Interfacing Routines

LGETYN

12.0  Description

___________

Fortran callable subroutine to access the console input device

for  a  yes  or  no response to a question.  This routine will

return a 1 if a break was requested, a 2 if a yes response was

entered,  and  a 3 if a no response was entered.  These return

values imply that the routine  completed  normally.   Negative

return  values are ALEDA errors.  The user supplies the prompt

for input.

12.1  Calling Sequence

_______ ________

Ierror = LGETYN ( Prompt, [ReturnCode] )

12.2  Parameter Description

_________ ___________

Prompt:             Required parameter.  User supplied  string

which  will  represent a question having a

yes   or   no   answer.     LGETYN    will

automatically append a question mark and a

blank to the string.

ReturnCode:         Optional  parameter.    Integer   variable

which  will,  after returning, contain the

same value as the functional return  value

of  the routine, ie.  1 = break requested,

2 = a yes response was entered, 3 =  a  no

response was entered, and negative value =

an ALEDA error occured.

ALEDA User Documentation                PAGE 17

Detailed Description of Aleda User Interfacing Routines

LBREAK

12.0  Description

___________

Fortran callable subroutine to  check  if  the  console  input

device  has a pending break request.  This routine will return

a true functional value if a  break  has  been  requested.   A

repercussion  of this routine is that the input buffer will be

drained, thus eliminating any characters which may  have  been

typed  ahead.  This routine is useful to escape from processes

on demand.

13.1  Calling Sequence

_______ ________

If (LBREAK) Goto Exit

ALEDA User Documentation                PAGE 18

Appendix A - Aleda Return Codes/Error Codes

The following is a brief description of the  meaning  of  each

numeric  return  code.  The meaning associated with each value

returned  should  be  consistent  across  all  calls  to   all

functions.

Return Code         Description

______ ____         ___________

1              Break requested.

0              Normal completion.

-1 .. -19          Required parameter  n not provided.

-20              No device descriptor.

-21              Device not enabled.

-22              Device already enabled.

-23              CSR not in I/O page.

-24              Vector out of range.

-25              Device is not an input device.

-26              Device is not an output device.

-27              ALVIN enable failure.

-28              ALVIN disable failure.

-29              Device already active (Seized) and using a

different buffer.

-30              Buffer not at least 20 bytes.

-31              Buffer not word aligned.

-32              Buffer overrun.

-33              Terminator   processing   specified    but

terminator array not provided.

-34              Terminator not found.

-35              Input  error   detected   (parity   error,

character overrun, or framing error).

-36              Console is seized.

-37              No   buffer   descriptor   for   attempted

operation.

-38              Failed to connect to CL.

-39              Could not configure  for  CL  under  RT-11

(RT-11 direct interrupt support not genned

in).

-40              Unit number is out of range.

-41              Can not enable this device under TSX-plus.

-42              Could not open channel to CL.

-43              Direct interrupt table is full.

-44              Vector is seized.