RT-11 Pseudo-Handlers to emulate EIS, FIS, and FPU instructions.

by Paul Lustgraaf

32 Carver Hall

Iowa State University

Ames, IA  50011

515-294-0324

Adapted from EI.SYS by Dr. P. C. Waggett.

This is a collection of three pseudo-device handlers for RT-11

which will enable programs that require certain optional hardware

to be run on PDP-11's that can not or just do not have that option.

For example, FORTRAN-77 requires the FPU option, but the PDP-11/03

does not have that option.  However, by using this package, F77

CAN be run on an 11/03.

The handlers included are:

EI	Supports EIS instructions plus a few others:

(MUL, DIV, ASH, ASHC, XOR, SXT, SOB)

Requires 498. words of memory.

FI	Supports FIS instructions as used on the 11/03 and 11/40:

(FMUL, FDIV, FSUB, FADD)

Requires 428. words of memory.

FP	Supports FPU instructions as used on 11/45, 11/23, etc.

Requires 1476. words of memory.

METHOD OF OPERATION

In order to enable emulation, the appropriate handler(s) must

be loaded and set ON.  For example:

LOAD FP: (or EI or FI)

SET FP ON

UNLOAD FP:

LOAD FP:

This sequence of commands, which may be included in your

startup command file, loads the handler into memory permanently

and then steals the interrupt vector at location 10 from the

monitor.  If you are not intimately familiar with the PDP-11

architecture, I will explain briefly what is happening here.

When an illegal instruction is executed, the PDP will trap

to the address stored at location 10.  The handler will

intercept any such trap and will inspect the instruction

causing the trap to see if it is one of the instructions

the handler is able to emulate.  If so, the operation will

be done and control will be returned to the program.  If not,

the handler will pass control to the address previously

pointed to by location 10.  Since the previous trap address

is stored inside the handler in memory and RT-11 only changes

the handlers image on disk, the unload-load sequence must be

done in order to load the previous trap address in memory.

Note that this mechanism allows us to have many such handlers

chained to from location 10.

If you no longer need to use these pseudo-handlers and you

wish to remove them from memory, you must first issue the

command to turn them off, then unload them.

SET FP OFF

UNLOAD FP:

The set OFF command restores the previous trap address to

location 10.  If you have more than one such pseudo-handler

in memory, you MUST turn them off in EXACTLY the REVERSE

order you turned them on.  Any attempt to unload one of

these handlers before turning it off will probably cause

the system to crash at some unknown future point.

DON'T DO IT!

RESTRICTIONS

These handlers are intended to allow programs to be run on

machines which would not otherwise support their execution.

These handlers will, of course, execute much more slowly

than the equivalent hardware.  Unlike the original EI.SYS,

these handlers do not use self-modifying code, so they

may be used in multi-tasking systems.

1.  Execution under XM may work, but has not been tested.

The problem is that under XM the handler has to run

at priority 7 in order to lock out all interrupts and

prevent XM from doing a context switch.  Needless to

say, the length of a typical fixed or floating point

instruction may adversely affect the interrupt

response time.  ANYONE CARE TO WORK ON THIS PROBLEM?

2.  An attempt has been made to emulate the exact behavior

of pathological instructions, particularly those which

cause some type of trap.  Due to the complexity of this

problem, however, some differences may exist with the

hardware behavior.  USER BEWARE!

3.  FP does not implement the interrupt for undefined

variables (sometimes referred to as -0).

4.  The addition/subtraction routine in FI: sometimes

differs in the least significant bit from the result

given by the hardware.  I believe this is due to some

difference in the rounding algorithm.  HELP!

5.  NO GUARANTEES expressed or implied, etc. etc. etc.

PERFORMANCE

Two test programs are included, TESTR4 and TESTR8, which

are versions of a test program published by Bill Savage

in the September 1983 issue of Dr. Dobb's Journal.  The

correct result is exactly 2500 and it is very educational

to try these programs on different machines.  The following

results were all obtained using FORTRAN 4 with the indicated

options:

Code	Hardware

Program	Machine	Option	Option		Result		Time(sec)

TESTR4	11/03	THR	none		2631.716	 145

TESTR4	11/03 	EIS	none (EI:)	2631.716	 714

TESTR4	11/03	FIS	none (FI:)	2972.816	 314

TESTR4	11/03	FPU	none (FP:)	2741.048	1017

TESTR4	11/03	EIS	EIS		2631.716	 103

TESTR4	11/03	FIS	FIS		2631.716	  28

TESTR4	11/23	FPU	FPU		2741.048	  25

TESTR8	11/03	EIS	EIS		2500.000	 556

TESTR8	11/03	FIS	FIS		2500.000	 494

TESTR8	11/03	FPU	none (FP:)	2500.000	1551

TESTR8	11/23	FPU	FPU		2500.000	  60

The moral of the story is:

If you have the source and your language gives you the choice,

RE-COMPILE FOR YOUR OWN HARDWARE!

Otherwise, you may be able to limp along using these handlers.

GOOD LUCK!