HS: A HANDSHAKING SERIAL LINE HANDLER

Written by

GARY PRECKSHOT

UNIVERSITY OF ARIZONA

TUCSON, ARIZONA

October 27, 1980

INTRODUCTION

____________

A major problem in  installations  where  a  number  of

small  computers  exist is that of transfering data from one

machine to another.  Usually this occurs when it is  desired

to move program source files developed on one machine having

peripherals convenient for program development to a  machine

less well equipped.  For example, transferring programs from

a hard disk machine to one having floppy disks or  no  disks

at  all.   I present here a serial line handler which allows

convenient file transfer between two machines running  under

DEC  RT-11  Version  3B  or  later, and whose protocol is so

simple that  a  complementary  serial  I/O  handler  may  be

written easily for other operating systems.

THE HS: HANDLER

___ ___ _______

The HS:  handler  is  a  modular  serial  line  handler

configured  for  installation under RT-11 V3B which achieves

block-by-block or line-by-line synchronized transmission  of

data  across  a  serial  data link.  It does this by using a

very simple handshake in  which  the  sending  end  requests

permission  to  send  data  before actually transmitting the

data object in question.  The  handler  operates  either  in

data  transparent  block  (512 bytes) mode, or in ASCII-only

line mode, parity suppressed.   Transmitting  mode  must  be

selected  by  using the SET command, while receiving mode is

selected automatically by handshake.

In  either  mode,  synchronization   is   acheived   by

transmitting  an  endless series of ENQ characters until the

receiver responds with a single ACK.  Clearly this  requires

that  the  link  be  operated  in  full  duplex  mode.   The

transmitter  indicates  its  mode  in  the   first   non-ENQ

character  it  transmits after receiving ACK.  STX indicates

block mode, and any other  ASCII  character  indicates  line

mode.

In block mode, the next 512 bytes are considered to  be

data,  and  are  terminated by the 513th byte, an ASCII ETB.

There are no redundancy checks,  and  the  block  terminator

represents  the  only  check on completed transmission.  The

buffer size given to the handler must be an integral  number

of  512 byte blocks, but resynchronization occurs after each

block, so that the receiver needs a minimum buffer  size  of

one block.

In line mode, transmission continues until a  linefeed,

control  Z, or end of transmission buffer is encountered, at

which  point  resynchronization  occurs.   End   of   buffer

resynchronization  is indicated to the receiver by appending

an ETB character.  Control Z is used to indicate an  end  of

file.

1

2

In both modes, end of file is indicated by transmission

of  control  Z.   Since  the block mode is data transparent,

this is actually implemented by replacing the first  non-ENQ

character  (STX  in  block  mode), with control Z, which the

receiver interprets  as  a  line  containing  but  a  single

control  Z,  indicating end of file.  However, control Z may

be embedded in a line in line mode, which will give rise  to

a receiver report of end of file.  The handler is configured

so  that  it  automatically  transmits  a  single  character

control  Z  line  upon  being  closed,  so  that closing the

transmitter  will  cause  an  end  of  file  report  at  the

receiver.

USING THE HS: HANDLER

_____ ___ ___ _______

Line mode of this handler  was  implemented  for  those

uses  that generally involve line-by-line dialogue between a

program and what it believes is a teletype or terminal.   It

is  generally  unsatisfactory  for use with systems programs

because it will usually result in files that have  only  one

line  per  block.   It  is  very satisfactory for those uses

which require  immediate  transmission  upon  a  terminating

character,  in  this  case  linefeed,  which  block oriented

handlers will not do.  The user should take care when  using

the special characters ETB, control Z, and STX, as these may

lead to unintended results in line mode.

Block mode of HS: was implemented  so  that  HS:  would

appear  to be a standard block oriented device that could be

substituted by use of the RT-11 ASSIGN command for any legal

device  in  the  system.   HS:DUMMYF.ILE  may be used in ANY

___

position  that  is  legal  for  an   RT-11   standard   file

specification.  In particular, it may be used with the RT-11

COPY command to copy a file to a link, on the other  end  of

which  is  another RT-11 system using COPY and HS: to copy a

file from the link.  In a close-coupled short-link situation

the  lack  of  redundancy  checking in HS: may be easily got

around by the following command strings in the  sending  and

receiving systems:

SEND:   COPY

FILEN.AME

HS:

DIF

HS:FILEN.AME

FILEN.AME

RECV:   COPY

HS:FILEN.AME

SY:

COPY

FILEN.AME

HS:

3

The DIF command will compare the retransmitted file  to

the original and provide an even more secure indication than

partial redundancy checking, since, in  fact,  what  we  are

doing is 100 % redundancy checking.

CONCLUSION

__________

A handshaking synchronizing handler  was  written  that

satisfies  a  need for local transmission of data regardless

of the speed of copying devices at either end  of  a  serial

link.   The  fact  that  it  is modular enables its use with

standard system utilities under RT-11 V3B in block mode, and

a  line  mode  is  provided  for  those wishing line-by-line

synchronized data transmission.  Because we envision its use

only   in   a  closely  coupled  local  environment,  100  %

redundancy  checking  for  accuracy   of   transmission   is

practical.    Commented   copies   are   available  from  G.

Preckshot.