Ken Demers

MS-48

United Technologies Research Center

Silver Lane

East Hartford, Conn.  06108

74-148 Santa Rosa Circle

Palm Desert, Ca. 92260

April 18, 1980

Dear Mr. Demers:

After having to recover data from three different  bad  disks  in  two

weeks,  the  thought occurred to me that some of your readers may also

want to know how to recover data from a corrupted disk.   But  just  a

warning:   Do  not  try  to  recover  data  from a disk which has been

involved in a "head crash.":

HOW A HARD DISK IS MADE

A hard disk is made from a carefully machined aluminum  platter  which

has  been coated on both sides with a magnetic coating.  When the disk

pack is put into a drive and it is spinning with the heads loaded, the

heads  are  suspended  above  the rotating disk on a thin film of air.

Only a few millionths of an inch of air separate the  heads  from  the

disk  surface.   As  forgiving as disks often are, a particle of dust,

smoke, or even a fingerprint on the surface of the disk  can  cause  a

colision with a head of the disk drive.  Since the disk at its outside

edge is traveling at about  l000  inches  per  second,  such  a  minor

collision can warp the head so that it makes physical contact with the

surface of the disk.  This can cause the head to actually plow a  neat

little  furrow around the disk platter, spewing flakes of the magnetic

coating and aluminum shavings throughout the inside of the disk drive.

The damage from a head crash can be considerable.   Just  picture  the

mechanics  and  electronics  of the inside of a drive throughly coated

with sparkling, conducting chips of aluminum.  This tends to  be  hard

on  both the drive and on the data on the disk pack.  (The description

comes from first hand experience -- I have a drive which  had  a  head

crash about three years ago.  We vacuumed out the drive and dismantled

and vacuumed and replaced the  heads  and  vacuumed  and  crossed  our

fingers, put an old pack in it and it worked!  And it is still working

as the main disk drive in my #2 computer system -- running  constantly

ever  since.   But  whenever  we  open  it  up to clean the heads, the

insides still glitter like a Las Vegas chorus line.

And I think we've all heard the story about the fellow who had a  disk

go  bad,  so  he put in another pack, but it didn't work either, so he

put that pack in another drive, ad nauseam, until all  of  his  drives

had  head  crashes  because of the crashed pack he kept moving around,

and of course these newly crashed drives destroyed  other  disk  packs

wiping out all of his backup packs ...  let's hear it for mag tape.

Page   2

Anyway, while I am telling you how to recover data  from  a  corrupted

pack,  keep  in  mind  that  you  should  never try to read physically

__________

damaged media lest you damage the drive also and any other media which

follows its way into the drive.

Well, then, what is corrupted media which can be read?

___

A.  Disks with corrupted directories.

B.  Disks with hard errors.

C.  Deleted files.

D.  Stupid errors ("REPLACE RKO:" in Basic).

Some of these can be recovered easier and more reliably  than  others.

But let's explain a few things first.

1.   Formatting  a  disk  (completely)  results  in  a  disk  with  no

recoverable data on it whatsoever.

2.  A partially formatted disk (you know,  one  of  those  late  night

sessions  when  you tell the machine to format the wrong disk and have

the immediate presence-of-mind to turn off the power on  the  computer

or  halt it before it stops formatting.  Control C's just don't cut it

in this situation.) has recoverable data on the portion  of  the  disk

which wasn't just freshly formatted.

3.  A zeroed directory (INIT)  does  just  that  ...   it  zeroes  the

directory.   The  data  is still on the disk -- only the directory has

been erased.  Given enough patience and a long  enough  lifespan,  the

entire disk can be recovered.

THE TOOLS

RT-11 provides several tools which are commonly  used  to  operate  on

files.   Some  of these can be used to also read and write a device --

not just a file on the device.

My favorites are (in order of preference):

TECO to find, read, and transfer.

BASIC to find, read, and transfer.  (I guess that  FORTRAN  will  too,

but it isn't interactive like TECO or BASIC).

DUMP -- to find -- especially RAD50.

PATCH -- to try to correct -- of limited usefulness generally, but  in

specific cases entremely valuable.

ODT - MostLy useful by first-class Wizards or better.  For getting  it

right  from  the  horse's  mouth.   Careless  keystrokes are sometimes

dangerous.

Page   3

THE PROCEDURE:

1) Copy the media to a freshly formatted like-media or a larger  media

(copy  a bad floppy onto an RK if you need to, or a bad RK onto an RL)

using the COPY/DEVICE command.  (If you are a wizard second-class  and

you only have two drives, use ODT microcode on an LSI (or on any other

machine BOOT RT-11 from a good pack, SET USR NOSWAP, RUN ODT.SAV (JUST

LINK  ODT.OBJ  by itself) and then remove your system pack).  Put in a

scratch pack and key in a format program if necessary using  ODT  (one

of the RT11-V2C Books had a format program in it -- I think it was the

System Reference Manual) and then key in a program to  read  a  sector

and  write  a  sector  and just spin on the ready bit.) But it is much

easier if you have 3 drives (a System drive (even floppy or TU58  will

do) and the corrupted disk and the new copy).

2) After you have a copy of the corrupted  disk,  you  can  breathe  a

little  easier and put a minimum-system disk in one drive and the copy

of the bad disk in the other.  The minimum-system disk should have  at

least:    RKMNXX.SYS,   SWAP.SYS,   RK.SYS,  TT.SYS,  LP.SYS,  NL.SYS,

TECO.SAV, DUP.SAV,  DIR.SAV,  PIP.SAV,  PATCH.SAV,  BASIC.SAV,ODT.SAV,

DUMP.SAV, and SRCCOM.SAV.

3)If you know exactly what you want  to  recover:   (for  example  you

backed  up  the  disk yesterday, but you really need to see the latest

copy of the program since then  and  was  stored  on  the  bad  disk),

assuming it began with ".TITLE FOO":

R TECO

*ERRK0:$$(the bad disk)

*_.TITLE FOO

$-T$$

and wait.  (You will encounter several  ".TITLE  FOO"s  on  your  disk

because  old  data  is  never  erased  from  a disk -- even when it is

DELETEd -- it is just written over again and again.  Your goal  is  to

find the latest copy.) Sooner or later, TECO will respond with:

.TITLE FOO

*

(Then type)

-L0,.K$<A>$$

(TECO responds with)

MEMORY OVERFLOW

(then type)

JS.ENDS$$

(The 'S' on the end keeps you from landing on '.ENDC', '.ENDM', or

'.ENDR').

*LHT$$

(And watch the world go by.  Adjust as necessary.)

*EWRK2:FOO1.MAC$0,.PWEF$$

*(Do the _.TITLE FOO

$$ again and copy it out onto 'RK2:FOO2.MAC', etc  until  you  get  an

input error.

An input error on TECO indicates an end-of-volume  or  a  hard  error.

TECO  stops and will not cross a hard error, but a COPY/DEVICE will --

that is one of the reasons for the COPY/DEVICE --  to  have  a  volume

free of hard errors.

Page   4

4) If you haven't backed up the volume for a couple of decades and you

want to try to recover everything, may I suggest BASIC?

5 L% = (however-many-characters at the start of each block you want to

see)

10 DIM  1, A$(4800,3)=128 \ REM THE 4800 IS THE NUMBER OF BLOCKS IN AN

RK

20 OPEN 'RK0:' FOR INPUT AS FILE  1

30 FOR A% = 0 TO 4800

40 A$ =A$(A%,0)

50 A1$=SEG$(A$,1,L%)

60 PRINT A%,A1%

70 NEXT A%

80 CLOSE  1

90 END

This works best on an LA120 where the page length can be  set  to  one

line because many FORM FEEDS will undoubtedly be encountered and using

this program to print out 4800 lines wastes enough paper as it is.

Once the block numbers to be saved have been determined, use:

.R DUP

*FILE1.TST=/C:(start block number).:(file length).

*FILE2.TST=/C: (... as above)

Read page 8-3 in the System User's Guide and don't forget  to  type  a

______ ______ _____

decimal  point  after  the  numbers.   Also  note  the contiguous-file

restrictions.

5) Patching a directory:

R PATCH

FILE NAME --- RK0:/A

*6000/ nnnnnn

Read section C.7.1 "RT-11 File Storage" beginning on page C-65 in  the

Advanced   Programmer's   Guide  for  information  on  what  an  octal

________   ____________   _____

representation of a directory looks like  and  what  all  the  numbers

mean.

There is an undocumented feature in PATCH which allows you  to  search

for  the  storage  location  of  any number in any file on any device.

That feature is:

*nnnnnn;S  (where nnnnnn is the number you are searching for)

If you have used the '/A' switch on the file name,  the  ';S'  command

will  cause  a  read  error  and  PATCH  will return to the monitor on

encountering end-of-file.  No harm has been done  --  just  run  Patch

again.

DUMP can be used to print out the contents of the directory.

R DUMP

*TT: (or LP:) =RK0:/S:6/E:20/X

where '6' is always the start of the directory on any device, and '20'

is  a pretty good guess about where most directories end.  If you need

more, then print more.

ODT.SAV can be used to make RAD50 to octal conversions.

R ODT

Page   5

*400/000000  X= F

402/000000   X= ILE

404/000000   X= BAD

*400/000006

402/035045

404/006254

The purpose of this document has been to spur your thinking a  little.

To  fill in all of the necessary details would take a whole book.  And

of course experience is the best teacher.

My final suggestion is that you try all of this first on a spare  COPY

of  a  scratch  disk  so that if you erase any information (by writing

over it) there will be no consequences.  For you who  don't  regularly

use  TECO  and  have  not  become intimate with its syntax and cryptic

error messages, do try to start to use TECO as  your  regular  editor.

It  is painful at first, but it and ODT.SAV (LINK ODT) are my favorite

tools for getting out of bad trouble.

Sincerely,

Ray Strackbein