Date:	2-MAR-82

To:	LX8BIT.SYS users  (serial line printer handler)

From:	Bob Frazer, 1523

Subj:	SET options

The LX8BIT handler is designed to be a serial line printer replacement

for the LP handler under RT11-SJ, and can be installed with a

COPY/SYS dev:LX8BIT.SAV name.SYS

followed by a reboot.  I would suggest using a dummy 'name' to get

the SET OPTIONS working before replacing your LP.SYS file.

SET name	result					default

-------		----------------------------------	-------

WIDTH=DDD	columns/line, decimal			132

CSR=000000	device RCSR address, octal		176500

VECTOR=000	device RCSR vector, octal		300

LENGTH=DD	lines/page, decimal			66

NULLS=DD	decimal NULLS after CR, for padding	10

TABS=DD		decimal spacing for soft tabs		8

and for column counting (TRUN, WRAP)

8 sets tabs at 9, 17, 25, etc.

(NO)HANG		hang, or abort, for hardware errors	HANG

(NO)EIGHT		eight-bit, or seven-bit			NOEIGHT

(NO)LC		lower-case, or not			LC

(NO)CTRL		pass control char's, or not		NOCTRL

(includes null char.)

(NO)TRUN		truncate lines greater than WIDTH,	TRUN

but do not advance line.  See Wrap.

printer generates

CRLF		print CR	   LF after CR

----		--------	-----------------

=00		   no			yes

=01		   no			no

=10		   yes			yes

=11		   yes			no		11

hardware	wrap - lines wider

TBWR		  tabs		than WIDTH force CRLF

----		--------	---------------------

=00		   no			no

=01		   no			yes

=10		   yes			no		10

=11		   yes			yes

hardware	form-feed at start of

FFB0		form-feed	print (block zero)

----		---------	---------------------

=00		   no			no

=01		   no			yes

=10		   yes			no

=11		   yes			yes		11

If no hardware FF, <CR, 6 LF's> substituted for FF.

To operate in a 'pass-all' mode (7 or 8 bit), set OPTIONS:

NULLS=0  LC  CTRL  NOTRUN  CRLF=11  TBWR=10  FFB0=10