TCFL: Terminal Control Function Library

by

S.C. CRIBBS

Pinawa, Manitoba

Canada  R0E1L0

Preliminary Document

ABSTRACT

TCFL  is  a  subroutine   library   that   provides   RT-11

programmers  with  a  convenient  method for controlling "smart"

terminals.  Fortran callable routines have been provided for the

issuing of most ANSI terminal control sequences and also for the

receipt and interpretation of response from the terminal.  RT-11

multi-terminal  or  single  console  I/O options can be selected

during program assembly.

TCFL: Terminal Control Function Library

TCFL is the collective name for a grouping  of  subroutines

which  are  used to control and monitor the operation of "smart"

terminals.  Smart in this context means terminals  that  respond

to   code   extension   techniques  defined  in  ANSI  standards

X3.41-1974 AND X3.64-1979 (ISO  2022  and  6429).   The  Digital

Equipment Corporation VT100 video and LA120 hardcopy families of

terminals support the  above  standard  control  techniques  and

additional DEC private control sequences.

This subroutine library provides RT-11 programmers  with  a

convenient  software  tool  for issuing control sequence strings

and the monitoring and interpretation of terminal response.  The

subroutines  are  written  in MACRO-11 to minimize memory usage,

but  have  been  designed  for  use  by   FORTRAN-IV   programs.

Specifically,  support  is provided for the ANSI mode control of

the VT100 series and VT125 video terminals and the LA120,  LA34,

LA100, and LA12 (Corespondent) hardcopy units.

Software assembly time options enable creation  of  subsets

of  the  control  function  subroutines  for  users  that do not

require the complete package.  The control strings generated  by

the  TCFL  routines  can be issued directly to the RT-11 program

console or optionally, through multi-terminal directives, to  an

alternate terminal or can be returned as ASCII character strings

in a user provided array.  Input from the console  terminal  (or

multi-terminal  unit)  may  be  routed through the input service

routines which  detect  and  interpret  ANSI  escape  sequences.

RT-11  monitor  timer features ensure minimal CPU usage when the

input   code   filter   routine    is    assembled    for    the

Foreground-Background  (FB)  or  Extended  Memory (XM) monitors.

Simulated timer watchdog code can be selected to  oversee  input

response  in  those  systems unable to support the RT-11 daytime

clock routine.

The TCFL  library  provides  the  basic  programming  tools

necessary   to   enable   the  creation  of  programs  with  the

sophistication of KED in handling cursor positioning and  keypad

interpretation.  I won't suggest that this software provides the

utility of FMS-11, but for many RT-11 applications simple screen

displays  controlled  directly  by  the  program  are  easier to

implement and require less memory.  As mentioned above, TCFL can

be  assembled  to  use  multi-terminal I/O.  Programs written to

perform all terminal communication through the TCFL  subroutines

can  be converted to multi-terminal use with just one subroutine

call.   Several  example  programs   have   been   provided   to

demonstrate TCFL:

Be sure to  SET TT: NOCRLF  before executing the demos!

------

Page 2

TCFL Demonstration Programs

1.  KTRMID  -  This  routine  solicits  and  interprets   a

terminal's  identification  string.   This  can be very

useful for applications that must  be  run  only  on  a

video   terminal!    File   TID.NTE,  included  in  the

distribution,  contains  the  identification  codes  of

commonly used DEC terminals.

2.  CURP - Demonstrates simple FORTRAN  control  of  screen

cursor position.

3.  KTCC - Shows that TCFL can properly interpret  any  key

code  or  character  sequence  sent  from  the terminal

keyboard.  In addition, KTCC demonstrates how to  write

an input interpretion routine using TCFL subroutines.

4.  SLVPLY - Uses the  TCFL  output  subroutines  to  build

screen  forms.   Rigid control of cursor positioning is

enforced and  each  character  input  by  the  user  is

subjected  to  validation  tests.   True,  the task the

program accomplishes is trivial, but it sure gave me an

appreciation  for  the finesse required if one wants to

attempt proper forms management.

5.  SLVPLM - Multi-terminal version of SLVPLY.  Just  start

it  from the consol;, it will prompt you for the number

of the  terminal  to  be  used.   Note  that  the  only

difference  in  the  FORTRAN program from SLVPLY is one

call  from  the  mainline  to  a  terminal   allocation

routine.

6.  SLVPLX - Differs from SLVPLY in that the  forms  to  be

displayed  are  stored  within  the  program  as  ASCII

strings.  SLVPLY generates each  form  as  required  by

calling  the  output  subroutines.   SLVPLX just routes

stored text to the terminal.  One look  at  the  actual

executable   code   required  in  this  version  should

convince  you  of  the  desirability  of  this  method.

Program  TXTWRT,  included  as  a  separate submission,

greatly simplifies forms design.

Page 3

The naming and descriptions of the  individual  subroutines

that  presently  make  up the TCFL library have been included in

the following appendices:

APPENDIX                   CONTENTS

A.   - All TCFL supported Terminal Control Output Commands

B.   - VT125 Terminal Control Commands subset

C.   - VT100 Terminal Control Commands subset

D.   - LA34 (LA12,LA100) Terminal Control Commands subset

E.   - LA120 Terminal Control Commands subset

F.   - Terminal Input and Utility subroutines

Considerable effort has been  expended  to  make  the  TCFL

files  useful  on  as  wide  a  range  of  terminal and computer

hardware  as  possible.   This  has  been  accomplished  through

extensive  use  of MACRO-11 conditional code.  The files must be

assembled with parameter file PTCFL.MAC which turns on  selected

options:

.ENABL LC

;		Conditional file for TCFL%%.MAC

;		Common Symbols

;	NHD	= 1	;Symbol should be defined for NHD mathematics

;		For TCFLO	- Output routines

ESC	= 33	;Define Escape Sequence Introducer

SZEM4	= 20.	;Length (bytes) of MSG4 buffer

VT$.$	= 1	;Define for default video terminal features

;	VT125	= 1	;Define for VT125 specific features

;	LA$.$	= 1	;Define for default hardcopy terminal features

;	LA34	= 1	;Define for LA34 specific features (LA12,LA100)

;		For TCFLIO	- Input - Output routines

;	ONLY$O	= 1	;Assemble only output code

;	MTT$Y	= 1	;Define for Multi-terminal I/O

;	USERBF	= 1	;Output routed to user's buffer

FBXM	= 1	;Monitor (FB or XM) supports .TWAIT

;	TIME$R	= 1	;Monitor provides .MRKT support (SJ)

The above configuration selects normal defaults  for  arithmetic

instructions  (EIS code), Escape Sequence introducer and maximum

length of generated output string.  Output will be routed to the

RT-11  program  console  (.PRINT) and will consist of only those

control codes recognized by the standard VT100 terminal.   Input

time-out  will  be  detected  through use of RT-11 FB/XM monitor

timer support routine ".TWAIT".

Page 4

Three  object  libraries  have  been  included  in  this

submission.  All three were built for RT-11 systems that support

atleast the EIS instruction set.  Should NHD code  be  required,

the  libraries  can  be rebuilt with the "NHD" symbol defined in

parameter file PTCFL.MAC.

1. TCFL.CSL   - I/O between computer and program console

Symbols ONLY$O, MTT$Y, USERBF not defined in PTCFL.MAC

.MACRO PTCFL+TCFLIO/OBJECT

.MACRO TCFLU1

.MACRO PTCFL+TCFLU2/OBJECT

.MACRO PTCFL+TCFLO/OBJECT

.LIBRARY/CREATE TCFL.CSL TCFLIO,TCFLU1,TCFLU2,TCFLO

2. TCFL.MTT   - I/O support for RT-11 multi-terminals

Symbols ONLY$O, USERBF not defined in PTCFL.MAC

Symbol MTT$Y defined

.MACRO PTCFL+TCFLIO/OBJECT

.MACRO TCFLU1

.MACRO PTCFL+TCFLU2/OBJECT

.MACRO PTCFL+TCFLO/OBJECT

.LIBRARY/CREATE TCFL.MTT TCFLIO,TCFLU1,TCFLU2,TCFLO

3.	TCFL.BFO   - Output only, to a user supplied text buffer

Symbol MTT$Y not defined in PTCFL.MAC

Symbols ONLY$O, USERBF defined

.MACRO PTCFL+TCFLIO/OBJECT

.MACRO PTCFL+TCFLO/OBJECT

.LIBRARY/CREATE TCFL.BFO TCFLIO,TCFLO

Note:  TCFL.BFO is used by program TXTWRT.SAV:  another related

submission to the 1983 DECUS Spring symposium

APPENDIX A

Alphabetical Index of All Terminal Control Commands

Alphabetical Index of All Terminal Control Commands             Page A-2

================================================================

Name	     Generated Code		Function Description

----------------------------------------------------------------

ALIGND		<"33>#8		-ALIGN Display (fill the text

screen with E's)

ANSI		<"33><		-set terminal to respond to ANSI

escape codes

BIGBOT		<"33>#4		-BIG BOTtom of double height and

width line

BIGTOP		<"33>#3		-BIG TOP of double height and

width line

CLRALL		<"33>[2J	-CLeaR ALL of the text screen

CLRBEG		<"33>[1K	-CLeaR the BEGinning of this

text line

CLRBOT		<"33>[0J	-CLeaR from cursor to BOTtom of

the screen

CLREND		<"33>[0K	-CLeaR from cursor to the END of

this text line

CLRLIN		<"33>[2K	-CLeaR this entire text LINe

CLRTOP		<"33>[1J	-CLeaR text from TOP of screen

to the cursor position

CRLF		<"15><"12>	-generate Carriage Return,

Line Feed sequence

CUB(m)		<"33>[~D	-move CUrsor Backward "m"

character positions

CUD(m)		<"33>[~B	-move CUrsor Down "m" lines

CUF(m)		<"33>[~C	-move CUrsor Forward "m"

character positions

CUP(m,n)	<"33>[~;~H	-CUrsor Position to line "m"

column "n"

CUU(m)		<"33>[~A	-move CUrsor Up "m" lines

DCS105		<"33>P0t	-Device Control String - emulate

VT105 graphics

DCS125		<"33>P0p	-Device Control String - enter

REGIS graphics

DCSDIF		<"33>P0q	-Device Control String - Delimit

Image Format

DWL		<"33>#6		-Double Width Line

GIMOAP		<"33>[?0i	-Graphics IMage Output to

Auxiliary Port

GIMOHC		<"33>[?2i	-Graphics IMage Output to Host

Computer

Alphabetical Index of All Terminal Control Commands             Page A-3

----------------------------------------------------------------

Name	     Generated Code		Function Description

----------------------------------------------------------------

GPOFF		<"33>2		-Graphics Processor OFF

GPON		<"33>1		-Graphics Processor ON

HPA(m)		<"33>[0`	-Horizontal Position Absolute

HPR(m)		<"33>[0a	-Horizontal Position Relative

HRDCPY		<"33>#7		-HaRDCoPY (Dump VT100 text

screen to printer)

HTBCLA		<"33>[3g	-Horizontal TaB CLear All

HTBCLC		<"33>[0g	-Horizontal TaB CLear at Current

position

HTBSTC		<"33>H		-Horizontal TaB SeT at Current

position

HTBSTS(m,n...)	<"33>[~;~;~;~u	-Horizontal TaB SeT at

Specified columns

IND		<"33>D		-INDex (move down one line,

maintain column)

KPAM		<"33>=		-KeyPad Application mode

KPNM		<"33>>		-KeyPad Numeric mode

LLEDS(m,n,o,p)	<"33>[~;~;~;~q	-Light specified LEDS (0 means

extinguish all)

LNEFDM		<"33>[20l	-LiNE FeeD Mode (<LF> generates

only <LF>)

MDCPY(m)	<"33>[~i	-Media CoPY mode option "m"

NEL		<"33>E		-NExt Line (move to beginning of

the next line)

NWLNEM		<"33>[20h	-NeW LiNE Mode (<LF> generates

<CR><LF>)

PLD		<"33>K		-Partial Line Down

PLU		<"33>L		-Partial Line Up

RESCUR		<"33>8		-REStore CURsor

RGSTRM		<"33>\		-ReGiS graphics mode terminator

RI		<"33>M		-Reverse Index (up one line,

maintain column)

RIS		<"33>c		-Reset to Initial State

RM(m)		<"33>[?0l	-Reset Mode for option "m"

Alphabetical Index of All Terminal Control Commands             Page A-4

----------------------------------------------------------------

Name	     Generated Code		Function Description

----------------------------------------------------------------

RPTCP		<"33>[6n	-RePorT Cursor Position

RPTDS		<"33>[5n	-RePorT Device Status

RPTID		<"33>[0c	-RePorT Terminal IDentification

code

SAVCUR		<"33>7		-SAVe CURsor

SCS0		<"17>		-Shift to Character Set G0

SCS1		<"16>		-Shift to Character Set G1

SCSAL1(m)	<"33>*1		-Select Character Set -

Alternate Standard set

SCSAL2(m)	<"33>*2		-Select Character Set -

Alternate Special set

SCSFIN(m)	<"33>*C		-Select Character Set - FINNISH

SCSFRA(m)	<"33>*R		-Select Character Set - FRENCH

(FRANCE)

SCSGER(m)	<"33>*K		-Select Character Set - GERMAN

SCSGRF(m)	<"33>*0		-Select Character Set - VT100

line graphics

SCSNOR(m)	<"33>*E		-Select Character Set -

NORWEGIAN

SCSQUE(m)	<"33>*Q		-Select Character Set - FRENCH

(Quebec)

SCSSWE(m)	<"33>*H		-Select Character Set - SWEDISH

SCSUK(m)	<"33>*A		-Select Character Set - ENGLISH

(BRITAIN)

SCSUSA(m)	<"33>*B		-Select Character Set - ENGLISH

(AMERICAN)

SETLPP(m)	<"33>[~t	-SET "m" Lines Per Page

SETLRM(m,n)	<"33>[~;~s	-SET Left (column "m") & Right

("n") Margins

SETTBM(m,n)	<"33>[~;~r	-SET Top (line "m") & Bottom

"n" Margins (scrolling region)

SGR(m,n,o,p)	<"33>[~;~;~;~m	-Select Graphic Rendition of

following chars.

SHPTCH(m)	<"33>[~w	-Set Horizontal PiTCH to

option "m"

SM(m)		<"33>[?~h	-Set Mode for option "m"

Alphabetical Index of All Terminal Control Commands             Page A-5

----------------------------------------------------------------

Name	     Generated Code		Function Description

----------------------------------------------------------------

SS2		<"33>N		-Single Shift 2 - select G2 set

for one character

SS3		<"33>O		-Single Shift 3 - select G3 set

for one character

SVPTCH(m)	<"33>[~z	-Set Vertical PiTCH to option "m"

SWL		<"33>#5		-Single Width Line

VPA(m)		<"33>[~d	-Vertical Position Absolute -

active line to "m"

VPR(m)		<"33>[~e	-Vertical Position Relative -

advance "m" lines

VTBCLA		<"33>[4g	-Vertical TaB CLear All

VTBCLC		<"33>[1g	-Vertical TaB CLear at Current

position

VTBSTC		<"33>J		-Vertical TaB SeT at Current

position

VTBSTS(m,n...)	<"33>[~;~;.;.v	-Vertical TaB SeT at Specified

lines "m,n..."

----------------------------------------------------------------

Notes:

1.  The symbol " ~  "  represents  the  ASCII  code  for  a

decimal numerical argument to a terminal command.

2.  Omitted or null  arguments  to  commands  that  require

arguments result in a zero character being generated in

the output string.  Argument values  too  large  for  a

particular   command  are  replaced  with  the  highest

acceptable value.  Excess arguments are ignored.   ANSI

protocol  does  not  limit the number of arguments that

may be passed to commands HTBSTS  &  VTBSTS.   However,

default  space  for  arguments  in the output string is

twenty characters, thus a  program  reassembly  may  be

necessary for special case applications.

3.  Arguments to the character set selection commands  (eg.

SCSUSA(m))  are  a  special  case.   The argument value

should be either zero or one.  If the argument is  zero

or  omitted,  the  "  * " in the generated code will be

replaced with a " ( " which designates  this  character

set  as the G0 set.  An argument value of one generates

the " ) " character which designates this character set

as the G1 set.

4.  Characters enclosed in angle parentheses  describe  the

octal value of the ASCII code generated.

APPENDIX B

Alphabetical Index of VT125 Terminal Control Commands

Alphabetical Index of VT125 Terminal Control Commands           Page B-2

================================================================

Name	     VT125 Generated Code	Function Description

----------------------------------------------------------------

ALIGND		<"33>#8		-ALIGN Display (fill the text

screen with E's)

ANSI		<"33><		-set terminal to respond to ANSI

escape codes

BIGBOT		<"33>#4		-BIG BOTtom of double height and

width line

BIGTOP		<"33>#3		-BIG TOP of double height and

width line

CLRALL		<"33>[2J	-CLeaR ALL of the text screen

CLRBEG		<"33>[1K	-CLeaR the BEGinning of this

text line

CLRBOT		<"33>[0J	-CLeaR from cursor to BOTtom of

the screen

CLREND		<"33>[0K	-CLeaR from cursor to the END of

this text line

CLRLIN		<"33>[2K	-CLeaR this entire text LINe

CLRTOP		<"33>[1J	-CLeaR text from TOP of screen

to the cursor position

CRLF		<"15><"12>	-generate Carriage Return,

Line Feed sequence

CUB(m)		<"33>[~D	-move CUrsor Backward "m"

character positions

CUD(m)		<"33>[~B	-move CUrsor Down "m" lines

CUF(m)		<"33>[~C	-move CUrsor Forward "m"

character positions

CUP(m,n)	<"33>[~;~H	-CUrsor Position to line "m"

column "n"

CUU(m)		<"33>[~A	-move CUrsor Up "m" lines

DCS105		<"33>P0t	-Device Control String - emulate

VT105 graphics

DCS125		<"33>P0p	-Device Control String - enter

REGIS graphics

DCSDIF		<"33>P0q	-Device Control String - Delimit

Image Format

DWL		<"33>#6		-Double Width Line

GIMOAP		<"33>[?0i	-Graphics IMage Output to

Auxiliary Port

GIMOHC		<"33>[?2i	-Graphics IMage Output to Host

Computer

Alphabetical Index of VT125 Terminal Control Commands           Page B-3

----------------------------------------------------------------

Name	     VT125 Generated Code	Function Description

----------------------------------------------------------------

GPOFF		<"33>2		-Graphics Processor OFF

GPON		<"33>1		-Graphics Processor ON

HPA(m)		<"33>[0`	-Horizontal Position Absolute

HPR(m)		<"33>[0a	-Horizontal Position Relative

HRDCPY		<"33>#7		-HaRDCoPY (Dump VT100 text

screen to printer)

HTBCLA		<"33>[3g	-Horizontal TaB CLear All

HTBCLC		<"33>[0g	-Horizontal TaB CLear at Current

position

HTBSTC		<"33>H		-Horizontal TaB SeT at Current

position

IND		<"33>D		-INDex (move down one line,

maintain column)

KPAM		<"33>=		-KeyPad Application mode

KPNM		<"33>>		-KeyPad Numeric mode

LLEDS(m,n,o,p)	<"33>[~;~;~;~q	-Light specified LEDS (0 means

extinguish all)

LNEFDM		<"33>[20l	-LiNE FeeD Mode (<LF> generates

only <LF>)

MDCPY(m)	<"33>[~i	-Media CoPY mode option "m"

NEL		<"33>E		-NExt Line (move to beginning of

the next line)

NWLNEM		<"33>[20h	-NeW LiNE Mode (<LF> generates

<CR><LF>)

RESCUR		<"33>8		-REStore CURsor

RGSTRM		<"33>\		-ReGiS graphics mode terminator

RI		<"33>M		-Reverse Index (up one line,

maintain column)

RIS		<"33>c		-Reset to Initial State

RM(m)		<"33>[?0l	-Reset Mode for option "m"

RPTCP		<"33>[6n	-RePorT Cursor Position

RPTDS		<"33>[5n	-RePorT Device Status

RPTID		<"33>[0c	-RePorT Terminal IDentification

code

Alphabetical Index of VT125 Terminal Control Commands           Page B-4

----------------------------------------------------------------

Name	     VT125 Generated Code	Function Description

----------------------------------------------------------------

SAVCUR		<"33>7		-SAVe CURsor

SCS0		<"17>		-Shift to Character Set G0

SCS1		<"16>		-Shift to Character Set G1

SCSAL1(m)	<"33>*1		-Select Character Set -

Alternate Standard set

SCSAL2(m)	<"33>*2		-Select Character Set -

Alternate Special set

SCSUK(m)	<"33>*A		-Select Character Set - ENGLISH

(BRITAIN)

SCSUSA(m)	<"33>*B		-Select Character Set - ENGLISH

(AMERICAN)

SETTBM(m,n)	<"33>[~;~r	-SET Top (line "m") & Bottom

"n" Margins (scrolling region)

SGR(m,n,o,p)	<"33>[~;~;~;~m	-Select Graphic Rendition of

following chars.

SM(m)		<"33>[?~h	-Set Mode for option "m"

SS2		<"33>N		-Single Shift 2 - select G2 set

for one character

SS3		<"33>O		-Single Shift 3 - select G3 set

for one character

SWL		<"33>#5		-Single Width Line

----------------------------------------------------------------

Notes:

1.  The symbol " ~  "  represents  the  ASCII  code  for  a

decimal numerical argument to a terminal command.

2.  Omitted or null  arguments  to  commands  that  require

arguments result in a zero character being generated in

the output string.  Argument values  too  large  for  a

particular   command  are  replaced  with  the  highest

acceptable value.  Excess arguments are ignored.

3.  Arguments to the character set selection commands  (eg.

SCSUSA(m))  are  a  special  case.   The argument value

should be either zero or one.  If the argument is  zero

or  omitted,  the  "  * " in the generated code will be

replaced with a " ( " which designates  this  character

set  as the G0 set.  An argument value of one generates

the " ) " character which designates this character set

as the G1 set.

4.  Characters enclosed in angle parentheses  describe  the

octal value of the ASCII code generated.

APPENDIX C

Alphabetical Index of VT100 Terminal Control Commands

Alphabetical Index of VT100 Terminal Control Commands           Page C-2

================================================================

Name	     VT100 Generated Code	Function Description

----------------------------------------------------------------

ALIGND		<"33>#8		-ALIGN Display (fill the text

screen with E's)

ANSI		<"33><		-set terminal to respond to ANSI

escape codes

BIGBOT		<"33>#4		-BIG BOTtom of double height and

width line

BIGTOP		<"33>#3		-BIG TOP of double height and

width line

CLRALL		<"33>[2J	-CLeaR ALL of the text screen

CLRBEG		<"33>[1K	-CLeaR the BEGinning of this

text line

CLRBOT		<"33>[0J	-CLeaR from cursor to BOTtom of

the screen

CLREND		<"33>[0K	-CLeaR from cursor to the END of

this text line

CLRLIN		<"33>[2K	-CLeaR this entire text LINe

CLRTOP		<"33>[1J	-CLeaR text from TOP of screen

to the cursor position

CRLF		<"15><"12>	-generate Carriage Return,

Line Feed sequence

CUB(m)		<"33>[~D	-move CUrsor Backward "m"

character positions

CUD(m)		<"33>[~B	-move CUrsor Down "m" lines

CUF(m)		<"33>[~C	-move CUrsor Forward "m"

character positions

CUP(m,n)	<"33>[~;~H	-CUrsor Position to line "m"

column "n"

CUU(m)		<"33>[~A	-move CUrsor Up "m" lines

DWL		<"33>#6		-Double Width Line

GPOFF		<"33>2		-Graphics Processor OFF

GPON		<"33>1		-Graphics Processor ON

HRDCPY		<"33>#7		-HaRDCoPY (Dump VT100 text

screen to printer)

HTBCLA		<"33>[3g	-Horizontal TaB CLear All

HTBCLC		<"33>[0g	-Horizontal TaB CLear at Current

position

HTBSTC		<"33>H		-Horizontal TaB SeT at Current

position

IND		<"33>D		-INDex (move down one line,

maintain column)

KPAM		<"33>=		-KeyPad Application mode

KPNM		<"33>>		-KeyPad Numeric mode

Alphabetical Index of VT100 Terminal Control Commands           Page C-3

----------------------------------------------------------------

Name	     VT100 Generated Code	Function Description

----------------------------------------------------------------

LLEDS(m,n,o,p)	<"33>[~;~;~;~q	-Light specified LEDS (0 means

extinguish all)

LNEFDM		<"33>[20l	-LiNE FeeD Mode (<LF> generates

only <LF>)

NEL		<"33>E		-NExt Line (move to beginning of

the next line)

NWLNEM		<"33>[20h	-NeW LiNE Mode (<LF> generates

<CR><LF>)

RESCUR		<"33>8		-REStore CURsor

RI		<"33>M		-Reverse Index (up one line,

maintain column)

RIS		<"33>c		-Reset to Initial State

RM(m)		<"33>[?0l	-Reset Mode for option "m"

RPTCP		<"33>[6n	-RePorT Cursor Position

RPTID		<"33>[0c	-RePorT Terminal IDentification

code

SAVCUR		<"33>7		-SAVe CURsor

SCS0		<"17>		-Shift to Character Set G0

SCS1		<"16>		-Shift to Character Set G1

SCSAL1(m)	<"33>*1		-Select Character Set -

Alternate Standard set

SCSAL2(m)	<"33>*2		-Select Character Set -

Alternate Special set

SCSGRF(m)	<"33>*0		-Select Character Set - VT100

line graphics

SCSUK(m)	<"33>*A		-Select Character Set - ENGLISH

(BRITAIN)

SCSUSA(m)	<"33>*B		-Select Character Set - ENGLISH

(AMERICAN)

SETTBM(m,n)	<"33>[~;~r	-SET Top (line "m") & Bottom

"n" Margins (scrolling region)

SGR(m,n,o,p)	<"33>[~;~;~;~m	-Select Graphic Rendition of

following chars.

SM(m)		<"33>[?~h	-Set Mode for option "m"

SWL		<"33>#5		-Single Width Line

----------------------------------------------------------------

Alphabetical Index of VT100 Terminal Control Commands           Page C-4

Notes:

1.  The symbol " ~  "  represents  the  ASCII  code  for  a

decimal numerical argument to a terminal command.

2.  Omitted or null  arguments  to  commands  that  require

arguments result in a zero character being generated in

the output string.  Argument values  too  large  for  a

particular   command  are  replaced  with  the  highest

acceptable value.  Excess arguments are ignored.

3.  Arguments to the character set selection commands  (eg.

SCSUSA(m))  are  a  special  case.   The argument value

should be either zero or one.  If the argument is  zero

or  omitted,  the  "  * " in the generated code will be

replaced with a " ( " which designates  this  character

set  as the G0 set.  An argument value of one generates

the " ) " character which designates this character set

as the G1 set.

4.  Characters enclosed in angle parentheses  describe  the

octal value of the ASCII code generated.

APPENDIX D

Alphabetical Index of LA34 Terminal Control Commands

Alphabetical Index of LA34 Terminal Control Commands            Page D-2

================================================================

Name	     LA34 Generated Code	Function Description

----------------------------------------------------------------

CRLF		<"15><"12>	-generate Carriage Return,

Line Feed sequence

CUU(m)		<"33>[~A	-move CUrsor Up "m" lines

DCSDIF		<"33>P0q	-Device Control String - Delimit

Image Format

HPA(m)		<"33>[0`	-Horizontal Position Absolute

HPR(m)		<"33>[0a	-Horizontal Position Relative

HTBCLA		<"33>[3g	-Horizontal TaB CLear All

HTBCLC		<"33>[0g	-Horizontal TaB CLear at Current

position

HTBSTC		<"33>H		-Horizontal TaB SeT at Current

position

HTBSTS(m,n...)	<"33>[~;~;~;~u	-Horizontal TaB SeT at

Specified columns

IND		<"33>D		-INDex (move down one line,

maintain column)

KPAM		<"33>=		-KeyPad Application mode

KPNM		<"33>>		-KeyPad Numeric mode

LNEFDM		<"33>[20l	-LiNE FeeD Mode (<LF> generates

only <LF>)

NEL		<"33>E		-NExt Line (move to beginning of

the next line)

NWLNEM		<"33>[20h	-NeW LiNE Mode (<LF> generates

<CR><LF>)

PLD		<"33>K		-Partial Line Down

PLU		<"33>L		-Partial Line Up

RGSTRM		<"33>\		-ReGiS graphics mode terminator

RI		<"33>M		-Reverse Index (up one line,

maintain column)

RPTID		<"33>[0c	-RePorT Terminal IDentification

code

Alphabetical Index of LA34 Terminal Control Commands            Page D-3

----------------------------------------------------------------

Name	     LA34 Generated Code	Function Description

----------------------------------------------------------------

SCS0		<"17>		-Shift to Character Set G0

SCS1		<"16>		-Shift to Character Set G1

SCSFIN(m)	<"33>*C		-Select Character Set - FINNISH

SCSFRA(m)	<"33>*R		-Select Character Set - FRENCH

(FRANCE)

SCSGER(m)	<"33>*K		-Select Character Set - GERMAN

SCSGRF(m)	<"33>*0		-Select Character Set - VT100

line graphics

SCSNOR(m)	<"33>*E		-Select Character Set -

NORWEGIAN

SCSQUE(m)	<"33>*Q		-Select Character Set - FRENCH

(Quebec)

SCSSWE(m)	<"33>*H		-Select Character Set - SWEDISH

SCSUK(m)	<"33>*A		-Select Character Set - ENGLISH

(BRITAIN)

SCSUSA(m)	<"33>*B		-Select Character Set - ENGLISH

(AMERICAN)

SETLPP(m)	<"33>[~t	-SET "m" Lines Per Page

SETLRM(m,n)	<"33>[~;~s	-SET Left (column "m") & Right

("n") Margins

SETTBM(m,n)	<"33>[~;~r	-SET Top (line "m") & Bottom

"n" Margins (scrolling region)

SHPTCH(m)	<"33>[~w	-Set Horizontal PiTCH to

option "m"

SVPTCH(m)	<"33>[~z	-Set Vertical PiTCH to option "m"

VPA(m)		<"33>[~d	-Vertical Position Absolute -

active line to "m"

VPR(m)		<"33>[~e	-Vertical Position Relative -

advance "m" lines

VTBCLA		<"33>[4g	-Vertical TaB CLear All

VTBCLC		<"33>[1g	-Vertical TaB CLear at Current

position

VTBSTC		<"33>J		-Vertical TaB SeT at Current

position

VTBSTS(m,n...)	<"33>[~;~;.;.v	-Vertical TaB SeT at Specified

lines "m,n..."

----------------------------------------------------------------

Alphabetical Index of LA34 Terminal Control Commands            Page D-4

Notes:

1.  The symbol " ~  "  represents  the  ASCII  code  for  a

decimal numerical argument to a terminal command.

2.  Omitted or null  arguments  to  commands  that  require

arguments result in a zero character being generated in

the output string.  Argument values  too  large  for  a

particular   command  are  replaced  with  the  highest

acceptable value.  Excess arguments are ignored.   ANSI

protocol  does  not  limit the number of arguments that

may be passed to commands HTBSTS  &  VTBSTS.   However,

default  space  for  arguments  in the output string is

twenty characters, thus a  program  reassembly  may  be

necessary for special case applications.

3.  Arguments to the character set selection commands  (eg.

SCSUSA(m))  are  a  special  case.   The argument value

should be either zero or one.  If the argument is  zero

or  omitted,  the  "  * " in the generated code will be

replaced with a " ( " which designates  this  character

set  as the G0 set.  An argument value of one generates

the " ) " character which designates this character set

as the G1 set.

4.  Characters enclosed in angle parentheses  describe  the

octal value of the ASCII code generated.

APPENDIX E

Alphabetical Index of LA120 Terminal Control Commands

Alphabetical Index of LA120 Terminal Control Commands           Page E-2

================================================================

Name	     LA120 Generated Code	Function Description

----------------------------------------------------------------

CRLF		<"15><"12>	-generate Carriage Return,

Line Feed sequence

HPA(m)		<"33>[0`	-Horizontal Position Absolute

HPR(m)		<"33>[0a	-Horizontal Position Relative

HTBCLA		<"33>[3g	-Horizontal TaB CLear All

HTBCLC		<"33>[0g	-Horizontal TaB CLear at Current

position

HTBSTC		<"33>H		-Horizontal TaB SeT at Current

position

HTBSTS(m,n...)	<"33>[~;~;~;~u	-Horizontal TaB SeT at

Specified columns

IND		<"33>D		-INDex (move down one line,

maintain column)

KPAM		<"33>=		-KeyPad Application mode

KPNM		<"33>>		-KeyPad Numeric mode

LNEFDM		<"33>[20l	-LiNE FeeD Mode (<LF> generates

only <LF>)

NEL		<"33>E		-NExt Line (move to beginning of

the next line)

NWLNEM		<"33>[20h	-NeW LiNE Mode (<LF> generates

<CR><LF>)

RPTID		<"33>[0c	-RePorT Terminal IDentification

code

SCS0		<"17>		-Shift to Character Set G0

SCS1		<"16>		-Shift to Character Set G1

SCSFIN(m)	<"33>*C		-Select Character Set - FINNISH

SCSFRA(m)	<"33>*R		-Select Character Set - FRENCH

(FRANCE)

SCSGER(m)	<"33>*K		-Select Character Set - GERMAN

SCSGRF(m)	<"33>*0		-Select Character Set - VT100

line graphics

SCSNOR(m)	<"33>*E		-Select Character Set -

NORWEGIAN

SCSSWE(m)	<"33>*H		-Select Character Set - SWEDISH

SCSUK(m)	<"33>*A		-Select Character Set - ENGLISH

(BRITAIN)

SCSUSA(m)	<"33>*B		-Select Character Set - ENGLISH

(AMERICAN)

SETLPP(m)	<"33>[~t	-SET "m" Lines Per Page

SETLRM(m,n)	<"33>[~;~s	-SET Left (column "m") & Right

("n") Margins

Alphabetical Index of LA120 Terminal Control Commands           Page E-3

----------------------------------------------------------------

Name	     LA120 Generated Code	Function Description

----------------------------------------------------------------

SETTBM(m,n)	<"33>[~;~r	-SET Top (line "m") & Bottom

"n" Margins (scrolling region)

SHPTCH(m)	<"33>[~w	-Set Horizontal PiTCH to

option "m"

SVPTCH(m)	<"33>[~z	-Set Vertical PiTCH to option "m"

VPA(m)		<"33>[~d	-Vertical Position Absolute -

active line to "m"

VPR(m)		<"33>[~e	-Vertical Position Relative -

advance "m" lines

VTBCLA		<"33>[4g	-Vertical TaB CLear All

VTBCLC		<"33>[1g	-Vertical TaB CLear at Current

position

VTBSTC		<"33>J		-Vertical TaB SeT at Current

position

VTBSTS(m,n...)	<"33>[~;~;.;.v	-Vertical TaB SeT at Specified

lines "m,n..."

----------------------------------------------------------------

Notes:

1.  The symbol " ~  "  represents  the  ASCII  code  for  a

decimal numerical argument to a terminal command.

2.  Omitted or null  arguments  to  commands  that  require

arguments result in a zero character being generated in

the output string.  Argument values  too  large  for  a

particular   command  are  replaced  with  the  highest

acceptable value.  Excess arguments are ignored.   ANSI

protocol  does  not  limit the number of arguments that

may be passed to commands HTBSTS  &  VTBSTS.   However,

default  space  for  arguments  in the output string is

twenty characters, thus a  program  reassembly  may  be

necessary for special case applications.

3.  Arguments to the character set selection commands  (eg.

SCSUSA(m))  are  a  special  case.   The argument value

should be either zero or one.  If the argument is  zero

or  omitted,  the  "  * " in the generated code will be

replaced with a " ( " which designates  this  character

set  as the G0 set.  An argument value of one generates

the " ) " character which designates this character set

as the G1 set.

4.  Characters enclosed in angle parentheses  describe  the

octal value of the ASCII code generated.

APPENDIX F

Terminal input and Utility Subroutines

Terminal input and Utility Subroutines                          Page F-2

1.  +++++++++++++++++++++++++

Look for Escape Sequences

Called by FORTRAN:	I = LESCSQ(STRING)

where	STRING	- Character string which returns received input

I	- -1  = Time out (no response received)

0  = Invalid Escape sequence detected

>0  = Number of characters in Escape sequence

NOTE:  When an invalid character sequence is detected,

all characters preceeding and the offending character are

returned in ASCIZ format in vector STRING.  If only an <ESCAPE>

character is received before timeout occurs, the <ESCAPE>

character will be returned as an ASCIZ string in STRING and an

invalid sequence detected code will be returned.  If no input

at all is received, STRING will contain a null string.  The

timeout period has been defined (in TCFLIO.MAC) as 0.2 seconds

for those systems supporting .TWAIT (FB or XM monitors) or

.MRKT under the SJ monitor.  For systems with no LTC, timeout

is a simulated 1 second (on a LSI-11/2), but is easily modified

before program assembly.

2.  +++++++++++++++++++++++++

Determine terminal's identity

Called from FORTRAN by:

I = KTRMID(NPARM,IPARM,LPRVTE)

where:

NPARM	- Maximum number of parameters to return

IPARM	- Integer vector to contain parameter values

LPRVTE	- -1 if private sequence found, else zero

I	- Returned funtion value

= -1	- Timeout (No response from terminal)

=  0	- Unrecognized response received

= >0	- The number of parameters available

3.  +++++++++++++++++++++++++

Get the video terminal's present cursor position

Called from FORTRAN by:

I = KCURP(IROW,ICOL)

where:

IROW	- Cursor position row number

ICOLM	-  & column number

I	- Returned value

= -1	- Timeout (No response from terminal)

=  0	- Unrecognized response received

=  1	- Proper format response recived

Terminal input and Utility Subroutines                          Page F-3

4.  +++++++++++++++++++++++++

KITCC	- Decodes Keypad and Cursor positioning key input

Note:  Terminal must have Keypad Application Mode set for

keypad keys (except PF1->PF4) to be detected.

ANSI Cursor Key Mode must be set for cursor

positioning keys to be detected.

Called from FORTRAN by:

I = KITCC(INCHAR,ICODE)

where:

INCHAR	- Provided by user, the termination character of

the just received Escape sequence

ICODE	- Integer returned, the numeric value of the received

terminal command key

-if numeric or punctuation, ICODE will

contain the actual character code:

','	= "54	44 decimal

'-'	= "55	45

'.'	= "56	46

'0'	= "60	48

'1'	= "61	49

'2'	= "62	50

'3'	= "63	51

'4'	= "64	52

'5'	= "65	53

'6'	= "66	54

'7'	= "67	55

'8'	= "70	56

'9'	= "71	57

-if Cursor control, Programmable function or

the <ENTER> key

Up	= "1	 1 decimal

Down	= "2	 2

Right	= "3	 3

Left	= "4	 4

<ENTER>	= "15	13

PF1	= "20	16

PF2	= "21	17

PF3	= "22	18

PF4	= "23	19

- or if an error or unrecognized character is detected

the ASCII character actually received

I	- Returned function value

= -1	- Timeout or unrecognized input

=  0	- Numeric or puntuation command

=  1	- Cursor control, Programmable function or

the <ENTER> key pressed

.END  LITERAL

.PAGE

.LITERAL

5.  +++++++++++++++++++++++++

Define the multi-terminal port to be used for I/O

Called from FORTRAN by:

Terminal input and Utility Subroutines                          Page F-4

CALL LUNTRM(NPORT)	<Symbol MTT$Y=1>

where:

NPORT	- The number of the multi-terminal port to be used.

Note:	The calling program must request use of the terminal port

from the monitor [CALL MTATCH(?)] before calling LUNTRM.

6.  +++++++++++++++++++++++++

Define address of user's buffer for TCFLO's returned strings

Called from FORTRAN by:

CALL DFNBUF(BUFFER)	<Symbol USERBF=1>

where:

BUFFER	- is a FORTRAN vector of appropriate length to handle

the maximum string length to be returned.

81 bytes is a nice round number.

7.  +++++++++++++++++++++++++

Device "somewhat" independent ASCIZ string output subroutine

Called from FORTRAN by:

CALL OUTTZ(STRING)	!Append <CR><LF>

or

CALL OUTTZN(STRING)	!Do not append <CR><LF>

where

STRING	- is an ASCIZ character string (ends with a null).

8.  +++++++++++++++++++++++++

Empty the terminal's input buffer

Called by FORTRAN:	CALL FLUSHB

9.  +++++++++++++++++++++++++

Device "somewhat" independent routine to define terminal's

operating characteristics.

Called from FORTRAN by:

CALL TTMDE(IARG)

where:

IARG	- Defines Terminal features wanted

= 0	- Uppercase only, line mode, wait

= 1	- Enable lowercase, line mode, wait

= 2	- Uppercase only, Single Character, wait

= 3	- Enable lowercase, Single Character, wait

= 4	- Uppercase only, line mode, no wait ???

= 5	- Enable lowercase, line mode, no wait ???

= 6	- Uppercase only, Single Character, no wait

= 7	- Enable lowercase, Single Character, no wait

Note:	IARG values of 3 or 4 are impractical - but may be set.