PURPOSE OF THIS DOCUMENT

This documentation is intended for anyone who desires to add a

graphics device driver to DIGLIB.  It is assumed that the reader is

proficient in FORTRAN or MACRO and has some knowledge of graphics.  In

addition, the reader should become intimately familiar with the target

graphics device.

THE COMMERCIAL.

DIGLIB is a free product brought to you by Hal R.  Brand.

courtesy of Jack Frazer and Ted Michels.  Since DIGLIB was provided to

you free, it is my sincere hope that DIGLIB device drivers will be

freely circulated.  To aid in this effort, and to provide a central

distribution facility, I request that debugged DIGLIB device drivers

be fowarded to Hal Brand, L308 x26312 for distribution and

maintenance.  Yes, that is right!  You write it, I'll distribute and

maintain it to the best of my ability and available time.  Please send

a floppy (single or dual density) with the graphic driver to Hal Brand

L308.  I will make a copy and return your floppy ASAP.  Note also,

that before embarking on an adventure through graphic device driver

land, it will probably be very useful to check with Hal Brand x26312

to see if such a driver already exists, or whether there is a very

similar driver which can be modified.  (It is nearly always easier to

make minor modifications than to start from scratch.) END OF

COMMERCIAL.

GUIDE TO WRITING A DIGLIB GRAPHICS DRIVER                                 Page 2

INTRODUCTION

INTRODUCTION

DIGLIB was written to be as device independent as possible.

Accordingly, it was assumed that the graphics device was only capable

of performing two simple graphics primitives:  move and draw.  DIGLIB

does, however, provide for some "extended" primitives such as:  set

color, graphics input, and define color.  Because DIGLIB only uses

move and draw, authors of graphics device drivers need not worry about

character generation, line types, etc.  DIGLIB performs these

functions using only move and draw.

DIGLIB graphics device drivers basically perform two functions:

1.  Formatting of graphics commands, and

2.  Outputing of the formatted graphics commands.

Formatting of graphics commands is very device specific, and will not

be discussed here.  See the programming manual for your graphics

device for this information.

Output of the formatted graphics commands is, however, common

among all DIGLIB drivers in the sense that all DIGLIB drivers either

handle the I/O themselves, or pass of the formatted graphics commands

to the operating system to be output.  All DIGLIB graphics device

drivers that drive graphic terminals use the terminal I/O capability

of the operating system to perform the output operation.  All the

remaining RSX-11M graphics device drivers use the QIO system call to

request RSX-11M to perform the output operation using an existing

RSX-11M device driver.  Thus, those persons desiring to add a graphics

device to DIGLIB under RSX-11M must have previously written, or

obtained an appropriate RSX-11M device driver for the target graphics

device.  Note:  a general purpose RSX-11M serial line driver is

available from Hal R.  Brand x26313.

Most RT-11 graphics device drivers handle the I/O function

themselves.  This avoids the need to obtain or write a RT-11 device

driver.

GUIDE TO WRITING A DIGLIB GRAPHICS DRIVER                                 Page 3

STANDARD DEVICE DRIVER FUNCTIONS

STANDARD DEVICE DRIVER FUNCTIONS

DIGLIB device drivers must perform 7 basic graphics primitives:

1.  Initialize the graphics device and prepare it for graphics

commands.

2.  Prepare a fresh plotting surface on the graphics device and

reset the drawing parameters to the "initial state".

3.  Move from the current (X,Y) position to a new (X,Y) position

without drawing a line.

4.  Draw a line from the current (X,Y) position to a new (X,Y)

position (in the current color).

5.  Transmit any graphics commands formatted, but not yet output

to the graphics device.

6.  Release the graphics device.

7.  Return to the caller information about this graphics device

(Device Characteristics Values).

INITIALIZE - function 1

To initialize a graphics device, the graphics driver should first

take exclusive control of that device on a multi-user system.  Then

the device should be sent commands to prepare the device for the

commands that the graphics driver will generate.  Commands should also

be included to set any special parameters (i.e.  line types) the

device may have to the proper values.  If possible, the initialize

function should not erase the screen.  This will allow for multiple

programs to all draw on the same "drawing surface".  Sometimes, to

initialize the device requires the screen to be erased.  If that is

the case, just document it as a feature.

On color devices, initialize should set the desired background

color and foreground color for future drawing if possible.  In

addition, for devices with definable color sets, initialize should set

color 2 to be red, color 3 to be green, color 4 to be blue, color 5 to

be yellow, color 6 to be magenta, and color 7 to be cyan.  Unless you

definitely want a different foreground/background color scheme, color

0 (background) should be set to black, and color 1 (foreground) to

white.

Initialize is also responsible for returning an error indicator

when the device can not be selected, or some other problem exists.

GUIDE TO WRITING A DIGLIB GRAPHICS DRIVER                                 Page 4

STANDARD DEVICE DRIVER FUNCTIONS

BEGIN PLOT - function 2

The begin plot function should erase the screen or (more

generally) prepare a fresh plotting surface on the graphics device

(i.e.  advance the page on a plotter, etc.).  In addition, it should

set colors 0 to 7 to their proper default values (black, white, red,

green, blue, yellow, magenta, cyan respectively).  It should also set

the current drawing color to color 1.  Care should be taken to make

sure that the driver's perception of the state of the device remains

consistant with the device's state.  Usually, it is necessary to

either force out any commands left in the command buffer (if one is

maintained) or to send commands/set state memory variables to make

sure that consistancy is maintained.  For example, if the driver

remembers the current position that the device is going to draw from,

it is typically updated when a command is buffered for the device.  If

this buffer is discarded instead of being sent to the device by BGNPLT

(as is done in some DIGLIB drivers), then care must be taken to ensure

that any possible device states that the driver perceived as changed

but really weren't are made consistant!

MOVE and DRAW - functions 3 and 4

DIGLIB always works in centimeter coordinates as far as the graphics

device drivers are concerned.  Thus all coordinates that flow into or

out of the graphics device driver are in centimeters.  Also, DIGLIB

performs clipping of all coordinates based on the device's Device

Characteristics Values, eliminating all coordinates that lie outside

the graphics devices accessable area.

ENDPLT - function 5

It is often desirable to buffer the graphics commands before

sending them to the graphics device.  In fact, a good size buffer will

usually increase the graphics throughput because of the overhead of

the operating system for I/O.  The problem with buffering graphics

commands is that some will almost always remain in the buffer after

the last graphics subroutine has been called and the plot will be

incomplete.  DIGLIB solves this problem by requiring the user to call

ENDPLT.  ENDPLT then calls the graphics driver with function code 5.

The graphics driver should then send out the last buffer (even though

it is not yet full).

RELEASE DEVICE - function 6

Release device is the antithesis to initialize device.  The

function of release device is to return the graphics device to its

initial state, or atleast to something close to its initial state.

Release device should not erase the graphics (if possible).

GUIDE TO WRITING A DIGLIB GRAPHICS DRIVER                                 Page 5

STANDARD DEVICE DRIVER FUNCTIONS

RETURN DEVICE CHARACTERISTICS - function 7

The definitions of each of the 8 device characteristics values is

given in appendix A.  The 4 values "X axis length", "Y axis length",

"X axis resolution", and "Y axis resolution" are simple to determine

by consulting the graphics device specifications, or by measurement

and calculation.  However, when using the measurement and calculation

method, the following pitfall exists:

1.  Because the graphics device is out of adjustment, the X

resolution and the Y resolution appear different, and

2.  Due to roundoff and measurement errors, the product of the

resolution times the screen size when rounded is greater than

the device's maximum addressable point.

The problem with 1) is that when a 45 degree line is drawn, it

will truly be drawn at 45 degrees, but the user will not think so

because the line is jagging every once in a while along its length.

My solution to this problem is to set the "X axis resolution" and "Y

axis resolution" to the same value unless it is specifically known

that they should be different.  Lines drawn at 45 degrees may not be

exactly 45 degrees, but the user will think so, and so you are not

plagued by users thinking the driver is sick!

The problem with 2) is that lines drawn at the supposed right

and/or top of the screen get clipped, or worse, the device driver will

get passed a coordinate that is outside the device's addressable

range.

The solution to this problem is to average the X and Y resolution

to get a single resolution for both X and Y unless it is specifically

known that the X and Y resolution a different.  Then, using the

device's maximum addressable points in X and Y, and the resolution,

back calculate the axis lengths.

Example:

Measurement yields an X axis length of 15.7 centimeters and a

Y axis length of 10.4 cm.  The device coordinate ranges are 0

to 639 in X, and 0 to 419 in Y.  So:

X axis resolution is 40.7 graphic units per centimeter Y

axis resolution is 40.3 graphic units per cm.

averaging, we get 40.5 GU/CM.

Thus, we set:

X axis resolution = 40.5 GU/CM.

Y axis resolution = 40.5 GU/CM.

X axis length = 639/40.5 = 15.7778 CM.

Y axis length = 419/40.5 = 10.3457 CM.

The 8th returned value also requires some discussion.  This value

is new to DIGLIB V3 and its support of hardware/software polygon fill.

For devices that do not support solid filled polygons, DIGLIB's

polygon primitive GSPOLY contains a software polygon fill algorithm.

The algorithm can handle any polygon (but maybe not the way you want -

intersecting polygons are of dubious value in my opinion).  The 8th

GUIDE TO WRITING A DIGLIB GRAPHICS DRIVER                                 Page 6

STANDARD DEVICE DRIVER FUNCTIONS

device characteristic value only pertains to the software fill

algorithm.  It tells the software fill algorithm just how dense to

draw the horizontal lines that fill the polygon.  You should choose

this value via trial and error.  I suggest choosing it large and

working down.  The way to get a first guess is to estimate the width

of the plotter pen in resolution (graphics) units.  Thus, for a pen

that drew a line 2 mm.  thick, on a plotter with 0.5 mm.  per graphics

unit, a good first guess would be 2.0/0.5 = 4.  Thus, drawing every

4th line (skipping 3) should still produce an acceptable solid fill.

Note that this parameter should always be one (1) when the size of the

(virtual) pen is 1 graphic unit as is the case with all raster scan

devices.

Since this 8th device characteristic value is new, DEVSEL will

automatically default it to be 1 if the device driver returns only 7

values.  However, this is a backward compatibility feature, and all

new DIGLIB device drivers should return all 8 values!

GUIDE TO WRITING A DIGLIB GRAPHICS DRIVER                                 Page 7

EXTENDED DIGLIB DEVICE DRIVER FUNCTIONS

EXTENDED DIGLIB DEVICE DRIVER FUNCTIONS

In addition to the 7 basic graphic primitives, DIGLIB defines 6

extended functions that you may optionally implement.  DIGLIB graphics

device drivers may implement none, all, or any combination of these

functions.

1.  8) Set current drawing color or intensity.

2.  9) Graphics input with pick character.

3.  10) Define a color numbers visual value using RGB

coordinates.

4.  11) Define a color numbers visual value using HLS

coordinates.

5.  12) Graphics input with device dependant "botton" input.

6.  1024+N) Draw a solid filled polygon of N sides

I suggest implementing 8) and either 9) or 12) if possible.

DIGLIB has a software polygon fill algorithm, so you can get filled

polygons regardless of whether you implement the polygon fill section.

However, hardware (firmware) fills are typically much faster.

SET COLOR - function 8

DIGLIB uses numbers to express the color it wants to use.  The

DIGLIB color convention is:

0       is the background color (usually black)

1       is the primary foreground color (usually white if possible)

2==>n   are the secondary foreground colors

The default colors are:

0       Black                   4       Blue

1       White                   5       Yellow

2       Red                     6       Magenta

3       Green                   7       Cyan

If your graphics device has fixed colors, then the graphics driver

should map the DIGLIB colors numbers into the graphics devices color

numbers so that the proper color correspondence exists.

GRAPHICS INPUT WITH PICK CHARACTER - function 9

This function requires a keyboard, since it must return a "pick

character".  This is no problem with graphics terminals, but with

graphics heads, there is often no keyboard for getting the "pick

character".  This can be handled by using the interactive terminal

keyboard.  I admit this is a kludge, but there seems no alternative.

GUIDE TO WRITING A DIGLIB GRAPHICS DRIVER                                 Page 8

EXTENDED DIGLIB DEVICE DRIVER FUNCTIONS

To perform a graphics input with pick character operation, the

following steps are necessary:

1.  Make the graphic cursor visible if it is not always visible,

2.  Allow the user to position the graphic cursor,

3.  When the user strikes a character at the graphics keyboard,

read the graphic cursor position,

4.  Return the character struck, the X position (in cm.) and the

Y position (in cm.) of the graphic cursor, and

5.  Make the graphic cursor invisible if possible.

If your graphic device does not have a keyboard, then the user's

terminal is the logical choice.

DEFINE COLOR USING RGB - function 10

DIGLIB provides a mechanism for selecting the color associated

with by DIGLIB color number using the (Red, Green, Blue) color

coordinate system.  Most devices do not allow the color associated

with a particular color number to be defined.  However, some devices

such as the Lexidata 3400, Ramtek 9400, AED 512, etc.  have this

option.  DIGLIB uses a percentage of full intensity for each of the

three colors.  Thus, the graphics driver receives the color number and

three numbers between 0 and 100.  These percentages will probably have

to be converted to the units your particular graphics device wants.

DEFINE COLOR USING HLS - function 11

DIGLIB also provides a mechanism for selecting the color

associated with any DIGLIB color number using the (Hue, Lightness,

Saturation) color coordinate system.  For more information concerning

the HLS color system, see the SIGGRAPH core standard manuals, or see

the Tektronix 4027 literature.  There are also numerous other sources

of information on the HLS color system.

DIGLIB uses the standard HLS coordinates (see appendix A).

GRAPHICS INPUT WITH BUTTON INPUT - function 12

This form of graphics input is designed for graphics devices that

use mice, digitizers, joysticks, or the like.  Theoritically, the

pointing device contains a set of buttons, any or all of which may be

"pressed" or otherwise caused to change binary state.  When a state

change occurs, the graphics input operation is concluded, and the new

state of the buttons is returned.  The buttons are numbered from 1 to

N for a N button device.  The state of each button is represented by a

bit in the "button state" returned variable.  The correspondance

between buttons and bits is:

Bit = 2**(n-1)  Where n is the button number.

GUIDE TO WRITING A DIGLIB GRAPHICS DRIVER                                 Page 9

EXTENDED DIGLIB DEVICE DRIVER FUNCTIONS

A bit value of one means the button is "set" or "pressed", and zero

means the corresponding button is "clear" or "not pressed".

For devices that support both graphics input with "pick

character" and graphics input with "button" input, it is desirable to

use different cursors if possible.  This will allow the user to

determine what action is necessary to terminate the graphics input.

To perform a graphics input with button input operation, the

following steps are necessary:

1.  Make the graphic (button) cursor visible if it is not always

visible,

2.  Allow the user to position the graphic cursor,

3.  When the selects the current position by pressing a "button",

read the graphic cursor position,

4.  Return the button state after the button was pressed, the X

position (in cm.) and the Y position (in cm.) of the graphic

cursor, and

5.  Make the graphic (button) cursor invisible if possible.

DRAW SOLID FILLED POLYGON - function 1024+N

The number of vertices are passed in as a part of the function

code.  I realize this is unorthodox, but everything else requires too

much overhead.  Two sophistications of hardware fill are suppored by

DIGLIB.  The first is the simple "convex polygon fill".  The second is

the "any polygon fill".  Some devices can only handle convex

(non-intersecting) polygons.  If you set the DEVICE CHARACTERISTICS

bit telling DIGLIB that the device can only handle convex polygons,

DIGLIB will supply software fill for the concave polygons, and call

the driver for the convex polygons.

DEVICE CHARACTERISTICS and the EXTENDED FUNCTIONS

All implementators of primitives 8, 10, 11 and/or 1024+N need be

aware of the interaction of these primitives with the Device

Characteristics Values (primitive 7).  Device characteristics value

number 6 is the number of foreground colors.  This includes color 1

but not color 0.  All devices have atleast one foreground color.  In

addition, device characteristics value 7 has the 2 bit set if the

device is capable of writing in the background color (color 0).  Pen

plotters and storage tubes are not capable of writing in the

background color, but most other graphic devices can handle this

function.  Let's take the example of the Tektronix 4027 terminal to

see how the device characteristics values (color only) are set.  A

4027 has the ability to display 8 colors simultaneously, out of a

palate of 64 total colors.  In addition, the 4027 has a programmable

Look-Up Table (LUT) so that colors may be defined.  Reading the

GUIDE TO WRITING A DIGLIB GRAPHICS DRIVER                                Page 10

EXTENDED DIGLIB DEVICE DRIVER FUNCTIONS

manual, we find that colors may be define by both RGB and HLS.  Thus

we would:

1.  Implement primitive 8,

2.  Implement primitives 10 and 11,

3.  Set device characteristics value 6 to 7 foreground colors

(one of the 8 colors must be background), and

4.  Set device characteristics value 7 to 1+4+32+64+128.

Note that we added 4 to the value of device characteristics value 7.

This reflects the fact that the 4027 can draw in the background color

(erasing color).

If we were to implement the polygon fill section of the 4027

driver, we would set bit 7 of the device characteristics bit (value

256) and leave bit 8 (value 512) clear because the Tek.  4027 can

handle polygons all polygons.

GUIDE TO WRITING A DIGLIB GRAPHICS DRIVER                                Page 11

DIGLIB DRIVERS - BEFORE YOU START CODING

Before you start writing a DIGLIB device driver, you should first

arm yourself with a good programmers manual for the target graphics

device.  Then, find a DIGLIB driver for a similar device and

plagiarize, plagiarize, plagiarize!  Make sure you don't just kluge it

though.  However, the RT and RSX drivers have built in buffering

subroutines that can be easily "lifted" and the VAX/VMS drivers have a

common set of terminal buffering subroutines that I suggest you use if

possible.

GUIDE TO WRITING A DIGLIB GRAPHICS DRIVER                                Page 12

DIGLIB GRAPHICS DRIVER EXAMPLES

DIGLIB GRAPHICS DRIVER EXAMPLES

The file "SKELTN.FTN" gives a skeleton graphics device driver

written in FORTRAN that may be edited with an editor and used as a

starting point for writing a DIGLIB graphics device driver.  However,

there is no reason that this skeleton need be used at all, nor even

that the device driver be written in FORTRAN as opposed to MACRO.  In

addition to "SKELTN.FTN", I have included the file "GD4025.FOR" which

is a working Tektronix 4025 driver for the VAX.

NOTE

By convention, all graphics device driver names  start

with the letters "GD" for Graphic Driver.

APPENDIX A

GRAPHICS DRIVER ARGUMENTS

The graphics device driver has 3 arguments:

1.  The graphics function to be performed,

2.  A real array whose  meaning  is  dependant  on  the  graphics

function to be performed, and

3.  A real array whose  meaning  is  dependant  on  the  graphics

function to be performed.

INPUT

IFXN            * TYPE: INTEGER CONSTANT OR VARIABLE.

* THE GRAPHICS FUNCTION TO PERFORM:

1 ==> INITIALIZE THE GRAPHICS DEVICE TO PREPARE IT

FOR GRAPHICS (AND ON MULTI-USER SYSTEMS, TAKE

EXCLUSIVE CONTROL OF THE DEVICE.)

2 ==> OBTAIN A FRESH PLOTTING SURFACE ON THE

GRAPHICS DEVICE.

3 ==> MOVE TO A NEW POSITION (X,Y), WHERE THE UNITS

OF BOTH X AND Y ARE IN CENTIMETERS.

4 ==> DRAW A LINE TO THE POSITION (X,Y), WHERE THE

UNITS OF X AND Y ARE IN CENTIMETERS.

5 ==> TRANSMIT ANY BUFFERED GRAPHICS COMMANDS THAT

HAVE NOT YET BEEN TRANSMITTED TO THE

GRAPHICS DEVICE.

6 ==> RELEASE CONTROL OF THE GRAPHICS DEVICE.

7 ==> RETURN TO THE CALLER THE DEVICE

CHARACTERISTICS OF THIS DEVICE.

8 ==> SET THE CURRENT DRAWING COLOR.

9 ==> PERFORM A GRAPHICS INPUT WITH PICK CHARACTER

OPERATION, RETURNING THE USER SELECTED

LOCATION (X,Y) WITH X AND Y SPECIFIED IN

CENTIMETERS.

10 ==> SET THE VISUAL COLOR ASSOCIATED WITH A

SPECIFIED COLOR NUMBER TO THE RGB VALUE

PROVIDED.

11 ==> SET THE VISUAL COLOR ASSOCIATED WITH A

SPECIFIED COLOR NUMBER TO THE HLS VALUE

PROVIDED.

Page A-2

1024+N==> DRAW FILLED POLYGON HAVING N SIDES.

XA              * TYPE: REAL ARRAY.

* DEPENDANT OF THE GRAPHICS FUNCTION GIVEN BY "IFXN".

1 ==> XA(1) IS RT-11 CHANNAL NUMBER ON WHICH

TO PERFORM I/O, OR RSX-11M LOGICAL UNIT

NUMBER ON WHICH TO PERFORM I/O.

XA(2) --> XA(7) UNUSED.

2 ==> XA(1) --> XA(7) UNUSED.

3 ==> XA(1) IS THE X COORDINATE IN CM. FOR THE

MOVE OPERATION.

XA(2) --> XA(7) UNUSED.

4 ==> XA(1) IS THE X COORDINATE IN CM. FOR THE

DRAW OPERATION.

XA(2) --> XA(7) UNUSED.

5 ==> XA(1) --> XA(7) UNUSED.

6 ==> XA(1) --> XA(7) UNUSED.

7 ==> XA(1) --> XA(7) UNUSED.

8 ==> XA(1) IS COLOR FOR SUBSEQUENT LINES.

XA(2) --> XA(7) UNUSED.

9 ==> XA(1) --> XA(7) UNUSED.

10 ==> XA(1) IS THE COLOR TO DEFINE.

11 ==> XA(1) IS THE COLOR TO DEFINE.

12 ==> XA(1) --> XA(7) UNUSED.

1024+N ==> XA(1) --> XA(N) X VERTICES OF POLYGON.

YA              * TYPE: REAL ARRAY

* DEPENDANT OF THE GRAPHICS FUNCTION GIVEN BY "IFXN".

1 ==> YA(1) IS THE ERROR FLAG FOR DEVICE SELECTION:

0 ==> NO ERRORS

>0 ==> ERROR (DEVICE SPECIFIC)

YA(2) --> YA(3) UNUSED.

2 ==> YA(1) --> YA(3) UNUSED.

3 ==> YA(1) IS THE Y COORDINATE IN CM. FOR THE

MOVE OPERATION.

YA(2) --> YA(3) UNUSED.

4 ==> YA(1) IS THE Y COORDINATE IN CM. FOR THE

DRAW OPERATION.

YA(2) --> YA(3) UNUSED.

5 ==> YA(1) --> YA(3) UNUSED.

6 ==> YA(1) --> YA(3) UNUSED.

7 ==> YA(1) --> YA(3) UNUSED.

8 ==> YA(1) --> YA(3) UNUSED.

9 ==> YA(1) --> YA(3) UNUSED.

10 ==> YA(1) IS % RED COMPONENT OF COLOR.

YA(2) IS % GREEN COMPONENT OF COLOR.

YA(3) IS % BLUE COMPONENT OF COLOR.

11 ==> YA(1) IS THE HUE (0-360).

YA(2) IS THE LIGHTNESS (0-100).

YA(3) IS THE SATURATION (0-100).

12 ==> YA(1) --> YA(3) UNUSED.

1024+N ==> YA(1) --> YA(N) Y VERTICES OF POLYGON.

OUTPUT

XA              * TYPE: REAL ARRAY.

Page A-3

* DEPENDANT OF THE GRAPHICS FUNCTION GIVEN BY "IFXN".

1 ==> XA(1) --> XA(7) UNUSED.

2 ==> XA(1) --> XA(7) UNUSED.

3 ==> XA(1) --> XA(7) UNUSED.

4 ==> XA(1) --> XA(7) UNUSED.

5 ==> XA(1) --> XA(7) UNUSED.

6 ==> XA(1) --> XA(7) UNUSED.

7 ==> XA(1) --> XA(7) ARE SET TO THIS DEVICES

CHARACTERISTIC WORDS:

XA(1) = DEVICE ID. (A UNIQUE DEVICE NUMBER

OBTAINED FROM HAL BRAND X26312.

XA(2) = X AXIS LENGTH OF THIS DEVICE IN CM.

XA(3) = Y AXIS LENGTH IN CM.

XA(4) = X AXIS RESOLUTION IN GRAPHICS UNITS

(GU) PER CENTIMETER.

XA(5) = Y AXIS RESOLUTION IN GU/CM.

XA(6) = NUMBER OF AVAILABLE FOREGROUND

COLORS ON THIS DEVICE.

XA(7) = DEVICE CHARACTERISTICS BITS.

THE SUM OF:

0 IF STROKE (PLOTTER) DEVICE.

1 IF RASTER DEVICE.

2 IF DVST (STORAGE TUBE)

3 PRINTER/PLOTTER

4 IF DRAWING IN THE BACKGROUND

COLOR (COLOR 0) IS AVAILABLE.

8 IF DEVICE IS A HARDCOPY DEVICE.

16 IF DEVICE IS A SHAREABLE

DEVICE.

32 DEVICE'S COLORS ARE HLS

SELECTABLE.

64 DEVICE'S COLORS ARE RGB

SELECTABLE.

128 GRAPHICS INPUT WITH "PICK

CHARACTER" IS AVAILABLE.

256 DEVICE DRAWS FILLED POLYGONS.

512 DEVICE REQUIRES CONVEX

POLYGONS.

1024 GRAPHICS INPUT WITH BUTTONS

IS AVAILABLE.

XA(8) = 1 + THE NUMBER OF SCAN LINES TO SKIP

WHILE DOING A SOFTWARE POLYGON FILL.

THIS VALUE IS USED TO TAKE ACCOUNT

OF THE FACT THAT PLOTTERS HAVE PENS

THAT ARE TYPICALLY MANY TIMES

THICKER THAN THE PLOTTER STEP SIZE.

THIS PARAMETER SHOULD BE ONE FOR

ALL RASTER DEVICES.   NOTE: FOR

BACKWARDS COMPATIBILITY,  THIS

VALUE IS DEFAULTED TO ONE, SO ANY

DRIVER THAT RETURNS ON THE FIRST 7

ARRAY ELEMENTS WILL GET THIS VALUE

SET TO 1.

8 ==> XA(1) = PICK CHARACTER FROM GRAPHICS DEVICE

OR FROM TERMINAL.

XA(2) = X COORDINATE SELECTED IN CM.

Page A-4

XA(3) = Y COORDINATE SELECTED IN CM.

XA(4) --> XA(7) UNUSED.

9 ==> XA(1) --> XA(7) UNUSED.

10 ==> XA(1) --> XA(7) UNUSED.

11 ==> XA(1) --> XA(7) UNUSED.

12 ==> XA(1) = BUTTON BITS FROM GRAPHICS DEVICE

XA(2) = X COORDINATE SELECTED IN CM.

XA(3) = Y COORDINATE SELECTED IN CM.

XA(4) --> XA(7) UNUSED.

1024+N ==> XA(1) --> XA(N) UNUSED.