A Double-Sided Double-Density 8" Floppy Disk Handler

Bob Schor

Eye and Ear Institute

203 Lothrop Street

Pittsburgh, PA   15213

(412) 647-2116

Most users of DEC 8" floppy drives know about the RX01, the

single-density, single-sided 8" floppy drive, with 494 formatted blocks,

and the RX02, the double-density, single-sided 8" floppy, with 988 (twice

as many) blocks.  There is some evidence that an RX03 was contemplated,

namely the RX02, using double-sided media (and, of course, two read/write

heads), but DEC never implemented such a drive (to my knowledge).

If one buys DEC-compatible hardware, however, it is possible to buy

double-sided drives, and controllers capable of selecting both heads.  Some

controllers even directly support 22-bit Q-Bus addressing, easing their use

on 11/73 systems.

This article describes modifications to the DY handler, DY.MAC, which

will enable double-sided floppy use.  It has been used and tested with

floppy controllers from Data Systems Design (DSD, now Qualogy) and Sigma

Information Systems (Sigma).  These changes have been installed in RT-11

since Version 5.0; they have been field-tested in RT-11 5.5 (pre-release),

and should be continue to be portable through future versions.

Modifications to enable the handlers to be used under TSX-Plus (S&H

Computer Systems) will also be described.

To aid in making changes, code segments will be divided into several

classes.  Code will be presented in the following sections:  header

comments, DSD-specific code, Sigma-specific code, double-sided code, and

TSX-specific code.  Code sections will be presented, along with a

description of the code location within the handler; this should provide

the most flexible means of dealing with various handler versions.

Header Code

_____ ____

The following code should be placed on the "Conditional Assembly

Summary" page; you could also include specific definitions here to enable

desired versions:

;************************* Insert *************************************

;The following conditionals are available --

;       tsx$p   = 1             ;modify handler for TSX

;       dsd$    = 1             ;modify handler for DSD-880

;       sis$    = 1             ;modify handler for Sigma RXV31

;       dy$ds   = 1             ;modify handler for double-sided operation

- 1 -

;The original code modifications were courtesy of S&H Computer Systems (TSX),

;Data Systems Design (DSD), and Sigma Information Systems (SIS).  Code

;consolidated, maintained, and tracked through successive RT-11 versions

;by Bob Schor.  Current version (August, 1989) tested with RT-11 5.4 and

;RT-11 5.5 DY.MAC handlers.  Attempts made to keep changes simple, logical,

;and understandable.

;To generate a specific version, choose some combination of the above

;conditionals.

;Note minor error in DY handler -- extra word allocated in DYRBUF.  Total

;space required = 6 words, two registers + 4-word error block, only first word

;of which returned to ERRLOG.

;The following code sets undefined defaults, ensures conditional values are

;0 or 1:

.iif ndf tsx$p  tsx$p   = 0     ;if undefined, default to 0

.iif ndf dsd$   dsd$    = 0

.iif ndf sis$   sis$    = 0

.iif ndf dy$ds  dy$ds   = 0

.iif ne  tsx$p  tsx$p   = 1     ;ensure 0 or 1

.iif ne  dsd$   dsd$    = 1

.iif ne  sis$   sis$    = 1

.iif ne  dy$ds  dy$ds   = 1

;If compiling TSX version, use memory-management code

.iif ne  tsx$p  mmg$t   = 1

.iif ndf mmg$t  mmg$t   = 0

.iif ne  tsx$p  bf      = 1

.iif ne  tsx$p  tim$it  = 1

;Ensure both dsd$ and sis$ are not specified

.iif ne  dsd$&sis$      .error  ;Cannot choose both dsd$ and sis$

;If neither dsd$ nor sis$ specified, do not generate double-side code

.iif eq  dsd$!sis$      dy$ds   = 0

;************************* End Insert *********************************

DSD-specific Code

___________ ____

The only DSD-specific code concerns returning the extended error

registers to the error-logging routine; apparently the DSD protocol differs

slightly from the RX02 specification.  The change, below, consists of

conditionally removing part of the standard DY handler, located in the

error-handling section of the handler, in the conditional code guarded by

the conditional ".IF NE  ERL$G".  Locate the request for access to the

- 2 -

extended registers ("MOV  #CSMAIN!CSGO, @R4").  Following it are normally

two wait loops, one testing CSTR, the second CSDONE.  The second loop

should be conditionally removed by replacing it with the following:

;************************* Replace ************************************

.if eq  dsd$                            ;skip if DSD controller

61$:    BIT     #CSDONE,(R4)

BEQ     61$

.endc   ; eq dsd$

;************************* End Replace ********************************

Sigma-specific Code

_____________ ____

The Sigma controller has 22-bit capability, which is easily utilized,

allowing the floppy disk to be used in systems with more than 256 kB, and

without clumsy remapping games to keep the buffers in low memory.

The first change is to add a definition to the command/status register

bits, located near the start of the handler, along with the other

definitions.  Add the following:

;************************* Insert *************************************

.if ne  sis$                            ;Sigma controller

cs22    =   2000                ;22-bit addressing

.endc

;************************* End Insert *********************************

The second change follows the Driver Request Entry Point.  Locate a

call to MPPTR, which retrieves an extended physical address from the

monitor.  Replace the two instructions "BIT  #1700, R4", "BNE.  DYERR" with

the following alternate:

;************************* Replace ************************************

.if eq  sis$                            ;alternate code for Sigma controller

BIT     #1700,R4

BNE.    DYERR

.iff                                    ;Sigma code

mov     r4, -(sp)               ;put high address (shifted) on stack

asr     (sp)                    ;shift right 4

asr     (sp)                    ;to put high bits in bits 0-5

asr     (sp)

asr     (sp)

mov     (sp)+, extadr           ;save extended address

bic     #1700, r4               ;clear unneeded bits

.endc   ; eq/iff sis$

;************************* End Replace ********************************

The above code fragment saves the high six address bits.  The next

fragment, located in Start Transfer or Retry, increments the high address

if a transfer "rolls over" the 16-bit boundary.  Near the end of the page,

after a call to DOSILO, a register TST, and a "BMI DYDONE", there should be

- 3 -

code to add to the buffer address (BUFRAD).  The conditional code guarded

by MMG$T will have a BCC instruction.  The code being skipped by the BCC

will cause #10000 to be added to a register (this will eventually set the

extended address bits in the CSR).  Include the following (just before the

.ENDC which ends the MMG$T-guarded code):

;************************* Insert *************************************

.if ne  tsx$p!sis$                      ;handle possible 22-bit address

bic     #40000, r0              ;forbid carry out of bit 18

.endc   ; ne tsx$p!sis$

.if ne  sis$                            ;for Sigma controller

inc     extadr                  ;adjust extended bus address

.endc   ; ne sis$

;************************* End Insert *********************************

The first instruction, which is also needed for TSX, guards against

incrementing addresses across an 18-bit boundary and inadvertantly setting

bit 14 in the CSR (bits 12 and 13 are the 18-bit extended address bits).

The second instruction increments the Sigma's 22-bit extended address.

Twenty-two bit operation is enabled in the DOSILO routine.  At the

beginning of the routine, a register (probably r0) is being made ready to

be sent to the CSR.  Locate the instruction which clears some of the

command bits (probably "BIC  #6*2,R0"), and insert

;************************* Insert *************************************

.if ne  sis$&mmg$t                      ;Sigma controller, 22-bit mode

bis     #cs22, r0               ;enable 22-bit addressing

.endc   ; ne sis$&mmg$t

;************************* End Insert *********************************

The end of the DOSILO routine should be a branch to DYDOFN (either a BR or

a .BR instruction, depending on version).  Replace it with the alternate

code (illustrating the .BR version, adjust as appropriate):

;************************* Replace ************************************

.if ne  sis$&mmg$t                      ;Sigma controller, 22-bit mode

br      dydof2                  ;do 22-bit command protocol

.iff

.BR     DYDOFN

.endc   ; ne sis$&mmg$t

;************************* End Replace ********************************

The final code section for the Sigma controller provides the alternate

protocol just requested to handle 22-bit data transfers.  This code can

immediately follow the DYDOFN routine.

- 4 -

;************************* Insert *************************************

.if ne  sis$&mmg$t                      ;Sigma controller, 22-bit mode

;alternative protocol for 22-bit fill buff, empty buff, read error code

dydof2: bitb    #cstr!csdone, @r4       ;transfer or done?

beq     dydof2

bpl     dyerr2                  ;controller should want data

mov     r3, @r5                 ;send command

1$:     bitb    #cstr!csdone, @r4       ;transfer or done?

beq     1$

bpl     dyerr2                  ;more data should be needed

mov     r2, @r5                 ;continue protocol

2$:     bitb    #cstr!csdone, @r4       ;transfer or done?

beq     2$

bpl     dyerr2                  ;more data should be needed

mov     extadr, @r5             ;load bus address extension register

return                          ;successful completion

extadr: .word   0                       ;extended address goes here

.endc   ; ne sis$&mmg$t

;************************* End Insert *********************************

It is possible that the above code will generate "A" errors, due to the

"BPL DYERR2" address being out of bounds.  If this occurs, replace this

instruction with "BPL 4$", and include the line "4$:  JMP DYERR2" just

following the "RETURN" instruction at the end of this section of code.

Double-sided Floppy Code

___________ ______ ____

The following code handles the actual double-sided floppy addressing

and head selection logic.  Basically, a double-sided floppy is treated the

same as a single-sided one, except that blocks higher than 988. (assuming

double density) are placed on the second side.  Note that this scheme

provides for maximal interchangability between single-sided RX02 and

double-sided RX03 disks -- if you less-than-half-fill a double-sided disk,

your colleagues with single-sided drives will have no trouble reading the

information (be sure you SQUEEZE the disk to keep all blocks below 988!).

What will ultimately determine whether a disk is treated as single or

double-sided will be DUP, which initializes the directory (and thereby sets

the total number of blocks present).  Code below will test for double-sided

media (identifiable by a changed location of the index hole) and will

return the proper size when queried with .SPFUN #SIZ$FN.

The initial changes are definitions, near the top of the handler.  The

first defines the "side-two present" bit in the error/status register.

;************************* Insert *************************************

.if ne  dy$ds                           ;double-side code

essid2  =      2

.endc   ; ne dy$ds

;************************* End Insert *********************************

- 5 -

The second definition should follow the definition of DDNBLK, and defines

the number of blocks for double-sided (double-density) devices:

;************************* Insert *************************************

.if ne  dy$ds                           ;double-side code

dsnblk  = ddnblk*2              ;twice as big

.endc   ; ne dy$ds

;************************* End Insert *********************************

The next code, near the start of Start Transfer or Retry, dynamically

adjusts the size if a double-sided floppy is installed.  Locate code where

#DYDSIZ is pushed on the stack.  At that point, before testing the #ESDN

(density) bit, include the additional code

;************************* Insert *************************************

.if ne  dy$ds                           ;double-side code

bit     #essid2, @r5            ;see if double-sided floppy

beq     2$

asl     @sp                     ;in which case, double the size

2$:

.endc   ; ne dy$ds

;************************* End Insert *********************************

In Error Handling -- Change Density, Retry, alternative code is

included to handle double-sided capability.  Locate the bit-test with

ESDNER, and replace the BEQ following it with the alternative code

;************************* Replace ************************************

.if eq  dy$ds                           ;single-side code

BEQ     5$

.iff                                    ;double-side code

bne     1$                      ;skip ahead if density error

bit     #escrc, @r5             ;CRC error?

bne     5$                      ;yes, report error

bit     #cshead, r0             ;see which side active

beq     5$                      ;report error if first side

bit     #csdn, r0               ;single or double density?

bne     5$                      ;report error if double

1$:     bit     #cshead, r0             ;density error or SDDS

beq     2$                      ;if double-sided,

add     #dsnblk, dylsn          ;adjust logical sector number

2$:     bic     #cshead, r0             ;use first side

.endc   ; eq/iff dy$ds

;************************* End Replace ********************************

At the start of DOXFER, Start a Sector Read or Write, the logical

sector number must be tested to see if the desired block is on the first or

second side.  This routine is entered with the desired status register in

R0, a pointer to the CSR in R4 -- you will replace the code loading the CSR

and loading registers with the word count and logical sector address as

shown below:

- 6 -

;************************* Replace ************************************

.if ne  dy$ds                           ;double-sided code

mov     dylsn, r3               ;get logical sector number

cmp     r3, #dsnblk             ;see if it fits on first side

blt     1$                      ;skip if so

sub     #dsnblk, r3             ;adjust location on side 2

bis     #cshead, r0             ;and set second head

1$:     mov     r0, @r4                 ;set command

mov     wrdcnt, r2              ;and word count

.iff

MOV     R0,@R4

MOV     WRDCNT,R2

MOV     DYLSN,R3

.endc   ; ne/iff dy$ds

;************************* End Replace ********************************

The final change in the handler is to modify the Bootstrap Read

Routine to use the appropriate head (depending on the logical sector

requested) for the read.  Locate the instruction which loads the bootstrap

CSR into R5 (it will look like "2$:  MOV  (PC)+,R5", "BOTCSR:  .WORD

DY$CSR").  Replace the first instruction as follows:

;************************* Replace ************************************

2$:

.if ne  dy$ds                           ;double-sided code

cmp     r0, #dsnblk             ;skip if requested block number

blt     12$                     ;on first side

sub     #dsnblk, r0             ;second side, adjust block number

bis     #cshead, redcmd         ;and enable second side

12$:

.endc   ; ne dy$ds

MOV     (PC)+,R5

;************************* End Replace ********************************

Code for TSX-Plus

___ ___ ________

For TSX-Plus, a few changes need to be made.  First, some of the SET

code in the handler does not appear to be necessary (the .SLP files which

come with TSX conditionally omit part of it -- I have omitted a never-used

subroutine as well).  Second, a change is made to allow a smooth transition

across 16-bit address boundaries.  Finally, a change in the use of the PAR

registers (TSX uses PAR 6 instead of PAR 1) is required.

The first changes are suggested by DYxxx.SLP, the files which come

with TSX-Plus.  They conditionally remove code in the handler which would

otherwise cause SET options to change the in-core loaded handler.  Two

pieces of code are involved.  The first is a routine labeled FINDRV, which

ends with a BR instruction (probably "BR O.GOOD"); the following location

following the removed code is labeled DAREA.  Insert the following just

before FINDRV:

- 7 -

;************************* Insert *************************************

.if eq  tsx$p                           ;RT only; TSX ignores loaded handler

;************************* End Insert *********************************

At the end of the routine, insert the following (probably right before

DAREA):

;************************* Insert *************************************

.endc   ; eq tsx$p

;************************* End Insert *********************************

This routine is called near the end of the Set Options.  Locate the "CALL

FINDRV" instruction; insert the following to remove it for TSX:

;************************* Insert *************************************

.if eq  tsx$p                           ;RT only; TSX ignores loaded handler

;************************* End Insert *********************************

After the FINDRV, there should be a BCS instruction, probably to O.GOOD;

the code which would be taken if the carry were clear should be eliminated.

This can best be accomplished by locating the unconditional branch to the

BCS target (or the target itself, if it should immediately follow), and

eliminating all of the code in between.  In Version 5.4, this would mean

inserting the following just before a "BR  O.GOOD" instruction, about seven

lines after the "CALL FINDRV":

;************************* Insert *************************************

.endc   ; eq tsx$p

;************************* End Insert *********************************

The next modification is in the Driver Request Entry Point.  Locate

the second call to $PTWRD.  Following it is an increment of R0, which is

being used as a buffer pointer.  For TSX, this instruction is modified to

handle carries into the higher address bits in the CSR:

;************************* Replace ************************************

.if eq  tsx$p                           ;RT/TSX difference

TST     (R0)+

.iff                                    ;for TSX,

add     #2, r0                  ;increment bus address

bcc     41$                     ;check for 16-bit overflow

add     #10000, (sp)            ;adjust high bits on stack

bic     #40000, (sp)            ;prevent 18-bit carry

41$:

.endc   ; eq/iff tsx$p

;************************* End Replace ********************************

Note that this above change appears to be an out-right "bug" in the

handler, which perhaps should be also included in the RT-11 versions.  DEC

has been notified of the potential problem.

- 8 -

The final change in in Start Transfer or Retry, and concerns the use

of PAR 6 by TSX.  Locate the instructions which compare the buffer address

with the end of PAR 1 ("CMP Q.BUFF-Q.BLKN(R4),#20000", "BHIS $55"), and

replace the comparison as shown:

;************************* Replace ************************************

.if eq  tsx$p                           ;RT/TSX use of PAR registers

CMP     Q.BUFF-Q.BLKN(R4),#20000

.iff                                    ;for TSX,

cmp     q.buff-q.blkn(r4), #140000      ;use PAR 6 instead of PAR 1

.endc   ; eq/iff tsx$p

;************************* End Replace ********************************

The above code changes for TSX-Plus assumes that you are working with

compatible versions of RT-11 and TSX, for example, RT-11 Version 5.2 to

5.4, and TSX Version 6.3 to 6.4.  At this particular time, the two systems

are slightly out-of-synch; the soon-to-be-released RT-11 Version 5.5

contains a few handler features which have not yet been incorporated into

the current TSX Version 6.4.  Once this version of RT-11 is released, an

update can be obtained from the author regarding these (minor) changes; this

may not be necessary, however, as S&H may also announce a compatible release

of TSX.

Assembly and linkage

_______ ___ _______

Install the above changes into the handler; in this example, I will

assume that the revised double-sided handler is called DYDS.MAC.  Decide

which version (or versions) you wish.  For the sake of discussion, suppose

you wish a double-sided handler, tailored for the Sigma controller, and

wish Single Job, Extended Memory, and TSX versions.  Create three .MAC

header files, here called SJDYDS, XMDYDS, and TSDYDS, with contents as

follows:

SJDYDS                  XMDYDS                  TSDYDS

sis$    = 1             sis$    = 1             sis$    = 1

dy$ds   = 1             dy$ds   = 1             dy$ds   = 1

mmg$t   = 1             mmg$t   = 1

tim$it  = 1             tsx$p   = 1

Note that in assembling the TSX-specific handler, code will automatically

be generated for device time-out support.  Many XM handlers also are genned

with this option; you would need to include the line "tim$it  = 1" in

XMDYDS to be compatible with these handlers.

- 9 -

Assemble the relevant object modules, using the following commands:

MACRO/OBJ:SJDYDS SJDYDS+DYDS

MACRO/OBJ:XMDYDS XMDYDS+DYDS

MACRO/OBJ:TSDYDS TSDYDS+DYDS

Link the actual handlers with

LINK/NOBITMAP/EXE:DY.SYS  SJDYDS

LINK/NOBITMAP/EXE:DYX.SYS XMDYDS

LINK/NOBITMAP/EXE:DY.TSX  TSDYDS

These should all now work with the appropriate operating systems.

Note, however, that in order to get twice the storage on your floppies, you

must have (a) a drive with two read/write heads (several third-party

half-height drives fulfill this requirement), (b) a controller which

supports two heads (such as the one from Sigma), (c) a floppy

"recognizable" as double-sided (sold by most major vendors, recognizable

from the two head cut-outs, the altered position of the index hole, and

frequently by a designation like DS/DD, or similar indication), and (d) a

double-sided floppy handler, such as described herein.  Good luck.

- 10 -