SETDZ - Set DZ(Q)-11 communications parameters V01.04	10-Sep-89

SETDZ is an RT-11 utility to set the communication parameters

on a DZ type serial interface board (DZ-11, DZV-11, DZQ-11).

It is modeled after the distributed SPEED utility.

The available options are:

/T:n  Terminal #

/S:n  Baud Rate (Speed)

/D    Parity Disabled

/E:n  Parity Enabled, n=0(even), n=1(odd)

/C:n  Character Length (5-8)

/H	Display options

Since the options are processed by the CSI, numeric parameters

are assumed octal by default.  Therefore the baud rate should be

followed by a period.

Each option may only appear once on a command line.

If the terminal port specified with the /T option is not a DZ

type port the command line is ignored.

Examples:

SETDZ /T:2/S:2400.	Set terminal 2 to 2400 bps

SETDZ /T:8./E:0/C:7	Set terminal 8 to 7 data bits, even parity

If several lines need to be set, a command file can be created as

follows:

R SETDZ

/T:1/S:2400.

/T:2/S:2400.

^C

Be sure to run this file with $@ if IND is turned on.