Reading Unix (tm) TAR tapes on RSTS/E and VAX/VMS

The  RDMT.BAS  and UNTAR.BAS programs read Unix TAR format tapes

on RSTS/E.  Equivalent programs, VRDMT.BAS  and  VUNTAR.BAS  are

available  for  Vax/VMS.   Source  is in [5,1] on RSTS/E Decus-C

distribution kits, and on [DECUSC.COMMAND] on VMS distributions.

These  programs  are part of the Decus-C distribution and may be

freely redistributed.

Both programs should be self-explanatory.

The  tape  is read onto a disk file using (V)RDMT.BAS.  Strictly

speaking,  this  is  not  essential,  but   simplified   writing

(V)UNTAR.  On VMS, note that you must have allocated and mounted

the magtape with sufficient buffer space:

$ mount mta0:/foreign/blocksize=10240

RDMT  creates  a  file,  nominally  TARTAP.DAT,  on your default

directory.

(V)UNTAR  copies  all  files  contained in the archive onto your

default (or otherwise specified) directory.  Some directory  and

name   editing  is  done  to  approximate  Dec-style  file  name

conventions.  If  a  file  cannot  be  created,  or  if  a  file

currently  exists with that name, an alternate name is requested

from the user.

Note

UNTAR  is fairly crude -- it cannot deal with embedded archives,

and does not deal well with tar archives containing both  binary

and textual information.

Almost every time I use UNTAR, I modify it in some manner.

When  creating  TAR  tapes  on  Unix for eventual reading on Dec

systems, you should collect the  files  to  be  written  into  a

specific directory, then:

$ cd /directorytotar

$ tar c0v *

This  creates  (c) an archive on magtape unit 0, typing the name

of each file as it is written to the tape (v).

Again,  note  that  you  will  be  happier  if you only transmit

"vanilla-ASCII" text files between the systems.

A  C  implementation  of  untar  is  stored on the [.MISC] [6,2]

directory.