C A L E N D A R   C L O C K   O P T I O N   R O U T I N E S

F O R   G T S C   3 0 6 A / 3 0 7

Version 4.01

08 October 1987

Author:  William K. Walker

RDB/Alpha Systems

740 Alpha Rd., Box 149

Alpha, OH  45301

(513) 426-7094/426-0344

GTSC 306A/307 Calendar Clock Routines - V04.01            Page 2

1.0  INTRODUCTION

This  package consists of a collection of routines which provide

some basic support for the calendar clock option  on  the  model

306A real-time clock board for Q-bus PDP-11s manufactured by the

Grant Technology Division of Data Products  Corporation.   These

routines will also work with the model 307 (calendar clock only)

board.  Included are programs to set and retrieve the  date  and

time, a sample RT-11 device handler for reading the clock regis-

ters, and an example program that utilizes the handler  to  pro-

duce a running date/time display on a VT100.

2.0  SET306 AND SETDT

SET306  and  SETDT  are  utility programs for use with the clock

calendar section of the 306A board.  SET306 allows  you  to  set

the date and time on the board, while SETDT loads the RT-11 date

and time from the values being kept by the board's  clock.   The

following  comprises  some  notes and information on running and

customizing these programs.  Additional information is contained

in the sources, and in the manual for the board itself.

These  programs have been written to run under RT-11, version 4,

or later.  They will also run under the  corresponding  versions

of TSX-Plus (you must have operator privilege).

2.1  Running SET306

SET306  can  be  run in any legal RT-11 manner (monitor R or RUN

command, "run-by-name" if you have V5), from either the console,

or  a command file.  Once it is running, it will prompt you in a

fairly unambiguous manner for the the date and time  information

necessary  to  set  up the board's calendar clock.  As a part of

each prompt, it will include a default value in square brackets.

This  is  the value that was stored on the board at the time you

ran SET306.  If you wish to retain the default, just  press  the

return key.

For  example,  the dialogue to set the clock for Sunday, 28 June

1987, at 11:14:15 might look like the following:

Year (1987=87)? [87]

Month (Jan=1)? [ 6]

Day? [28]

Hour? [11]

Minute? [13] 14

Second? [ 5] 15

Press <return> to set clock...

If  SET306  is  set up for AM/PM style times, the "Hour?" prompt

will be followed an additional prompt  asking  you  whether  the

time is AM or PM.  E.g.,

AM or PM (A or P)? [ A]

GTSC 306A/307 Calendar Clock Routines - V04.01            Page 3

If  you abort the program at any time during the above dialogue,

the current setting of the clock is not changed

The  clock  will  also  remain unaffected if none of the default

values are changed (i.e., the return key is pressed in  response

to  every prompt).  In this instance, the program exits with the

message "?SET306-I-Clock not changed."

2.2  Running SETDT

SETDT  may  also be run in any legal RT-11 manner.  A very handy

way to use this program is to run it from your start-up  command

file  --  any  time you bootstrap the system, the RT-11 date and

time will automatically be set from the clock board.   When  you

run SETDT, it will also cause the current day of the week, date,

and time to be printed on the console.

NOTE

SETDT uses the .SDTTM programmed request.  It will not

run under versions of RT-11 earlier than version 4.

2.3  Configuration and Installation

The  306A  calendar  clock  has more options than a Cadillac Se-

ville.  As you might expect, SETDT and SET306 reflect  this  si-

tuation internally.  There are a number of assembly conditionals

in both routines that can be set, disabled, or  changed  to  re-

flect  your  system  configuration, and your choice of clock op-

tions.  The various options are summarized below.

System:

o  has/does not have EIS instructions

o  line-time clock is 50Hz or 60Hz

o  base address of 306A board (default is 170400)

Clock:

o  clock registers read as BCD or binary

o  time format is 24-hour or am/pm

o  clock  does/does  not adjust automatically for Daylight

Savings Time

You  may  also enable or disable the printing of the system date

and time on exit from SETDT.

GTSC 306A/307 Calendar Clock Routines - V04.01            Page 4

NOTE!!!

Make  sure  that  both SET306 and SETDT are set up for

the same clock board configuration!

The  copies  of SET306.SAV and SETDT.SAV on the distribution are

assembled with the following options:

o  EIS instructions

o  60Hz line-time clock

o  base address = 170400

o  registers read in binary format

o  24-hour clock format

o  automatic adjustment for Daylight Savings Time

o  SETDT prints day-of-week, date and time on exit

If  you  need  to change the assembly conditionals for SET306 or

SETDT, edit the "conditionals" section of the source file.  This

involves  inserting  or removing a semicolon at the beginning of

the appropriate line.

Building  the programs from the sources is straightforward.  As-

suming that MAC306.MAC, SET306.MAC, and  SETDT.MAC  are  on  the

default  device, DK:, the necessary dialogue will look something

like the following sequence:

.LIB/MAC MAC306 MAC306

.MACRO/OBJ:SET306 MAC306/LIB+SET306

.LINK SET306

.MACRO/OBJ:SETDT MAC306/LIB+SETDT

.LINK SETDT

2.4  GSDTM Subroutine

If you wish to incorporate the ability to set the RT-11 date and

time from the 306A calendar clock registers into your  own  pro-

gram,  a  FORTRAN-callable  routine  named GSDTM can be produced

from the SETDT source code by defining the assembly  conditional

FIV.   Calling  GSDTM from within your applications program will

have the same effect as running  the  SETDT  program.   See  the

SETDT sources for more details.

GTSC 306A/307 Calendar Clock Routines - V04.01            Page 5

3.0  CK HANDLER AND CKDEMO

The CK handler is a simple RT-11 device handler which allows you

to read the contents of the calendar clock registers  as  binary

data.   "Standard", pre-assembled versions of CK are included in

the distribution for RT-11 (CK.SYS  and  CKX.SYS)  and  TSX-Plus

(CK.TSX).  See the CK source code for details on it's operation.

The  CK  handler  is  built for the default clock CSR address of

170400 and vector of 450.  If your board  is  installed  with  a

non-standard  CSR  and  vector,  you can reconfigure CK to match

your values by using the SET options.  SET  CK  CSR=n  sets  the

base  CSR  address  for  the  calendar  clock registers.  SET CK

VECTOR=n sets the base vector address.

The  CK handler also implements two additional SET options.  SET

CK SHOW displays the current CSR and vector values being used by

CK.  SET CK HELP lists all of the SET options as well as a short

explanation of each.

Note  that  CK  uses some RT-11 macros introduces with V5.1.  If

you need to assemble CK under an earlier version of RT, you will

need to modify the offending macro calls.

If  you  need to rebuild CK from the sources, follow the handler

assembly and link instructions in the RT-11 documentation.

A  FORTRAN  program, CKDEMO, is also included to demonstrate the

use of the CK handler.  It uses  the  CK  handler  to  read  the

calendar  clock  and  produce  a  running date/time display on a

VT100 compatible terminal.   CKDEMO  will  run  under  RT-11  or

TSX-Plus.

4.0  BUGS

I want to know about bugs!

If you find any, write to me or call me:

William K. Walker

P. O. Box 149

Alpha, OH  45301

(513) 426-7094/426-0344

I  can  also  be contacted via several different electronic mail

addresses.

CompuServe:  Bill Walker 71066,24

EasyLink:  62752448 (or Telex 62752448 ESL UD)

MCI Mail:  333-1675 / RDBALPHA (or Telex 6503331675 MCI UW)