SPECIFICATIONS FOR THE RT-11/TSX-PLUS SERIAL PORT I/O HANDLER (IO.SYS).

-----------------------------------------------------------------------

R. D. BROWNRIGG

ABSTRACT:  A new device handler for the RT-11 and TSX-PLUS operating systems

is described.  This handler provides full-duplex input and output

to several serial interface ports simultaneously.  Installation

instructions and information necessary to communicate with the

handler using the MACRO assembly language are detailed.

1.  INTRODUCTION.

The IO handler allows programs to communicate with serial ports under the

RT-11 or TSX-PLUS operating systems.  This may be done either in an output

only manner or by using a two-way communication, in which both output and

input may be achieved concurrently to and from the same port.  Further, the

handler is capable of performing output or I/O to several ports

simultaneously.

This report firstly describes how to install the handler in an RT-11 or

TSX-Plus system, then describes the two means of communicating with the

handler.

2. INSTALLATION REQUIREMENTS.

Before the IO handler can be installed in a user's system, it must be

tailored to suit the hardware available. Further, there are some operating

system considerations which will influence just which hardware can be

considered 'available'.

2.1 Operating System Considerations.

The main operating system consideration to affect the installation of the

IO handler arises from possible conflicts with other handlers designed to

communicate with serial interface I/O ports on the system. For example, if

one wishes to retain the LS (serial printer) handler, a conflict may arise

if the IO handler is configured to communicate with the particular port

assigned to the LS handler. Under the TSX-Plus operating system it is not

possible to have two different handlers accessing the same hardware

interface. This is because all handlers under TSX-Plus are permanently

LOADed, or resident in memory, with their interrupt vectors set up, and

there is no LOAD command, which could modify the interrupt vector for the

device. Under the RT-11 monitors, however, it is usually the case that a

handler is LOADed only when it is required, and therefore it is possible to

have more than one handler capable of accessing a particular device.

Another consideration affecting installation under TSX-Plus is that it is

not possible to use the IO handler to access terminal ports, those serial

interface I/O ports which have been assigned line definitions during the

TSX-Plus SYSGEN process. This is also because the interrupt vectors for the

terminals are set up once and for all when TSX-Plus first starts up.

Finally, there is a difficulty under TSX-Plus if one of the IO ports is

used for a spooled device, such as a serial printer. This configuration

will work in general, the difficulty being that the port being used for

spooling cannot be used as an I/O port. Thus normal .WRITE programmed

requests will behave as expected and be spooled by the operating system,

but .SPFUN programmed requests will not be handled correctly, and are

likely to crash the operating system.

- 2 -

2.2 Installation Procedure.

Given that a decision has been made as to which serial interface I/O ports

are to be accessible via the IO handler, it remains to modify the handler

to reflect this decision, and then install the handler as part of the

operating system.

There are several parts of the IO handler which may need modifying if an

unusual combination of ports is required to be accessible to the handler.

The most straightforward setup is to specify as being accessible a number

of consecutive ports, starting with the lowest vectored non-console port,

this involving the least amount of modification to the handler. However it

is possible to specify non-consecutive ports, and in any order if desired.

Items within the handler which may need modification in this situation are

the constant specifying the number of ports, the base CSR and vector

addresses, the vector addresses in the vector table, and the register

addresses in the local port tables.

The number of ports accessible to the handler is specified by the constant

PORTS, which must be at least 1 and no greater than 8. The base CSR and

vector addresses are defined by the constants IO$CSR and IO$VEC which by

default are set to octal 176500 and octal 300 respectively. The vector

table is used to associate vectors with their corresponding processor

status values for the single interrupt entry point. The input and output

vectors are specified for each port, in the order in which it is desired to

define them. This then sets up a correspondence between the physical ports

and the handler's unit numbers, the first set of vectors defining port 0,

IO0:, the second set defining port 1, IO1:, and so on. By default the

vectors are defined consecutively from IO$VEC. Finally the local port

tables must also be set up to reflect this same order of port definition.

For each port, the local port table contains one word, initially zero, to

hold the current queue element address, four words which specify the four

register addresses for the port and four words for further port

information. The four register addresses must be set up for each port

required, and the groups of four addresses must appear in the same order as

the corresponding vectors in the vector table. The MACRO source file IO.MAC

has been set up with conditionals so that if the first N ports are to be

accessible to the IO handler, then all the user has to do to select these

ports is to set the constant PORTS to the value N.

Further constants which must be set by the user are the constants TSX and

XM, which set the handler up specially for the TSX-Plus operating system or

the RT-11 XM monitor respectively. If the TSX constant is set to 1 then the

handler's memory management configuration is set up for the TSX-Plus

operating system (irrespective of the setting of the XM constant).

Otherwise if the TSX constant is set to 0, then the XM constant determines

if the handler is configured for the RT-11 SJ and FB monitors (XM = 0) or

the RT-11 XM monitor (XM = 1).

- 3 -

3.  STANDARD OUTPUT.

The .WRIT(C, E, or W) request behaves in the normal RT-11 manner except that

several .WRIT(C or E) requests to different ports will result in concurrent

output to those ports.

In PASCAL for example, use:

rewrite(f, 'io3:');	{open port 3 (CSR=176530) for output}

write(f, data);

In MACRO for example, use the sequence:

.FETCH	loadaddr, devicename	; if handler not already LOADed.

.LOOKUP	emtarea, channo, devname

.WRITC	emtarea, channo, buffaddr, wordcnt, comproutine, blockno

Note that the device name io: is the same as io0:, referring to port zero,

which is usually associated with VECTOR=300 and CSR=176500.

4.  SETTING UP A TWO-WAY COMMUNICATION.

Two-way communication is achieved with the .SPFUN programmed request which

sets up a communication between the handler and the user via a buffer

communication area.  As with standard output, a number of ports may be

active simultaneously.  The MACRO sequence to set up this communication is

as follows:

.FETCH	loadaddr, devicename	; if handler not already LOADed.

.LOOKUP	emtarea, channo, devname

.SPFUN	emtarea, channo, #200, buffaddr, wordcnt, blockno, comproutine

Thereafter all I/O is achieved by the user reading from and writing to the

input and output ring buffers within the main buffer, while manipulating the

appropriate buffer pointers in the buffer communication area.

5.  STRUCTURE OF THE BUFFER.

The buffer referenced by the .SPFUN programmed request to the handler acts

as an interface between the user and the handler.  This buffer may be of any

length up to 20000 bytes (8kB) residing wholly within the user's program.

The first part of the buffer is a communication area which contains various

pointers used by both the handler and the user.  Some of these pointers

point into the user's address space and others point into the handler's

address space (using kernel mode mapping).  The next part of the buffer is

an output ring buffer of a size specified by the user.  The handler

determines this size from the value supplied by the user.  The last part of

the buffer is an input ring buffer of a size similarly specified by the user

and determined by the handler, but not necessarily the same size as the

output ring buffer.  Detail of the buffer contents is shown below:

- 4 -

offset			data			set by:	   modified by:

_______________________________________

0     | OG.PTR  next byte from output buf.  |	handler	   handler

|-------------------------------------|

2     | OP.PTR  next byte to output buffer  |	user	   user

|-------------------------------------|

4     | A.DIFF  addressing difference	    |   handler	   (read by both)

|-------------------------------------|

6     | O.BUFL  length of output buffer	    |   user	   (read by handler)

|-------------------------------------|

10    | O.FULL  output buffer full (byte)   |   user	   reset by handler

11    | O.STAT  output line status (byte)   |   either	   reset by handler

|-------------------------------------|

12    | IG.PTR  next byte from input buffer |   user	   user

|-------------------------------------|

14    | IP.PTR  next byte to input buffer   |   handler	   handler

|-------------------------------------|

16    | I.BUFL  length of input buffer	    |   user	   (read by handler)

|-------------------------------------|

20    | I.GRN   input length for X-ON	    |   user	   (read by handler)

|-------------------------------------|

22    | I.RED   input length for X-OFF	    |   user	   (read by handler)

|-------------------------------------|

24    | I.FULL  input buffer full (byte)    |   handler	   reset by user

25    | I.EXIT  the user is finished (byte) |   either	   (read by handler)

|-------------------------------------|

26    | I.RESP  respond to X-OFF/X-ON (byte)|   user	   (read by handler)

27    | SPARE			      (byte)|	 -		-

|-------------------------------------|

30    | I.RBO   # input ring buffer o'flows |   handler	   reset by user

|-------------------------------------|

32    | I.OVR   # DL11 overrun errors	    |   handler	   reset by user

|-------------------------------------|

34    | I.FRA   # DL11 frame errors	    |   handler	   reset by user

|-------------------------------------|

36    | I.PAR   # DL11 parity errors	    |   handler	   reset by user

|-------------------------------------|

|					    |

40    | output buffer of length O.BUFL	    |

|					    |

|-------------------------------------|

|					    |

40+   | input buffer of length I.BUFL	    |

O.BUFL|					    |

|					    |      _______________________________________

NOTES:

1) If I.RED is zero, then no X-OFF characters are sent by the

handler when the user buffer becomes full.

2) Under SJ and FB monitors, A.DIFF will be zero, otherwise the user

must add A.DIFF to the user's address to arrive at the corresponding

handler's address.

3) If IG.PTR is equal to IP.PTR (taking A.DIFF into account), then

either the buffer is empty, or it is full.  In the latter case,

I.FULL will be non-zero.

- 5 -

6.  EXITING FROM A TWO-WAY COMMUNICATION.

To terminate the two-way communication, the byte I.EXIT within the user's

buffer must be set by the user.  If this is not done, then any .EXIT request

will hang, waiting for the I/O to complete.  The MACRO sequence to achieve

this is as follows:

INCB	I.EXIT

.WAIT	channo

.EXIT

7.  ERROR RETURNS.

The completion routine versions of the .WRITEC or .SPFUN request are

recommended since the handler may return from the request immediately if the

port is not available.  The Channel Status Word reflects the error, with the

EOF bit always set if an error has occurred.  The HDERR bit is set if the

error was that the port was already in use by a different job or for a

different function.  If the HDERR bit is not set, this indicates that the

error is a buffer overflow (buffer larger than 8Kb, in XM or TSX-PLUS

systems only).