TCP/IP Package

for

TSX-Plus

TCP/IP Package for TXS-Plus

Submitted by Alan R.  Baldwin,

Kent State University, Kent, Ohio 44242

Operating System:  TSX-Plus(RT-11) Version 6.30 or later

Source Langauge:  DECUS C and RT-11 Macro

Abstract:

The  'TCP/IP Package for TSX-Plus' provides TCP/IP networking

services for multiple users with TSX-Plus.  The package provides

client/server  programs  for  TELNET, FTP, and MAIL.  A security

system allows usernames/passwords and  authorizations  for  each

client/server component to be specified for each user.

The  current  implementation uses 8 global regions to support

multiple client/server programs simultaneously.  The  number  of

regions used by each service is TELNET(1), FTP(2), and MAIL(1).

The  sources  for  the  'TCP/IP Package for TSX-Plus' include

header, command, C language, and macro files.   Included  is  an

executable  version  of  the DECUS C compiler and assembler, as-

sorted header files, and the DECUS C library files which may  be

used to rebuild the package.

CHAPTER 1  TSX-PLUS SYSTEM GENERATION                    1-1

1.1     BUSTYPE                                        1-1

1.2     NGR                                            1-2

1.3     DEVXMR                                         1-2

1.4     IOABT                                          1-2

1.5     DEVDEF                                         1-3

1.6     MESSAGE COMMUNICATION CHANNELS                 1-3

1.7     LINE DEFINITIONS                               1-4

1.8     DETACH                                         1-5

1.9     RECORD LOCKING                                 1-5

CHAPTER 2  HARDWARE INSTALLATION                         2-1

2.1     DEQNA INSTALLATION                             2-1

2.2     SERIAL PORT INSTALLATION                       2-1

CHAPTER 3  SOFTWARE INSTALLATION                         3-1

3.1     TCPIP NETWORK SERVER                           3-3

3.2     PASSWORD SETUP                                 3-7

3.3     MAIL SETUP                                     3-8

CHAPTER 4  PROGRAM OPTIONS                               4-1

4.1     TCPIP                                          4-1

4.2     SKTMON                                         4-2

4.3     TELNET                                         4-2

4.4     RTELNT                                         4-3

4.5     CNCT                                           4-4

4.6     RCNCT                                          4-5

4.7     FTP                                            4-6

4.8     RFTP                                           4-8

4.9     MAIL                                           4-9

4.10    RSMTP                                         4-11

4.11    UTIL                                          4-12

4.11.1    LCD                                         4-12

4.11.2    LDEL                                        4-12

4.11.3    LDIR                                        4-13

4.11.4    LLS                                         4-13

4.11.5    LMKDIR                                      4-13

4.11.6    LPRO                                        4-13

4.11.7    LRENAM                                      4-13

4.11.8    LRMDIR                                      4-14

4.11.9    LUNP                                        4-14

4.12    LDISKS                                        4-14

CHAPTER 5  EQ HANDLER                                    5-1

5.1     RESTRICTIONS                                   5-1

5.2     USE OF SPECIAL FUNCTIONS (.SPFUN)              5-2

5.2.1     Unit Specification (.SPFUN 200)              5-3

5.2.1.1     Allocate Unit                              5-3

5.2.1.2     Deallocate Unit                            5-4

5.2.2     Promiscuous Mode (.SPFUN 201)                5-5

5.2.2.1     Enable Promiscuous Mode                    5-5

5.2.2.2     Disable Promiscuous Mode                   5-6

5.2.3     Protocol Type (.SPFUN 202)                   5-7

5.2.3.1     Enable Protocol Type                       5-7

5.2.3.2     Disable Protocol Type                      5-8

5.2.4     Multicast Address (.SPFUN 203)               5-9

5.2.4.1     Enable Multicast Address                   5-9

5.2.4.2     Disable Multicast Address                 5-10

Page ii

5.2.5     Transmit Ethernet Frame (.SPFUN 204)        5-11

5.2.6     Receive Ethernet Frame (.SPFUN 205)         5-12

5.2.7     Frame Queueing (.SPFUN 206)                 5-13

5.2.7.1     Enable Frame Queueing                     5-14

5.2.7.2     Disable Frame Queueing                    5-14

5.2.8     Get Ethernet Address (.SPFUN 207)           5-15

5.2.9     Get Ethernet Status Block (.SPFUN 210)      5-16

CHAPTER 6  BUILDING THE TCP/IP PACKAGE                   6-1

6.1     C COMPILER                                     6-1

6.2     TCP/IP PACKAGE                                 6-1

APPENDIX A  UNDOCUMENTED OPTIONS                         A-1

A.1     DISABLE PASSWORD                               A-1

A.2     SUBDIRECTORY UNIT SELECTION                    A-1

APPENDIX B  DISTRIBUTION DIRECTORIES                     B-1

B.1     DISK 1 DIRECTORY                               B-1

B.2     DISK 2 DIRECTORY                               B-2

B.3     DISK 3 DIRECTORY                               B-2

B.4     DISK 4 DIRECTORY                               B-4

B.5     DISK 5 DIRECTORY                               B-4

Page iii

I N T R O D U C T I O N

The  'TCP/IP Package for TSX-Plus' is an assemblage of server

and client programs designed to provide Telnet,  FTP,  and  Mail

services  for  local  and remote users of TSX-Plus systems.  The

programs are written in C and RT-11 macro.  The DECUS C compiler

(with  some  bug  fixes)  and AS assembler are included with the

software distribution to allow the package to be recompiled from

the supplied sources.

The TCP/IP Package requires a TSX-Plus system of version 6.30

or later.  TSX-Plus versions 6.30 and later have a system EMT to

abort  any  job  (not  just a detached job), this is used by the

remote Telnet server (and  the  TSX-Plus  specific  remote  CNCT

server).

The  hardware  requirements  are  a  minimum of 512k bytes of

memory, a DEQNA  (Q-Bus)  Ethernet  controller  with  associated

thick  or  thin wire adapter, and optionally a pair of dedicated

serial lines for each simultaneously active remote  Telnet  ses-

sion.

Page iv

Alan R. Baldwin

Physics Department

Kent State University

Kent, Ohio  44242

Internet:

baldwin@shop-pdp.kent.edu

baldwin@ksuvxd.kent.edu

BITNET:

BALDWIN@KENTPHYS

CHAPTER 1

TSX-PLUS SYSTEM GENERATION

The TSX-Plus system must be regenerated to provide the neces-

sary device support, system services, and  options  required  by

the  TCP/IP  Package.  What follows is a description of each op-

tion that should be examined and updated  to  allow  the  proper

operation of the various hardware and software components of the

TCP/IP Package.

1.1  BUSTYPE

NOTE

The  DEQNA device used by the TCP/IP Package is a QBUS

device.

BUSTYP defines the machine bus structure for TSX-Plus.  Their

are two possible machine bus structures supported by TSX-Plus  -

the  QBUS  (LSI) and the UNIBUS.  Select one of these parameters

below to specify the bus support desired.  Use the following in-

formation for choosing the correct bus structure.

QBUS - 11/23, 11/23-Plus, 11/73, and Professional.

UNIBUS - 11/24, 11/34a, 11/44, and 11/60.

BUSTYP = QBUS   ;Specify machine bus structure (UNIBUS/QBUS)

TSX-PLUS SYSTEM GENERATION                              PAGE 1-2

NGR

1.2  NGR

NOTE

The  TCP/IP Package limits its use to 8.  PLAS regions

for passing data and parameters  between  the  network

server  and  the  various  client/server  tasks.   The

number of regions used by  each  active  client/server

program  is:   TELNET(1),  RTELNT(1), FTP(2), RFTP(2),

MAIL(1), RSMTP(1), CNCT(1) and RCNCT(1).

The  number of shared global PLAS regions that can be created

by all jobs.

NGR = 20.       ;Number of global PLAS regions

1.3  DEVXMR

NOTE

The DEQNA device handler EQ.TSX uses 1 PLAS region.

Some  device handlers allocate extended memory (PLAS) regions

for their use.  For example, the DU and MU handlers each require

one  PLAS region.  If you are using any other handlers which re-

quire extended memory regions, include the number of regions re-

quired.

DEVXMR = 4.     ;Number of XM regions for device handlers

1.4  IOABT

NOTE

The TCP/IP Package Network Server which uses the DEQNA

EQ.TSX handler internally queues the network requests.

If  IOABT is not set to 1 then the Network Server pro-

gram cannot be aborted.  This will also  be  true  for

the  remote  TELNET  and CNCT servers which must abort

I/O through the CL device handler.

TSX-PLUS SYSTEM GENERATION                              PAGE 1-3

IOABT

The  IOABT  parameter  controls  the action taken by TSX-Plus

when a job terminates execution.  If IOABT=0, TSX-Plus will wait

for  all  outstanding I/O pending for the job to complete before

the job is actually terminated.  If IOABT=1, TSX-Plus will  call

the  handler  abort  entry point for all outstanding I/O pending

for the job.  Note, the "SET IO NO ABORT" keyboard  command  may

be used to change the value of this parameter.

IOABT = 1       ;0==>I/O rundown, 1==>I/O abort

1.5  DEVDEF

NOTE

The device handler EQ.TSX must be added to the list of

system handlers loaded by TSX-Plus at system  startup.

The  handler  may  be mapped, uses DMA, and should not

allow SET operations on the loaded handler.

DEVDEF  <EQ>,DMA,MAPH,NOSET

1.6  MESSAGE COMMUNICATION CHANNELS

NOTE

Each  active client/server program of the TCP/IP Pack-

age uses 1 message channel.  Those programs using mes-

sage  channels are:  TCPIP, TELNET, RTELNT, FTP, RFTP,

MAIL, RSMTP, CNCT, and RCNCT.

Define  parameters  pertaining  to  the inter-program message

communication feature.  If this feature is not wanted,  set  all

four parameters to 0 (zero).

Maximum number of message communication channels which may be

simultaneously in use.

MAXMC   =       10.     ;Max message channels

Maximum message length (bytes).

TSX-PLUS SYSTEM GENERATION                              PAGE 1-4

MESSAGE COMMUNICATION CHANNELS

MSCHRS  =       200.    ;Max message length (bytes)

Maximum  of messages which may be held in queue.

MAXMSG  =       10.     ;Max queued messages

Maximum  of requests for messages that may be held in queue.

MAXMRB  =       10.     ;Max  pending message requests

1.7  LINE DEFINITIONS

NOTE

If  the  remote  TELNET  or  CNCT  servers  are  to be

anabled, then a pair of cross connected  serial  lines

must  be  added  to the Line Definition table for each

active session.  The  following  Line Definitions  il-

lustrate  the  additions  required.   If more than one

pair of lines are added then  select  sequential  time

share  and  sequential  CL lines.  The SILO length de-

finition for the CL line should  be  greater  than  or

equal  to the maximum number of characters it will re-

ceive in .5 seconds (120 for 2400 baud or 240 for 4800

baud).   This  will reduce XON/XOFF sequences and max-

imize the serial transfer rate.

The TBLDEF macro call requires four arguments:

1. The number of real time-sharing lines on machine.

2. The number of subprocess jobs.

3. The number of detached jobs.

4. The number of dedicated CL lines.

TBLDEF  3.,10.,6.,1.    ;# Real, # Subprocess,

;# Detached, # CL lines

#3 time-sharing line is cross linked to CL0:

LINDEF  340,175600

CMDFIL  LINE3.TSX

TRMTYP  VT100

FLAGS   NRMFLG

LINEND

CL0: is cross linked to #3 time-sharing line

CLDEF   0,350,175610

SILO    256.,12.,4.

TSX-PLUS SYSTEM GENERATION                              PAGE 1-5

LINE DEFINITIONS

CLEND

1.8  DETACH

NOTE

The  command  file  DETACH.TSX  is used to startup the

Network Server.

Use  the  "DETACH" macro here to declare any start-up command

files and associated parameters (up to 80 characters) to be  run

as detached jobs:

DETACH  <SY:DETACH.TSX>         ;Start-up detached job

1.9  RECORD LOCKING

NOTE

If  the  MAIL  or  RSMTP  programs are to be used then

record locking is necessary  to  properly  update  the

mail sequence files.

Define  parameters  pertaining  to record (block) locking for

shared files.  If the shared file block locking facility is  not

wanted, set all of these parameters to 0 (zero).

Maximum  number  of  shared  files  which  may be open simul-

taneously.  Note that several  users  accessing  the  same  file

count as 1.

MAXSF   =       4.      ;Max number of shared files

Maximum  number  of  I/O  channels  which  all  users may simul-

taneously have open to shared files.  Note, this  is  the  total

number for all users not for each user.

MAXSFC  =       16.     ;Max  shared file channels

Maximum number of blocks which may be simultaneously held locked

TSX-PLUS SYSTEM GENERATION                              PAGE 1-6

RECORD LOCKING

by any channel.  That is, max blocks locked per channel.

MXLBLK  =       16.     ;Max blocks locked per channel

CHAPTER 2

HARDWARE INSTALLATION

The  TSX-PLUS system must be Q-Bus based (or have a Unibus to

Q-Bus adapter) to  allow  the  installation  of  a  DEQNA  Q-Bus

Ethernet  controller.   A  system memory of at least 512k is re-

quired when installing the TCP/IP  Package.   The  DEQNA  EQ.TSX

handler requires 36k bytes for code and packet buffers, the net-

work server program TCPIP will use 63k bytes,  and  each  global

PLAS  region  created  by  TCPIP  uses 8k bytes for a maximum of

approximately 163k bytes  before  activating  any  client/server

programs.   Each  active  client/server program will use upto an

additional 63k bytes.  The client/server programs are not  over-

layed and may be run from floppy disk drive only systems.

2.1  DEQNA INSTALLATION

The DEQNA should be the highest priority device on the Q-Bus,

that is, the DMA device nearest to the CPU.  Blockmode memory is

recommended although not necessary.  The DEQNA interface is con-

nected via a bulkhead cable assembly (with perhaps an  extension

Ethernet  transceiver  cable)  to a type H4000 transceiver (with

either a thick or thin wire connection).

2.2  SERIAL PORT INSTALLATION

If  the system is to be configured for remote Telnet sessions

then a pair of dedicated  serial  lines  is  required  for  each

simultaneously  active session.  Each dedicated pair consists of

a TSX-PLUS timeshare line cross connected to a  CL  line  via  a

NULL-MODEM cable.

For  a  DLV11-J  four port serial board the cross connections

may be easily obtained by constructing a 4" cable  with  the  10

wire  flat cable flipped between the two ends (1->10, 2->9, ....

10->1).  Then cut wires 1,2,9,and 10 at both connectors  leaving

HARDWARE INSTALLATION                                   PAGE 2-2

SERIAL PORT INSTALLATION

a   6-wire  NULL-MODEM  cable.   Two  such  cables  allow  PORT1

(TSX-timeshare line) to connect  to  PORT2(CL  line)  and  PORT3

(TSX-timeshare  line)  to connect to PORT3(CL line) allowing two

simultaneous remote Telnet  sessions.   The  DLV11-J  baud  rate

should  be  limited to 4800 as two serial ports are required for

each session and  is  a  relatively  heavy  burden  even  for  a

LSI-11/73.

CHAPTER 3

SOFTWARE INSTALLATION

The TCP/IP Package software components should be installed as

described  in  the  following  sections.   The  Ethernet  device

handler  and  all the *.com files must be copied to the TSX-Plus

system device.  The command files assume that the  client/server

programs  are  on  the system device.  If the client/server pro-

grams are placed on an alternate device then modify the  command

files appropriately.

For  security reasons it is suggested that a paswrd subdirec-

tory be created, PASWRD.DSK[100], and may be on any device.  The

files  PASWRD.SAV  and  PASWRD.FIL should be copied to this sub-

directory.  The PASWRD.COM command file should be updated to re-

flect  the location of the password files.  A seperate directory

allows the exclusion of its access by remote FTP sessions.   Lo-

cal  users  or  remote telnet sessions with appropriate TSX-Plus

access rights may still access the password files.

The   following   annotated   list  summarizes  the  software

components:

1.  Device Handler

1.  EQ.TSX           DEQNA device handler

2.  TCP/IP Network Server

1.  DETACH.TSX       detached job startup command file

2.  TCPIP.COM        TCPIP startup command file

3.  TCPIP.SAV        TCPIP network server Program

4.  TCPIP.CFG        TCPIP configuration file

3.  Telnet Client

1.  TELNET.COM       TELNET startup command file

2.  TELNET.SAV       TELNET client program

SOFTWARE INSTALLATION                                   Page 3-2

4.  Remote TELNET Server

1.  RTELNT.SRV       TELNET server detached job startup

2.  RTELNT.COM       TELNET server startup command file

3.  RTELNT.SAV       TELNET server program

4.  RTELNT.MSG       TELNET server message file

5.  RTELNT.HLP       TELNET server help file

5.  CNCT Client

1.  CNCT.COM         CNCT startup command file

2.  CNCT.SAV         CNCT client program

6.  Remote CNCT Server

1.  RCNCT.SRV        CNCT server detached job startup

2.  RCNCT.COM        CNCT server startup command file

3.  RCNCT.SAV        CNCT server program

4.  RCNCT.MSG        CNCT server message file

5.  RCNCT.HLP        CNCT server help file

7.  FTP Client

1.  FTP.COM          FTP startup command file

2.  FTP.SAV          FTP client program

3.  FTPCMD.TXT       FTP command help text

8.  Remote FTP Server

1.  RFTP.SRV         FTP server detached job startup

2.  RFTP.COM         FTP server startup command file

3.  RFTP.SAV         FTP server program

4.  RFTP.MSG         FTP server message file

5.  RFTP.HLP         FTP server help file

9.  MAIL Client

1.  MAIL.COM         MAIL startup command file

2.  MAIL.SAV         MAIL client program

3.  MAICMD.TXT       MAIL command help text

10.  Remote MAIL Server

1.  RSMTP.SRV        MAIL server detached job startup

2.  RSMTP.COM        MAIL server startup command file

3.  RSMTP.SAV        MAIL server program

4.  RSMTP.HLP        MAIL server help file

11.  Password and Authorization Files

1.  PASWRD.COM       Password startup command file

2.  PASWRD.SAV       Password program

SOFTWARE INSTALLATION                                   Page 3-3

3.  PASWRD.FIL       Password and authorization data file

12.  Client/Server Monitor

1.  SKTMON.COM       Monitor startup command file

2.  SKTMON.SAV       Monitor program

13.  Subdirectory Utilities

1.  LCD.COM          Change directory command file

2.  LDEL.COM         Delete file command file

3.  LDIR.COM         List directory command file

4.  LLS.COM          List directory command file

5.  LMKDIR.COM       Make directory command file

6.  LPRO.COM         Protect file command file

7.  LRENAM.COM       Rename file command file

8.  LRMDIR.COM       Delete Subdirectory command file

9.  LUNP.COM         Unprotect file command file

10.  UTIL.SAV         Utilities program

14.  Subdirectory Summary Utility

1.  LDISKS.COM       Subdirectory summary command file

2.  RFTHLP.COM       Generate RFTP.HLP command file

3.  LDISKS.SAV       Subdirectory summary program file

15.  Miscellaneous Files

1.  ABRTCP.SAV       Abort all Client/Server programs

2.  KILLM.SAV        Kill all Client/Server Messages

3.  REMOVE.COM       Remove all Client/Server global regions

3.1  TCPIP NETWORK SERVER

The  network  server  program TCPIP.SAV provides the software

interface between  the  DEQNA  device  handler  EQ.TSX  and  the

client/server  programs  in  the  TCP/IP  Package.   The network

server program handles the processing of various network packets

and  services  (TCP, IP, UDP, ICMP, BOOTP, ARP, RARP, and DOMAIN

name lookup).

The   TCPIP.SAV   program  requires  the  configuration  file

TCPIP.CFG which defines all the parameters required to configure

the  network  server.   Typical configuration parameters are de-

tailed in the following configuration file:

SOFTWARE INSTALLATION                                   PAGE 3-4

TCPIP NETWORK SERVER

#

#  Example host file for TCP/IP server

#

#  This file is free form

#  Separators are any char <32, 'space' and '='

#  Comments are preceeded by #

#

#  The form is keyword=value for each parameter.

#  The first set of parameters refer to the whole program's defaults.

#  These parameter values can be in any order.

#  Following this are the individual machine specs.

#  If the first machine is name "default", then it contains default

#  values for the rest of the machines.

#

#       These options are specified for this host machine:

#

#       myname=____.____.____   # my internet name

#       myip=___.___.___.___    # host machine's IP number

#           =BOOTP              # to use BOOTP to get host info

#           =RARP               # to use RARP to get IP number

#       netmask=___.___.___.___ # subnetting mask

#       domain=____.____        # default domain for lookup

#       nndomto=____            # timeout for domain lookups

#       nnretry=____            # number of retries for lookup

#       nnarpto=____            # timeout for ARPs

#       nndto=____              # timeout for data layer

#       nnwin=____              # TCPIP upper limit window size

#       nnseg=____              # TCPIP upper limit input segment size

#       nnmtu=____              # TCPIP upper limit output transfer size

#       service=___,___,_____   # TCPIP services specification

#       logsession=__________   # TCPIP session logging filespec

#

#

#       These are the options which may be specified for each

#       individual machine specification:

#

#       name=____               # name of session

#       host=____               # full name of host computer

#       hostip=___.___.___.___  # IP number of host

#       port=____               # specify initial connection port

#       nameserver=____         # name server level

#       gateway=____            # gateway level

#       retrans=____            # initial retransmit timeout

#       contime=____            # opening connection timeout

#       mwin=____               # advertized window size for this host

#       mseg=____               # maximum advertized input segment size

#       mtu=____                # maximum output transfer size

#       delete=backspace        # value to use for character deletion

#       crmap=4.3BSDCRNUL       # for 4.3BSD null CR

#       duplex=half             # for half duplex TELNET connections

#       copyfrom=____           # copy parameters from this name

#

SOFTWARE INSTALLATION                                   PAGE 3-5

TCPIP NETWORK SERVER

myname=shop-pdp.kent.edu

myip=131.123.1.153      # required, this machine's IP number

netmask=255.255.255.0   # subnetting mask

domain="kent.edu"       # domain name search path

nndomto=1               # domain timeout

nnretry=4               # number of retries

nnarpto=1               # arp timeout in seconds

# affects machines on your local network

#  The following line specifies the otpional TCPIP logging file

#  which will contain information for each session started.

logsession=du0:tcpip.log[100]

#  The following are network servers supported by TCPIP.

#  Each entry contains the listen port, maximum number of

#  sessions and the name of the command file to initiate

#  the service.

service=21,2,sy:rftp.srv        # remote ftp server

service=23,2,sy:rtelnt.srv      # remote telnet server

service=25,2,sy:rsmtp.srv       # remote smtp server

service=27,2,sy:rcnct.srv       # remote cnct server

#

#  Following are individual machine specifications

#  Gateways are used in the order that they appear in the file

#  Nameservers rotate, #1, #2, #3, #1, #2 when a request fails

#

#  The machine named "default" contains the fields which

#  are automatically filled in for later hosts

#

name=default            # Session name, "default" is a reserved name

#   Not a real machine, default parameters only

delete=backspace        # Assume mostly UNIX connections

#  These entries affect the tuning of TCP connections to this host.

#  They should be set by the network administrator who is familiar

#  with the requirements of your specific network.

contime=12              # timeout in seconds to try connection

#   before returning error to user

retrans=5               # starting retransmit time out in seconds

mtu=1408                # maximum transmit unit in bytes

#   outgoing packet size, MAX=1408

mseg=1408               # largest segment we can receive

#   whatever the hardware can take, MAX=1408

mwin=2816               # most bytes we can receive without ACK

#   =TCP window size, MAX=3071

#   larger isn't always better

SOFTWARE INSTALLATION                                   PAGE 3-6

TCPIP NETWORK SERVER

#

#  Below this line, most of the communication parameters are obtained

#   from the "default" host entry.

#  Machine names, IP addresses, and special communication parameters are

#   present when needed.

#

name=mygateway          host=ksupra.kent.edu

hostip=131.123.1.3

gateway=1

name=vaxa               host=ksuvxa.kent.edu

hostip=131.123.1.1

delete=delete

nameserver=1

name=mathserver         host=mcs.kent.edu

hostip=131.123.2.130

nameserver=2

name=vaxc               host=ksuvxc.kent.edu

hostip=131.123.1.14

copyfrom=vaxa

name=vaxd               host=ksuvxd.kent.edu

hostip=131.123.1.156

copyfrom=vaxa

name=scorpio            host=scorpio.kent.edu

hostip=131.123.1.50

name=djohn-sun          host=djohn-sun.kent.edu

hostip=131.123.1.151

name=ksuacq             host=ksuacq.kent.edu

hostip=131.123.1.60

copyfrom=vaxa

name=nmr11              host=nmr11.kent.edu

hostip=131.123.1.150

copyfrom=vaxa

name=shop-pdp           host=shop-pdp.kent.edu

hostip=131.123.1.153

copyfrom=vaxa

name=djohn-pdp          host=djohn-pdp.kent.edu

hostip=131.123.1.154

copyfrom=vaxa

SOFTWARE INSTALLATION                                   PAGE 3-7

TCPIP NETWORK SERVER

The  network  server  needs to know the ip address(number) of

the machine to which it is connecting.  If  the  TCPIP.CFG  file

does not contain the named machine's ip number then it will make

a request to the specified name server to obtain the machines ip

number.   If  a name server is not specified then the connection

cannot be made using the machines name (place the machines  name

and ip address in the configuration file).  If the ip address is

given as the input to the client program rather  than  the  name

then a domain name lookup is not required.

All changes to the configuration file will take effect at the

next startup of TCPIP.SAV.

3.2  PASSWORD SETUP

A  password  file  (eg  PASWRD.FIL)  must be built if the lo-

cal/remote MAIL services are to used or if remote network access

(ie  remote FTP, TELNET or CNCT) must be restricted.  It is sug-

gested that a seperated password directory be  created  to  hold

the  files PASWRD.SAV and PASWRD.FIL.  This directory need be no

larger than about 200 blocks.  The maximum number of  user/pass-

word  specifications  is  limited  by the program memory size to

about 50.  The PASWRD.FIL contains 1 block for  each  user/pass-

word specification.

Copy  the  file PASWRD.SAV (and optionally the supplied pass-

word file PASWRD.FIL) to  the  created  directory.   Modify  the

PASWRD.COM  file  on  the system device to access this subdirec-

tory.

Initiating  PASWRD.SAV with a file specification will open an

existing file or create the file if no existing file  is  found.

The PASWRD program screens will indicate all the options and the

file USRBLK.H illustrates the  formats  for  the  directory/sub-

directory/file specifications required by the password program.

NOTE

The  password  for  any added user must also be speci-

fied,  any  user  not  requiring  a  password  (ie  an

'anonymous'  account)  must have this password entered

as a null string by typing only a carriage return  for

the password.

To  use the created password file be sure the program startup

command files have specified the correct path  to  the  password

file.

SOFTWARE INSTALLATION                                   PAGE 3-8

PASSWORD SETUP

NOTE

The directory/subdirectory/file access rights are only

used by the remote FTP server.  Logins via  TELNET  or

CNCT  rely  upon  any  security  features  enabled  in

TSX-Plus and not by the PASWRD.FIL specifications.

3.3  MAIL SETUP

The  MAIL  programs  MAIL.SAV  and  RSMTP.SAV make use of the

password file to restrict access to the mail system.  A user may

have local and/or remote MAIL access (or no access) depending on

the password file parameters.

Incoming  mail  may be directed to a user's default directory

or to a specific  mail  directory  (specified  in  the  password

file).   A  typical configuration is to create a mail directory,

DU0:MAIL.DSK, and have mail subdirectories for each user.   Thus

the  path  DU0:/MAIL/USER1/  specifies  the  user's  unique mail

directory and should be specified in the password configuration.

The  local  mail  program  MAIL.SAV  and the remote mail program

RSMTP.SAV check the password file to verify that  the  user  has

the privilege to send or receive mail via the network and places

any received mail in the specified mail directory.   These  pro-

grams  will  use/create  a mail index file in the mail directory

which is used to keep an index of all received mail.

CHAPTER 4

PROGRAM OPTIONS

The various client/server/utility program startup options are

outlined and typical startup command files listed.

4.1  TCPIP

The  TCPIP.SAV  program may be started interactively (usually

for diagnostic purposes) with the following options:

TCPIP [?] [-d level] [-f filename]

?               List this Help Text and Exit TCPIP

d  level        Debug Level

1 -   enable event printing

2 -   enable packet type printing

4 -   enable general packet dumping to disk

8 -   enable ntsleep process printing

16 -   enable transq sequence printing

32 -   enable arp/rarp packet dumping to disk

64 -   enable udp packet dumping to disk

128 -   enable icmp packet dumping to disk

256 -   enable tcp packet dumping to disk

Enter level as sum of debug values

f  filename     Configuration File

The  startup command file TCPIP.COM deassigns all devices and

then assigns the network server device EN to be the DEQNA device

EQ.   Four EN units are used by TCPIP, three are used to receive

IP(EN0:), ARP(EN1:), and RARP(EN2:) packets and one is  used  to

transmit(EN3:) all packet types.

PROGRAM OPTIONS                                         PAGE 4-2

TCPIP

deassign

assign eq0: en0:

assign eq1: en1:

assign eq2: en2:

assign eq3: en3:

r/single tcpip

^1 ^2 ^3 ^4 ^5 ^6

The  TCPIP program is usually started at TSX-Plus startup us-

ing the file DETACH.TSX:

detach sy:tcpip.com

4.2  SKTMON

The  SKTMON  program provides a diagnostic tool for observing

the state of all TCPIP connections.  SKTMON displays the current

state  of  network  connections  by  showing  the global regions

through  which  the  TCPIP  network  server  program   and   the

client/server programs communicate.  The program shows in detail

the various network packets,  the  incoming  and  outgoing  data

streams, and the status of the network Ethernet driver.

The  startup  command  file  contains  an assignment to allow

SKTMON to access the status of the device driver:

assign eq0: en0:

r/single sktmon 0 ^1

4.3  TELNET

The  TELNET  program allows the user to initiate a connection

to a remote host for a terminal session.   The  startup  options

for TELNET are:

TELNET destination [?] [-hp] [-d level]

?               List the Help Text and Exit TELNET

d   level       Debug Level

1 -  enable event printing

2 -  enable telnet negotiation printing

h               List the Help Text and Open Connection

p   filename    Specify the Password Filespec

The  destination  host  specification may be the host name or

host ip number.  If the connection is to be made to an alternate

port then follow the name/ip with the port number (decimal).

PROGRAM OPTIONS                                         PAGE 4-3

TELNET

The  TELNET  program  supports the following command line op-

tions which are activated by the META character 'control A':

M->C    open capture file

M->D    close capture file

M->F    FTP [internet address]

M->H    this help screen

M->I    type my internet address

M->K    erase character

M->O    abort output

M->Q    are you there?

M->R    toggle <CR> follow character

M->S    skip to end of buffer

M->T    toggle backspace/delete

M->U    erase line

M->X    close connection

M->Y    interrupt process

M->@    send a true null character

^?      abort Telnet session

The startup command file TELNET.COM follows:

r/single telnet

-hb -p du0:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5 ^6

4.4  RTELNT

RTELNT  is  a  server program for remote TELNET sessions into

TSX-Plus.  The program startup options are:

RTELNT [?] [-d level] [-hmp filespec] [-acelrt] [parameters]

?       List the Help Text and Exit RTELNT

d  l    Debug Level

1 -  enable event printing

a       Attach to Service

c  i    CL line i is cross connected to TSX line j

e       Enable Monitoring of all Transactions

h       Specify the Help Filespec

l  n    Number of CL/TSX line pairs

m       Specify the Message Filespec

p       Specify the Password Filespec

r       Restart RTELNT after Disconnect

t  j    TSX line j is cross connected to CL line i

The program may be started interactively (for diagnostic pur-

poses) by the command file RTELNT.COM:

PROGRAM OPTIONS                                         PAGE 4-4

RTELNT

r/single rtelnt

-ctl 0 2 1 -p du0:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5

^6

The  program  is normally invoked by the TCPIP network server

program by the command file RTELNT.SRV:

deassign

assign sy: wf:

r/single rtelnt

-a -ctl 0 2 1 -p du0:/paswrd.dsk/paswrd.fil

The   RTELNT  help  and  message  files  are  RTELNT.HLP  and

RTELNT.MSG respectively.  The h or m  options  may  be  used  if

these  files  are  not  on  the  system device.  These files are

created by the system manager and are not required.

4.5  CNCT

The  CNCT  program allows the user to initiate a connection to a

remote TSX-Plus host for a terminal session.   This  program  is

the  TELNET program stripped of all TELNET negotiations and uses

a non standard TCP port (27).  The startup options for CNCT are:

CNCT destination [?] [-hp] [-d level]

?               List the Help Text and Exit CNCT

d   level       Debug Level

1 -  enable event printing

2 -  enable telnet negotiation printing

h               List the Help Text and Open Connection

p   filename    Specify the Password Filespec

The  destination  host  specification may be the host name or

host ip number.  If the connection is to be made to an alternate

port then follow the name/ip with the port number (decimal).

The  CNCT program supports the following command line options

which are activated by the META character 'control A':

M->C    open capture file

M->D    close capture file

M->F    FTP [internet address]

M->H    this help screen

M->I    type my internet address

M->O    abort output

M->Q    are you there?

M->S    skip to end of buffer

M->X    close connection

M->Y    interrupt process

PROGRAM OPTIONS                                         PAGE 4-5

CNCT

^?      abort CNCT session

The startup command file CNCT.COM follows:

r/single cnct

-hb -p du0:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5 ^6

4.6  RCNCT

RCNCT  is  a  server  program  for  remote CNCT sessions into

TSX-Plus.  This program is the RTELNT program  stripped  of  all

TELNET  negotiations and uses a non standard TCP port (27).  The

startup options for RCNCT are:

RCNCT [?] [-d level] [-hmp filespec] [-acelrt] [parameters]

?       List the Help Text and Exit RCNCT

d  l    Debug Level

1 -  enable event printing

a       Attach to Service

c  i    CL line i is cross connected to TSX line j

e       Enable Monitoring of all Transactions

h       Specify the Help Filespec

l  n    Number of CL/TSX line pairs

m       Specify the Message Filespec

p       Specify the Password Filespec

r       Restart RCNCT after Disconnect

t  j    TSX line j is cross connected to CL line i

The program may be started interactively (for diagnostic pur-

poses) by the command file RCNCT.COM:

r/single rcnct

-ctl 0 2 1 -p du0:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5

^6

The  program  is normally invoked by the TCPIP network server

program by the command file RCNCT.SRV:

deassign

assign sy: wf:

r/single rcnct

-a -ctl 0 2 1 -p du0:/paswrd.dsk/paswrd.fil

The  RCNCT help and message files are RCNCT.HLP and RCNCT.MSG

respectively.  The h or m options may be used if these files are

not on the system device.  These files are created by the system

manager and are not required.

PROGRAM OPTIONS                                         PAGE 4-6

FTP

4.7  FTP

The  FTP  program  provides  reliable  data  transfer between

cooperating computer systems.  The FTP program  startup  options

are:

FTP [?] [-d level] [-f filename] [-p filename]

[-ghinrv] [destination host]

?               List this Help Text and Exit FTP

d  level        Debug Level

1 -     enable event printing

2 -     enable command printing

4 -     enable ftpdo() printing

8 -     enable ftpport() printing

16 -     enable ftpreplies printing

32 -     enable rgetline() printing

64 -     enable userftpd() printing

128 -     enable getword() printing

256 -     enable getnname() printing

f  filename     Command File

g               Wildcard Expansion Disabled

h               FTP Help List

i               Interactive Prompting Off

n               AutoLogin Disabled on Connect

p  filename     Specify the Password Filespec

r               Output Redirection Disabled

v               Verbose Mode Disabled

The  destination  host  specification may be the host name or

host ip number.  If the connection is to be made to an alternate

port then follow the name/ip with the port number (decimal).

The FTP startup command file is:

r/single ftp

-hb -p du0:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5 ^6

The FTP program supports the following command line options:

?               print local help information

ascii           set ascii transfer type

bell            toggle command completed beep

bget            get a file in binary mode

binary          set binary transfer type

bput            put a file in binary mode

bye             terminate ftp session and exit

cd              change remote working directory

close           terminate ftp session

debug           toggle/set debugging mode

PROGRAM OPTIONS                                         PAGE 4-7

FTP

delete          delete remote file - inquires if prompting is on

dir             list contents of remote directory

get             receive file

hash            toggle printing `#' for each 1024 bytes sent

help            print local help information

home            set default directory to home

interactive     turn on prompting for multiple commands

lcd             change local working directory

ldelete         delete local files

ldir            list contents of local directory

lls             list contents of local directory

lmkdir          make a local directory

lprotect        set protection flag on local file

lpwd            show local working directory

lrename         rename a local file

lrmdir          remove a local directory

ls              list contents of remote directory

ltype           print a local file to the screen

lunprotect      clear protection flag on a local file

mdelete         delete multiple files

mdir            list contents of multiple remote directories

mget            get multiple files

mkdir           make directory on the remote machine

mls             list contents of multiple remote directories

mode            set file transfer mode

mput            send multiple files

noninteractive  turn off prompting on multiple commands

open            connect to remote ftp

prompt          toggle interactive prompting

protect         set protection flag on file

put             send one file

pwd             print working directory on remote machine

quit            terminate ftp session and exit

quote           send arbitrary ftp command

recv            receive file

remotehelp      get help from remote server

rename          rename remote file

rm              delete remote file

rmdir           remove directory on the remote machine

send            send one file

sendport        toggle use of PORT cmd for each data connection

status          show current status

struct          set file transfer structure

system          query remote system type

type            print a remote file to the screen

unprotect       clear protection flag on file

user            send new user information

verbose         toggle verbose mode

version         FTP-11 version displayed

wild            toggle wildcard expansion of local file names

>   <filespec>  redirect remote response to specified file

PROGRAM OPTIONS                                         PAGE 4-8

RFTP

4.8  RFTP

RFTP  is  a  server  program  for  remote  FTP  sessions into

TSX-Plus.  The program startup options are:

RFTP [?] [-d level] [-hmp filespec] [-aer]

?               List the Help Text and Exit RFTP

d  level        Debug Level

1 -   enable event printing

2 -   enable dosess() printing

4 -   enable rftpd() printing

8 -   enable ftpd() printing

a               Attach to Service

e               Monitor all RFTP Transactions

h               Specify the Help Filespec

m               Specify the Message Filespec

p               Specify the Password Filespec

r               Restart RFTP after Disconnect

The program may be started interactively (for diagnostic pur-

poses) by the command file RFTP.COM:

r/single rftp

-r -p du0:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5 ^6

The  program  is normally invoked by the TCPIP network server

program by the command file RFTP.SRV:

deassign

assign sy: wf:

r/single rftp

-a -p du0:/paswrd.dsk/paswrd.fil

The  RFTP  help  and  message files are RFTP.HLP and RFTP.MSG

respectively.  The h or m options may be used if these files are

not on the system device.  These files are created by the system

manager and are not required.  The command file RFTHLP.COM  uses

the  subdirectories  utility LDISKS to create a logical disk map

for inclusion in the RFTP.HLP file.

PROGRAM OPTIONS                                         PAGE 4-9

MAIL

4.9  MAIL

The  MAIL  program provides local and internet mail services.

The startup options for MAIL are:

MAIL [?] [-d level] [-fp filespec] [-i IP Address]

?               List the Help Text and Exit MAIL

d  level        Debug Level

1 -     enable event printing

2 -     enable mailpi() printing

4 -     enable checkusps printing

f  filename     Command File

h               Help list

i  IP Address   Intermediate Host IP Address

p  filename     Specify the Password Filespec

The  Intermediate  Host IP option provides a means of routing

the internet mail through another host  machine.   The  TSX-Plus

MAIL  program does not support retry operations if the mail can-

not be delivered, using an intermediate host supporting  retries

will  provide this feature to the TSX-Plus MAIL system.  Permis-

sion to use a particular host as a MAIL relay should be obtained

from  the  hosts'  system manager.  The option must be specified

using the complete name (an IP address may  also  be  used).   A

typical invocation follows:

-i ksuvxd.kent.edu

The  Intermediate Host IP option may also allow users to send

mail through BITNET if the intermediate host is  a  BITNET  node

and  can  perform  the  translation  from internet addressing to

BITNET addressing.  The normal BITNET style address is typically

extended to form a psuedo internet address:

USER@KENTPHYS -->> USER@KENTPHYS.BITNET

The  intermediate host performs the necessary message format and

address translation and forwards the  message  via  BITNET.   If

BITNET  messages are to be forwarded to the TSX-Plus system then

the intermediate host must be configured to perform  the  BITNET

to internet translation and forward the BITNET mail via internet

to your TSX-Plus system.

The MAIL program is invoked from the command file MAIL.COM:

assign sy: wf:

r/single mail

-h -p du0:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5 ^6

PROGRAM OPTIONS                                        PAGE 4-10

MAIL

The  specification of device wf:  is required by MAIL to copy

a mail file to a specified directory or create a temporary  mail

file  before  delivering a local mail message.  If wf:  is unde-

fined, inaccessable, or not initialized  then  these  operations

will  fail.   The  specification  of wf:  may be included in the

TSX-Plus LINEx.TSX startup files rather than in MAIL.COM.

The MAIL program supports the following command line options:

?               print local help information

bye             terminate the MAIL program

copy            copy a message to a file

debug           toggle/set debugging mode

delete          delete a message

dir             list of unread mail messages

exit            terminate the MAIL program

hash            toggle printing `#' for each 1024 bytes sent

help            print local help information

home            go to home (initial) directory

index           an index of received mail

interactive     turn on prompting for multiple commands

lcd             change local working directory

ldelete         delete local files

ldir            list contents of local directory

listmail        list of unread mail messages

lls             list contents of local directory

lmkdir          make a local directory

lprotect        set protection flag on local file

lpwd            show local working directory

lrename         rename a local file

lrmdir          remove a local directory

ltype           print a local file to the screen

lunprotect      clear protection flag on a local file

mail            go to mail directory

newmail         count number of new mail messages

noninteractive  turn off prompting on multiple commands

prompt          toggle interactive prompting

quit            terminate the MAIL program

read            read a mail message

scan            scan directory for all mail messages

send [filespec] initiate the sending of a message

status          show current status

user            enter a new user/password access key

verbose         toggle verbose mode

version         MAIL-11 version displayed

wild            toggle wildcard expansion of local file names

Mail  is sent to a local user if only a name is specified and

to a remote machine if the user and IP address are specified.

Local:          username

PROGRAM OPTIONS                                        PAGE 4-11

MAIL

Internet:       @username:shop-pdp.kent.edu

4.10  RSMTP

RSMTP is a server program for remote MAIL into TSX-Plus.  The

program startup options are:

RSMTP [?] [-d level] [-hp filespec] [-aer]

?               List the Help Text and Exit RSMTP

d  level        Debug Level

1 -     enable event printing

2 -     enable dosess() event printing

4 -     enable rsmtp() printing

8 -     enable putmail() / index() printing

16 -     enable checkuser() printing

a               Attach to Service

e               Monitor all RSMTP Transactions

h               Specify the Help Filespec

p               Specify the Password Filespec

r               Restart RSMTP after Disconnect

The RSMTP help file RSMTP.HLP is of diagnostic use only.

The program may be started interactively (for diagnostic pur-

poses) by the command file RSMTP.COM:

r/single rsmtp

-r -p du0:/paswrd.dsk/paswrd.fil ^1 ^2 ^3 ^4 ^5 ^6

The  program  is normally invoked by the TCPIP network server

program by the command file RSMTP.SRV:

deassign

assign sy: wf:

r/single rsmtp

-a -p du0:/paswrd.dsk/paswrd.fil

The specification of device wf:  is required by RSMTP to tem-

porarily store incoming mail messages before writing the mail to

the users mail directories.  If wf:  is undefined, inaccessable,

or not initialized then incoming mail will not be delivered.

PROGRAM OPTIONS                                        PAGE 4-12

UTIL

4.11  UTIL

The program UTIL is a compendium of various functions used to

perform useful operations on TSX-Plus subdirectories.  The  com-

mand options for UTIL are:

UTIL [?] [-cdlmnpqru] [file-spec(s)]

?       List this Help Text and Exit UTIL

c       change directory

d       delete a file

l       list a directory

m       make a directory

n       delete a directory

p       set protection flag on a file

q       disable query on delete

r       rename a file

u       clr protection flag on afile

Each of the options is invoked by a command file more indica-

tive of its intended function.

4.11.1  LCD

Change directory command file LCD.COM:

r/single util

-s 0 2 -c ^1

@wf:cmdfil

4.11.2  LDEL

Delete file command file LDEL.COM:

r/single util

-d ^1

PROGRAM OPTIONS                                        PAGE 4-13

UTIL

4.11.3  LDIR

List directory command file LDIR.COM:

r/single util

-l ^1

4.11.4  LLS

List directory command file LDIR.COM:

r/single util

-l ^1

4.11.5  LMKDIR

Make a subdirectory command file LMKDIR.COM:

r/single util

-m ^1

4.11.6  LPRO

Set protection flag command file LPRO.COM:

r/single util

-p ^1

4.11.7  LRENAM

Rename a file command file LRENAM.COM:

r/single util

-r ^1 ^2

PROGRAM OPTIONS                                        PAGE 4-14

UTIL

4.11.8  LRMDIR

Remove a subdirectory command file LRMDIR.COM:

r/single util

-n ^1

4.11.9  LUNP

Clear protection flag command file LUNP.COM:

r/single util

-u ^1

4.12  LDISKS

The  LDISKS  program  generates  a  subdirectory map for each

device specified and optionally outputs the result to a file.

LDISKS [?] [-f filename] [dev1] [dev2] ...

?               List this Help Text and Exit LDISKS

f  filename     Output File Name

The   command   file   LDISKS.COM   allows   arbitrary  input

parameters:

r ldisks

^1 ^2 ^3 ^4 ^5 ^6

An example of its use is:

LDISKS -f disk.map du2:

The output file disk.map might look like:

Logical Subdirectories for du2:

du2:

`--> maclib[ 2000]

`--> pictur[ 6000]

`-->    phd[14000]

`--> paswrd[  500]

`-->   mail[ 1000]

`--> baldwi[  500]

`-->  guest[  200]

`--> anonym[  200]

CHAPTER 5

EQ HANDLER

The  EQ  Ethernet  handler  provides  support  for  the DEQNA

Ethernet controller for Q-BUS systems.  The handler supports one

controller  and  a  maximum of 8 units.  The EQ handler supports

unit numbers EQ0 through EQ7.  These unit numbers are used as  a

logical  connection  between  a user program and an address/pro-

tocol pair to be recognized by the Ethernet hardware.

5.1  RESTRICTIONS

Observe the following restrictions for the EQ handler:

1.  The handler runs only under TSX-Plus.

2.  Programs  that  call the Ethernet handler must follow a

certain order.  Write such programs with the  following

elements in the order indicated.

1.  Use  the .LOOKUP programmed request to open a chan-

nel to the device unit.

2.  Allocate the unit using SPFUN 200.

3.  Perform the Ethernet operation(s).

4.  Deallocate the unit using SPFUN 200.

5.  Use  the  .CLOSE  programmed  request  to close the

channel to the specified device unit.

EQ HANDLER                                              PAGE 5-2

RESTRICTIONS

NOTE

Special  functions  207 (get physical address) and 210

(get handler status block) do not required device  al-

location.

5.2  USE OF SPECIAL FUNCTIONS (.SPFUN)

The  EQ  Ethernet  handler  operates  the DEQNA Ethernet con-

troller using the .SPFUN programmed request with  the  following

function codes:

Function                        Code

--------                        ----

Allocate unit                   200

Deallocate unit                 200

Enable Promiscuous Mode         201

Disable Promiscuous Mode        201

Enable protocol type            202

Disable protocol type           202

Enable multicast address        203

Disable multicast address       203

Transmit Ethernet frame         204

Receive Ethernet frame          205

Enable frame bufferring         206

Disable frame bufferring        206

Get physical address            207

Get handler status block        210

Successful  completion  of  a .SPFUN request clears the carry

bit.  Completion with an error  sets  the  carry  bit,  and  the

status word in the buffer contains an error code.

The  EQ  handler  is  functionally  identical to the RT-11 NQ

Ethernet handler with the addition  of  receive/transmit  device

timeout support and special functions 206, 207 and 210.

EQ HANDLER                                              PAGE 5-3

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.2.1  Unit Specification (.SPFUN 200)

The  allocate  unit  special  function allocates a unit for a

job's exclusive use.

The deallocate unit special function deallocates a unit so it

can be used by another job.

5.2.1.1  Allocate Unit  -

The  following  is  the form of the special function allocate

unit (.SPFUN 200):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 200

buf     is the address of a 4-word buffer

containing the status word and space

for the station's physical address.

The buffer contents are returned by

the allocate unit special function.

The high byte of the status word contains a 0.

Allocate unit returns one of the following

octal status codes in the low byte of the

status word:

0  Success

2  Controller error while attempting to

initialize the network interface

3  No resources (unit in use)

11 Reserved

wcnt    is 0

blkn    is 1

crtn    is the entry point of a completion routine

EQ HANDLER                                              PAGE 5-4

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.2.1.2  Deallocate Unit  -

The  following is the form of the special function deallocate

unit (.SPFUN 200):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 200

buf     is the address of a 1-word buffer

containing the status word.

The high byte of the status word contains a 0.

Deallocate unit returns one of the following

octal status codes in the low byte of the

status word:

0  Success

1  Unknown unit. The specified unit was not

opened by the job issueing the request.

2  Controller error while attempting to

initialize the network interface

11 Unit still active

wcnt    is 0

blkn    is 0

crtn    is the entry point of a completion routine

EQ HANDLER                                              PAGE 5-5

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.2.2  Promiscuous Mode (.SPFUN 201)

The  enable  promiscuous  mode  special  function enables the

Ethernet hardware to receive all network traffic  regardless  of

address  or  protocol.   Any read request to the EQ handler will

return the first available packet received by the  DEQNA  inter-

face.

The disable promiscuous mode special function enables the ad-

dress/protocol filtering provided by the Ethernet  hardware  and

enables the specified protocol.

5.2.2.1  Enable Promiscuous Mode  -

The  following  is  the  form  of the special function enable

promiscuous mode (.SPFUN 201):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 201

buf     is the address of a 1-word buffer

containing the status word.

The high byte of the status word contains a 0.

Enable promiscuous mode returns one of the

following octal status codes in the low byte

of the status word:

0  Success

6  address/protocol active

wcnt    is 0

blkn    is 1

crtn    is the entry point of a completion routine

EQ HANDLER                                              PAGE 5-6

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.2.2.2  Disable Promiscuous Mode  -

The  following  is  the  form of the special function disable

promiscuous  mode  (which  performs  an  enable  protocol  type)

(.SPFUN 201):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 201

buf     is the address of a 2-word buffer that

contains the status word, followed by

the 1-word protocol type that is to be

enabled when the promiscuous mode is

disabled.

The high byte of the status word contains a 0.

The disable promiscuous mode returns one of the

following octal status codes in the low byte

of the status word:

0  Success

1  Unknown unit. The specified unit was not

opened by the job issueing the request.

2  Controller error while attempting to

initialize the network interface

3  Reserved

6  Reserved

10 Protocol type in use

wcnt    is 0

blkn    is 0

crtn    is the entry point of a completion routine

EQ HANDLER                                              PAGE 5-7

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.2.3  Protocol Type (.SPFUN 202)

The  enable  protocol  type  special function adds a protocol

type to those to be recognized by the Ethernet controller.  Only

one  protocol type can be specified for each unit.  At least one

protocol type must be enabled to receive Ethernet frames.

The  disable  protocol type special function removes the pro-

tocol type from the list of those  recognized  by  the  Ethernet

controller.

5.2.3.1  Enable Protocol Type  -

The following is the form of the special function enable pro-

tocol type (.SPFUN 202):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 202

buf     is the address of a 2-word buffer

that contains the status word followed

by the protocol type word.

The high byte of the status word contains a 0.

Enable protocol type returns one of the

following octal status codes in the low byte

of the status word:

0  Success

1  Unknown unit. The specified unit was

not opened by the job issuing the

request

2  Controller error while attempting to

initialize the network interface

3  No resources (unit's protocol table full)

6  Reserved

EQ HANDLER                                              PAGE 5-8

USE OF SPECIAL FUNCTIONS (.SPFUN)

10 Protocol type in use

The protocol type is specified by the user

wcnt    is 0

blkn    is 1

crtn    is the entry point of a completion routine

5.2.3.2  Disable Protocol Type  -

The  following  is  the  form of the special function disable

protocol type (.SPFUN 202):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 202

buf     is the address of a 2-word buffer

that contains the status word followed

by the protocol type word.

The high byte of the status word contains a 0.

Disable protocol type returns one of the

following octal status codes in the low byte

of the status word:

0  Success

1  Unknown unit. The specified unit was

not opened by the job issuing the

request

2  Controller error while attempting to

initialize the network interface

If the disabled protocol type is not the

same as the enabled protocol type for this

unit then 'success' is returned.

wcnt    is 0

EQ HANDLER                                              PAGE 5-9

USE OF SPECIAL FUNCTIONS (.SPFUN)

blkn    is 0

crtn    is the entry point of a completion routine

5.2.4  Multicast Address (.SPFUN 203)

The enable multicast address special function adds the multi-

cast address to those recognized  by  the  Ethernet  controller.

Ypu  need  not specify the unit's physical or broadcast address.

Only one multicast address is allowed per handler unit.

The  disable  multicast  address special function removes the

multicast address from those to be recognized  by  the  Ethernet

controller.

5.2.4.1  Enable Multicast Address  -

The following is the form of the special function enable mul-

ticast address (.SPFUN 203):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 203

buf     is the address of a 4-word buffer

that contains the status word, followed

by the 3-word multicast address. The

low order bit of the first address

word should be a 1.

The high byte of the status word contains a 0.

Enable multicast address returns one of the

following octal status codes in the low byte

of the status word:

0  Success

1  Unknown unit. The specified unit was not

opened by the job issueing the request.

2  Controller error while attempting to

EQ HANDLER                                             PAGE 5-10

USE OF SPECIAL FUNCTIONS (.SPFUN)

initialize the network interface

3  No resources (unit's address table is full,

or hardware address table full)

wcnt    is 0

blkn    is 1

crtn    is the entry point of a completion routine

5.2.4.2  Disable Multicast Address  -

The  following  is  the  form of the special function disable

multicast address (.SPFUN 203):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 203

buf     is the address of a 4-word buffer

that contains the status word, followed

by the 3-word multicast address. The

low order bit of the first address

word should be a 1.

The high byte of the status word contains a 0.

Disable multicast address returns one of the

following octal status codes in the low byte

of the status word:

0  Success

1  Unknown unit. The specified unit was not

opened by the job issueing the request.

2  Controller error while attempting to

initialize the network interface

If the disabled multicast address is not the

same as the enabled multicast address for this

unit then 'success' is returned.

EQ HANDLER                                             PAGE 5-11

USE OF SPECIAL FUNCTIONS (.SPFUN)

wcnt    is 0

blkn    is 0

crtn    is the entry point of a completion routine

5.2.5  Transmit Ethernet Frame (.SPFUN 204)

The  special  function  transmit Ethernet frame transmits the

Ethernet frame pointed to in the buf argument.   If  the  source

field  of  the  frame  is not zero, it is kept and used.  If the

source field of the frame is zero, the controllers physical  ad-

dress is inserted in the source field before transmission.

The  following  is  the form of the special function transmit

Ethernet frame (.SPFUN 204):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 204

buf     is the address of a variable size buffer

containing the status word, followed by

the Ethernet packet to be transmitted.

The first word following the status word

is reserved. Next is the 3-word destination

address field followed by the 3-word

source address field. The next word is the

packet protocol type. Following this is the

variable length packet containing from 23 to

750 words.

The transmit Ethernet frame returns one of the

following octal status codes in the low byte

of the status word:

0  Success

1  Unknown unit. The specified unit was not

opened by the job issueing the request.

2  Controller error while attempting to

EQ HANDLER                                             PAGE 5-12

USE OF SPECIAL FUNCTIONS (.SPFUN)

initialize the network interface

13 Transmit failed

When status code 13 is returned in the low byte

of the status word, transmit Ethernet frame

returns one of the following octal status codes

in the high byte of the status word:

1  Invalid frame length

2  Excessive collisions

3  Carrier check failed

wcnt    is determined by the variable size of the user

buffer (including the status word and reserved

word). The packet size (including the status

word and reserved word) can vary between 32

and 759 words.

blkn    is 0

crtn    is the entry point of a completion routine

5.2.6  Receive Ethernet Frame (.SPFUN 205)

The  receive Ethernet frame special function returns the next

Ethernet packet with the desired unit address and protocol  type

to  the  buffer.   The  function does not return Ethernet frames

that are received with errors.

The  following  is  the  form of the special function receive

Ethernet frame (.SPFUN 205):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 205

buf     is the address of a variable size buffer

containing the status word, a word

containing the frame size, and followed by

EQ HANDLER                                             PAGE 5-13

USE OF SPECIAL FUNCTIONS (.SPFUN)

upto 757 words to receive the Ethernet

frame. The buffer contents are returned

by the receive Ethernet frame function.

The high byte of the status word contains a 0.

The receive Ethernet frame special function

returns one of the following octal status

codes in the low byte of the status word:

0  Success

1  Unknown unit. The specified unit was not

opened by the job issueing the request.

2  Controller error while attempting to

initialize the network interface

wcnt    is the size of the user buffer including

the status and frame size words

blkn    is 0

crtn    is the entry point of a completion routine

5.2.7  Frame Queueing (.SPFUN 206)

The  Enable  frame  queueing  special  function allows the EQ

handler to internally queue  received  packets  that  match  any

enabled  address/protocol  even though a receive request has not

been issued for that address/protocol or a previous  packet  has

not been processed.  The EQ handler normally allocates space for

internally queueing upto 20 packets (this may be changed by mod-

ifying 'rbcnt' in EQ.MAC and rebuilding the EQ handler).

The  disable frame queueing special function disables the in-

ternal queueing of packets.  This function may only be issued if

all address/protocol pairs have been disabled.

EQ HANDLER                                             PAGE 5-14

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.2.7.1  Enable Frame Queueing  -

The  following  is  the  form  of the special function enable

frame queueing (.SPFUN 206):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 206

buf     is the address of a 1-word buffer

for the returned status word which

is always 0

wcnt    is 0

blkn    is 1

crtn    is the entry point of a completion routine

5.2.7.2  Disable Frame Queueing  -

The  following  is  the  form of the special function disable

frame queueing (.SPFUN 206):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 206

buf     is the address of a 1-word buffer

containing the status word.

The high byte of the status word contains a 0.

Disable frame queueing returns one of the

following octal status codes in the low byte

of the status word:

EQ HANDLER                                             PAGE 5-15

USE OF SPECIAL FUNCTIONS (.SPFUN)

0  Success

6  address/protocol active

wcnt    is 0

blkn    is 0

crtn    is the entry point of a completion routine

5.2.8  Get Ethernet Address (.SPFUN 207)

The  get Ethernet address special function returns the physi-

cal address of the Ethernet controller card.

The  following  is  the  form  of  the  special  function get

Ethernet address (.SPFUN 207):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 207

buf     is the address of a 4-word buffer

in which the status word, which is

always 0, and the six byte physical

Ethernet address are returned.

wcnt    is 0

blkn    is 0

crtn    is the entry point of a completion routine

EQ HANDLER                                             PAGE 5-16

USE OF SPECIAL FUNCTIONS (.SPFUN)

5.2.9  Get Ethernet Status Block (.SPFUN 210)

The get Ethernet status block special function returns the EQ

handler's internal parameter block.  The  information  contained

in  the  parameter block includes the number of packets received

and transmitted for each EQ unit as well as timeout information.

The  following  is  the  form  of  the  special  function get

Ethernet status block (.SPFUN 210):

Macro Call:

.SPFUN  area,chan,func,buf,wcnt,blkn,[crtn]

where:

area    is address of 6-word EMT argument block

chan    is a channel number in the range

0 to 376 (octal)

func    is 210

buf     is the address of a buffer into which

wcnt words are written from the

EQ handlers status block.

wcnt    is the number of words to return

to the buffer.  The status word

must be included in the word count.

blkn    is 0

crtn    is the entry point of a completion routine

The complete list of parameters in the status block may be found

in the EQ Ethernet handler files EQ.MAC and EI.MAC.   The  first

word  is  always  a status word of value 0, the following 4-byte

(long integer) parameters are:

eiipkt          total received packets

eiopkt          total transmitted packets

eirtmo          total receive timeouts

eixtmo          total transmit timeouts

eirun[8]        received packets for each unit

eixun[8]        transmitted packets for each unit

These  are  the  only  parameters  used by any program in the

'TCP/IP Package for TSX-Plus'.

CHAPTER 6

BUILDING THE TCP/IP PACKAGE

6.1  C COMPILER

The DECUS C Compiler and Assembler and the RT-11 Macro Assem-

bler and Linker are required to build the the TCP/IP Package for

TSX-Plus  from  the  source  files included in the distribution.

The DECUS C Compiler is from  the  DECUS  11-SP-18  distribution

with  bug fixes made to CC103, CC300, and CC202A of the compiler

and also to FCLOSE and FWILD of the library.  A patched  version

of  the  compiler  along  with  the  assembler, library, and the

necesary header files are included with the distribution to  al-

low  the package to be recompiled.  The CC.SAV file was modified

by the TSX-Plus SETSIZ program to allow it to  use  an  extended

program space of 64Kb.

The  build  procedures  assume  that  the compiler and header

files are on the logical device 'C:'.

6.2  TCP/IP PACKAGE

The  TCP/IP Package is supplied with the complete source code

as an assemblage of C files (*.c), macro files  (*.mac),  header

files  (*.h),  and  an assortment of command files (*.com).  The

command file BLDALL.COM  will  compile  and  link  the  complete

TCP/IP  Package.   Inspect BLDALL.COM to see how each individual

component is built.  An unloaded 11/73 processor will take  from

30 to 40 minutes to compile and link the complete package.

The  file  CPYEXE.COM  may  be  used  to  copy  all the files

necesary to configure a complete implementation  of  the  TCP/IP

Package.   The  command  files  must be modified to reflect your

system's disk complement and the location of the password files.

The  password  file  must be updated to specify users/passwords,

access rights, and MAIL configuration.

APPENDIX A

UNDOCUMENTED OPTIONS

A.1  DISABLE PASSWORD

The  -b  option  will disable the user/password query in pro-

grams TELNET, RTELNT, FTP, RFTP, CNCT, RCNCT, and  MAIL.   Local

users normally are not required to use passwords when activating

TELNET, FTP, or CNCT.  The MAIL program and the  network  server

programs  RTELNT,  RFTP, and RCNCT should not specify the -b op-

tion for security reasons.

A.2  SUBDIRECTORY UNIT SELECTION

The  default  LD units used by the client/server/utility pro-

grams  for  subdirectory  access  are  LD2:   and  LD3:.   These

defaults may be changed with the undocumented -s option:

-s arg1 arg2

where arg1 is the first LD unit number to use and arg2 is the

maximum number of nested subdirectories.  The value of arg1+arg2

must  be  less than or equal to 8, the total number of LD units.

The default is arg1=2 and arg2=2.

APPENDIX B

DISTRIBUTION DIRECTORIES

The  "TCP/IP  Package  for TSX-Plus" is distributed on 5 RX02

type floppy disks.  Disks 1 and 2  contain  a  complete  set  of

files  to  configure  a working system.  These disks contain the

executable, command, and assorted help, message, text, and  con-

figuration  files.   Disks  3  and 4 contain the source code and

command files to rebuild the package.  Disk 5 contains the DECUS

C  compiler, AS assembler, and library.  This disk also contains

the source for the patches applied to the compiler and  library.

Disks  2  and  4 also contain identical copies of the documenta-

tion.

B.1  DISK 1 DIRECTORY

20-Aug-92

CPYEXE.COM     3  20-Aug-92      EQ    .TSX     9  20-Aug-92

DETACH.TSX     1  20-Aug-92      TCPIP .SAV   108  20-Aug-92

TCPIP .COM     1  20-Aug-92      TCPIP .CFG    10  20-Aug-92

TELNET.SAV    82  20-Aug-92      TELNET.COM     1  20-Aug-92

RTELNT.SAV    87  20-Aug-92      RTELNT.COM     1  20-Aug-92

RTELNT.SRV     1  20-Aug-92      RTELNT.MSG     1  20-Aug-92

RTELNT.HLP     1  20-Aug-92      CNCT  .SAV    71  20-Aug-92

CNCT  .COM     1  20-Aug-92      RCNCT .SAV    77  20-Aug-92

RCNCT .COM     1  20-Aug-92      RCNCT .SRV     1  20-Aug-92

RCNCT .MSG     1  20-Aug-92      RCNCT .HLP     1  20-Aug-92

FTP   .SAV   105  20-Aug-92      FTP   .COM     1  20-Aug-92

FTPCMD.TXT     5  20-Aug-92      RFTP  .SAV   100  20-Aug-92

RFTP  .COM     1  20-Aug-92      RFTP  .SRV     1  20-Aug-92

RFTP  .MSG     1  20-Aug-92      RFTP  .HLP     1  20-Aug-92

MAIL  .SAV   106  20-Aug-92      MAIL  .COM     1  20-Aug-92

MAICMD.TXT     3  20-Aug-92      RSMTP .SAV    88  20-Aug-92

RSMTP .COM     1  20-Aug-92      RSMTP .SRV     1  20-Aug-92

RSMTP .HLP     1  20-Aug-92      PASWRD.SAV    49  20-Aug-92

PASWRD.FIL     1  20-Aug-92      USRBLK.H      15  20-Aug-92

PASWRD.COM     1  20-Aug-92

39 Files, 941 Blocks

33 Free blocks

DISTRIBUTION DIRECTORIES                                PAGE B-2

DISK 2 DIRECTORY

B.2  DISK 2 DIRECTORY

20-Aug-92

20-Aug-92

SKTMON.SAV    96  20-Aug-92      SKTMON.COM     1  20-Aug-92

UTIL  .SAV    53  20-Aug-92      LLS   .COM     1  20-Aug-92

LDEL  .COM     1  20-Aug-92      LPRO  .COM     1  20-Aug-92

LUNP  .COM     1  20-Aug-92      LDIR  .COM     1  20-Aug-92

LCD   .COM     1  20-Aug-92      LMKDIR.COM     1  20-Aug-92

LRMDIR.COM     1  20-Aug-92      LRENAM.COM     1  20-Aug-92

LDISKS.SAV    53  20-Aug-92      LDISKS.COM     1  20-Aug-92

RFTHLP.COM     1  20-Aug-92      ABRTCP.SAV    11  20-Aug-92

KILLM .SAV     2  20-Aug-92      REMOVE.COM     2  20-Aug-92

MANUAL.DOC   217  20-Aug-92

19 Files, 446 Blocks

528 Free blocks

B.3  DISK 3 DIRECTORY

20-Aug-92

CLEAN .COM     1  20-Aug-92      LDEL  .COM     1  20-Aug-92

LPRO  .COM     1  20-Aug-92      LUNP  .COM     1  20-Aug-92

LDIR  .COM     1  20-Aug-92      LCD   .COM     1  20-Aug-92

LMKDIR.COM     1  20-Aug-92      LRMDIR.COM     1  20-Aug-92

LRENAM.COM     1  20-Aug-92      LDISKS.COM     1  20-Aug-92

TCPIP .COM     1  20-Aug-92      REMOVE.COM     2  20-Aug-92

RFTHLP.COM     1  20-Aug-92      LLS   .COM     1  20-Aug-92

MAILFX.COM     1  20-Aug-92      COPYOB.COM     1  20-Aug-92

COPYSV.COM     1  20-Aug-92      COPY  .COM     1  20-Aug-92

BLDMAC.COM     1  20-Aug-92      CMPRCN.COM     1  20-Aug-92

CMPLDI.COM     1  20-Aug-92      CMPFTP.COM     1  20-Aug-92

CMPMAI.COM     1  20-Aug-92      CMPUTL.COM     1  20-Aug-92

CMPRFT.COM     1  20-Aug-92      CMPTCP.COM     1  20-Aug-92

CMPMON.COM     1  20-Aug-92      CMPRSM.COM     1  20-Aug-92

BLDSUP.COM     1  20-Aug-92      CMPTEL.COM     1  20-Aug-92

CMPRTL.COM     1  20-Aug-92      CMPPAS.COM     1  20-Aug-92

CMPCNC.COM     1  20-Aug-92      SKTMON.COM     1  20-Aug-92

TELNET.COM     1  20-Aug-92      MAIL  .COM     1  20-Aug-92

FTPL  .COM     1  20-Aug-92      RTELNT.COM     1  20-Aug-92

FTP   .COM     1  20-Aug-92      CNCT  .COM     1  20-Aug-92

TELNTL.COM     1  20-Aug-92      SKTMNL.COM     1  20-Aug-92

RCNCT .COM     1  20-Aug-92      RFTP  .COM     1  20-Aug-92

RSMTP .COM     1  20-Aug-92      BLDTST.COM     1  20-Aug-92

PASWRD.COM     1  20-Aug-92      BLDALL.COM     2  20-Aug-92

BLDEQ .COM     1  20-Aug-92      CPYEXE.COM     3  20-Aug-92

FTP   .BLD     1  20-Aug-92      TELNET.BLD     1  20-Aug-92

PASWRD.BLD     1  20-Aug-92      RFTP  .BLD     1  20-Aug-92

SKTMON.BLD     1  20-Aug-92      UTIL  .BLD     1  20-Aug-92

ABRTCP.BLD     1  20-Aug-92      TCPIP .BLD     1  20-Aug-92

LDISKS.BLD     1  20-Aug-92      RSMTP .BLD     1  20-Aug-92

MAIL  .BLD     1  20-Aug-92      CNCT  .BLD     1  20-Aug-92

DISTRIBUTION DIRECTORIES                                PAGE B-3

DISK 3 DIRECTORY

RCNCT .BLD     1  20-Aug-92      RTELNT.BLD     1  20-Aug-92

TCPIP .LNK     1  20-Aug-92      TELNET.LNK     1  20-Aug-92

RTELNT.LNK     1  20-Aug-92      FTP   .LNK     1  20-Aug-92

RFTP  .LNK     1  20-Aug-92      CNCT  .LNK     1  20-Aug-92

RCNCT .LNK     1  20-Aug-92      MAIL  .LNK     1  20-Aug-92

RSMTP .LNK     1  20-Aug-92      UTIL  .LNK     1  20-Aug-92

LDISKS.LNK     1  20-Aug-92      PASWRD.LNK     1  20-Aug-92

SKTMON.LNK     1  20-Aug-92      ENDRVR.MAC    19  20-Aug-92

TERMIO.MAC     6  20-Aug-92      PRGREQ.MAC     3  20-Aug-92

INISRV.MAC     1  20-Aug-92      INICLI.MAC     1  20-Aug-92

SKTSKT.MAC    10  20-Aug-92      LCLSKT.MAC     8  20-Aug-92

GBLSKT.MAC     9  20-Aug-92      SUSPND.MAC     3  20-Aug-92

HDRCHK.MAC     3  20-Aug-92      KILLM .MAC     2  20-Aug-92

DETACH.MAC     3  20-Aug-92      NPRMPT.MAC     1  20-Aug-92

BYTPRC.MAC     4  20-Aug-92      CTICKS.MAC     2  20-Aug-92

TLPRMT.MAC     1  20-Aug-92      EI    .MAC    39  20-Aug-92

SVDRVR.MAC    14  20-Aug-92      CLDRVR.MAC    13  20-Aug-92

JOBMON.MAC     5  20-Aug-92      RCDLCK.MAC    11  20-Aug-92

LOGDSK.MAC     9  20-Aug-92      SLUNIO.MAC    16  20-Aug-92

TSXCND.MAC     4  20-Aug-92      EQ    .MAC    37  20-Aug-92

VT1XX .H       5  20-Aug-92      INISRV.H       1  20-Aug-92

PRGREQ.H       1  20-Aug-92      INICLI.H       1  20-Aug-92

KBDUTL.H       2  20-Aug-92      SERVER.H       1  20-Aug-92

BTPDAT.H       3  20-Aug-92      TCPDAT.H       6  20-Aug-92

DOMDAT.H       5  20-Aug-92      HDRCHK.H       1  20-Aug-92

BYTPRC.H       1  20-Aug-92      ENDRVR.H       1  20-Aug-92

HSTDEF.H       6  20-Aug-92      RTFILE.H       2  20-Aug-92

FMTDAT.H       2  20-Aug-92      EVTDEF.H       5  20-Aug-92

SUSPND.H       1  20-Aug-92      LOGDSK.H       1  20-Aug-92

KBKEYS.H       3  20-Aug-92      PRODAT.H       1  20-Aug-92

DETACH.H       1  20-Aug-92      LDISKS.H       2  20-Aug-92

CTICKS.H       1  20-Aug-92      MAPSKT.H       1  20-Aug-92

DFAULT.H       4  20-Aug-92      CLDRVR.H       1  20-Aug-92

SVDRVR.H       1  20-Aug-92      TSXUTL.H       8  20-Aug-92

SCMND .H       1  20-Aug-92      RCMND .H       2  20-Aug-92

TERMIO.H       2  20-Aug-92      RCDLCK.H       1  20-Aug-92

DATIME.H       1  20-Aug-92      FTPPI .H       9  20-Aug-92

ACCESS.H       2  20-Aug-92      SRVUTL.H       4  20-Aug-92

CLIUTL.H       7  20-Aug-92      JOBMON.H       1  20-Aug-92

USRBLK.H      15  20-Aug-92      MAILBX.H       4  20-Aug-92

MAILPI.H       6  20-Aug-92      PRODEF.H      24  20-Aug-92

VRTEL .H       1  20-Aug-92      VMAIL .H       1  20-Aug-92

VRFTP .H       1  20-Aug-92      VRSMTP.H       1  20-Aug-92

VTEL  .H       1  20-Aug-92      VCNCT .H       1  20-Aug-92

VFTP  .H       1  20-Aug-92      VRCNCT.H       1  20-Aug-92

VTCPIP.H       1  20-Aug-92      KBDUTL.C       7  20-Aug-92

SCRNAC.C      14  20-Aug-92      SCRNJO.C      13  20-Aug-92

FMTDAT.C       6  20-Aug-92      SCRNDI.C      17  20-Aug-92

SCRNPX.C      24  20-Aug-92      VT1XX .C      14  20-Aug-92

ABRTCP.C       1  20-Aug-92      BOOTP .C      15  20-Aug-92

SKTMON.C      21  20-Aug-92      CONFIG.C      49  20-Aug-92

SCRNZ .C       5  20-Aug-92      PROTIN.C      19  20-Aug-92

DISTRIBUTION DIRECTORIES                                PAGE B-4

DISK 3 DIRECTORY

IP    .C       8  20-Aug-92      UDP   .C       8  20-Aug-92

DMPFIL.C       1  20-Aug-92      ARP   .C      20  20-Aug-92

SCMND .C       2  20-Aug-92      RCMND .C       2  20-Aug-92

DATIME.C       2  20-Aug-92      MAILBX.C       5  20-Aug-92

NET   .C       6  20-Aug-92      TCP   .C      41  20-Aug-92

ACCESS.C      16  20-Aug-92      RTFILE.C      10  20-Aug-92

SRVUTL.C      65  20-Aug-92      MAILB .C      31  20-Aug-92

RTLNTB.C      25  20-Aug-92      CLIUTL.C      52  20-Aug-92

182 Files, 960 Blocks

10 Free blocks

B.4  DISK 4 DIRECTORY

20-Aug-92

RCNCT .C      39  20-Aug-92      CNCT  .C      34  20-Aug-92

RTLNTA.C      49  20-Aug-92      SERVER.C      23  20-Aug-92

TELNET.C      78  20-Aug-92      FTP   .C     123  20-Aug-92

DOMAIN.C      18  20-Aug-92      PASWRD.C      42  20-Aug-92

RSMTP .C      45  20-Aug-92      MAILA .C      72  20-Aug-92

TSXUTL.C      47  20-Aug-92      RFTP  .C      78  20-Aug-92

LDISKS.C      13  20-Aug-92      UTIL  .C      15  20-Aug-92

MAICMD.TXT     3  20-Aug-92      FTPCMD.TXT     5  20-Aug-92

RFTP  .MSG     1  20-Aug-92      RTELNT.MSG     1  20-Aug-92

RCNCT .MSG     1  20-Aug-92      RSMTP .HLP     1  20-Aug-92

RFTP  .HLP     1  20-Aug-92      RTELNT.HLP     1  20-Aug-92

RCNCT .HLP     1  20-Aug-92      RFTP  .SRV     1  20-Aug-92

RSMTP .SRV     1  20-Aug-92      RCNCT .SRV     1  20-Aug-92

RTELNT.SRV     1  20-Aug-92      DETACH.TSX     1  20-Aug-92

TCPIP .CFG    10  20-Aug-92      PASWRD.FIL     1  20-Aug-92

EQ    .TSX     9  20-Aug-92      MANUAL.DOC   217  20-Aug-92

DISK1 .DIR     3  20-Aug-92      DISK2 .DIR     2  20-Aug-92

DISK3 .DIR    12  20-Aug-92      DISK4 .DIR     3  20-Aug-92

DISK5 .DIR     2  20-Aug-92

37 Files, 955 Blocks

19 Free blocks

B.5  DISK 5 DIRECTORY

20-Aug-92

CC    .SAV   125P 04-Mar-88      AS    .SAV    36P 01-Jul-85

GREP  .SAV    28P 16-Dec-91      SUPORT.OBJ     1P 29-Nov-91

CLIB  .OBJ   107P 29-Nov-91      MALLOC.H       1P 01-Jul-90

TIME  .H       3P 12-Jul-84      STDIO .H      15P 08-Sep-90

STRING.H       1P 29-Nov-91      CC103 .MAC   103P 20-Feb-88

CC300 .MAC    25P 20-Feb-88      CC103 .TXT     8P 10-Feb-88

DOPRNT.TXT     3P 10-Feb-88      DOSCAN.TXT     5P 10-Feb-88

FCLOSE.TXT     2P 10-Feb-88      IOABUF.TXT     3P 10-Feb-88

DOPRNT.MAC    14P 20-Feb-88      DOSCAN.MAC    34P 20-Feb-88

FCLOSE.MAC    11P 20-Feb-88      IOABUF.MAC     4P 20-Feb-88

TMAKCC.COM     6P 28-Feb-88      TCOMLB.COM     4P 28-Feb-88

DISTRIBUTION DIRECTORIES                                PAGE B-5

DISK 5 DIRECTORY

TCCBLD.COM     2P 28-Feb-88      RT11  .EIS     5P 12-Jul-84

CC202A.MAC    15P 04-Mar-88      FWILD .MAC    94P 29-Nov-91

TMAKLB.COM     5P 29-Nov-91      DISK5 .DIR     2  20-Aug-92

28 Files, 662 Blocks

312 Free blocks