Programs for testing the speed of CPUs                   Page 1

B. Z. Lederman

NOTE

This  is  a  reduced  version of my original article:

the original is rather long, and I  thought  that  it

might  be unwieldy for people to download it over 300

or 1200 baud lines, especially considering the cost.

With  the  introduction of the PDP-11/84, it was suggested

that applications could be moved from 11/70s, but  some  method

of  verifying the speed of the 11/84 was desired and there were

no  published  instruction  times.   I  therefore  wrote   some

programs  which  will  measure  the  speed  at which a CPU will

execute instructions with reasonable accuracy.

The method I use is quite simple:  a single instruction is

executed  many  times  (generally  a   minimum   of   1,000,000

executions),  and the time this takes is measured.  Because the

largest single program segment on  a  PDP-11  is  about  32,000

words,  it  is  necessary  to  use  a loop:  a long sequence of

instructions is repeated as many times as  needed  to  get  the

total number of executions wanted.  There is also the effect of

cache:  a short loop will fit  entirely  into  cache  and  will

execute  at the fastest possible speed;  a longer loop will not

all fit into cache, and the effect of the speed of main  memory

can  be  seen.   Finally,  it is necessary to test the speed of

many different instructions (one at a time) to get an  adequate

profile of a given CPU's performance.

The   program   S1D.MAC  implements  this  type  of  test.

Although I  have  tried  to  make  it  as  straight-forward  as

possible,  it  should  be  remembered  that this is basically a

diagnostic program,  and  that  it  does  some  things  that  I

normally wouldn't allow in a production program:  specifically,

it treats instruction space as  data,  is  self-modifying,  and

there  are two tables which must be kept in alignment manually.

Sometimes this sort of  thing  is  necessary  in  a  diagnostic

program.   While  modifications  would  be  fairly  easy  for a

programmer with some experience, I  don't  suggest  making  any

changes  unless  you  are  fairly  certain you know what to do.

(The program is not privileged, however, so the worst that  can

happen is the program will abort:  it won't crash your system.)

There  are  two  global  equates  at  the beginning of the

program:  $NREP is set to the number of instructions wanted  in

the  loop,  and  $NPAS  is  the  number  of  times  the loop is

executed.  Any combination which will multiply to a large  even

number  will  do, such as 1,000 and 1,000;  3125 and 320;  2500

and 400, or any other suitable combination.  Having the numbers

multiply  out  to  1,000,000  or  10,000,000  isn't  absolutely

Programs for testing the speed of CPUs                   Page 2

B. Z. Lederman

necessary, but it makes it easier to read out  the  times:   at

1,000,000  repetitions  the  time  measured  in  seconds is the

instruction time in micro-seconds.  Because  the  loop  has  to

have  3  instructions  to  form the loop repeat logic, there is

another global called $NREAL which is $NREP minus 3.   As  long

as the loop is reasonably long (several hundred instructions or

more) the effect  of  the  loop  repeat  instructions  will  be

negligible.   I considered making them task build time options,

but it would have made the program far too complicated.   Don't

be  surprised  if  the program takes more time to assemble when

$NREP is large, as it takes more time to fill the ISTRT area.

There  are  two tables which must match:  the first starts

at MNTAB, and is a list of mnemonics which print out  when  you

run  the  program to tell you what instruction is being tested.

This must match the set of instructions starting  at  TABLE  if

you  want  to  know  what  is  going on.  The instructions were

placed in their present order after running  the  program  many

times  so  the data comes out in what I feel is the most useful

form, though the instructions could be in any  order.   All  of

the  instructions  are position independent, as they have to be

moved by the program into the execution  area:   testing  other

addressing  modes would be difficult as the addresses would all

have to be adjusted, and I don't believe that  the  information

obtained  would  significantly  improve the comparisons between

machines.  In this program, all instructions are one word long:

another  program  called  S2D.MAC  is identical except that two

word  instructions  are  tested  (actually  instructions   with

addressing  modes  that cause the instruction to be followed by

one word of address information or literal data), and similarly

for  S3D.MAC  which tests three word instructions.  Using three

separate programs makes things very much easier  (for  me,  the

programmer).   There  are also a few special cases:  the BRanch

instructions are set to always branch to the next  instruction,

which  allows  them to be tested even though they are relocated

within the program.  They have been selected so that  one  will

always  branch  and  the  other  will  never  branch:   on many

machines there will be a  difference  in  execution  speed  for

these  two  conditions.   The other special case is the Jump to

SubRoutine instruction:  for this to be tested there must be  a

RETurn  from  Subroutine  instruction  (which  is  in  a  fixed

location), and  the  time  of  execution  really  is  for  both

instructions  together.  As most subroutines end with a return,

this should be a valid test as long as one remembers  that  the

times  obtained  are  for  two instructions.  The data which is

used in the  instructions  is  either  absolute  references  or

loaded  into  registers:   the data for the multiply and divide

instructions may  not  be  the  best  possible  test  of  these

instructions,   but   it   does   work.    Auto-increment   and

auto-decrement addressing modes are difficult to  test  as  the

loop   is   at  least  1,000,000  instructions  long,  and  any

Programs for testing the speed of CPUs                   Page 3

B. Z. Lederman

continuous test would  push  the  register  out  of  addressing

range:   the  only  solution  I  could see was to have the same

instruction do a decrement and increment together, so only  two

addressing  locations  are  used;   this  may  be  seen  in the

MOV -(SP), (SP)+ test.

It  should  be  noted that these programs include floating

point processor instructions:   though  it  would  be  easy  to

include  an  assembly  condition  to  include  these  only when

wanted, I haven't been able to get around to  doing  that  yet.

Persons  wishing  to  test machines which don't have FPP should

conditionalize or comment out the FPP instructions:  this would

also  apply  to anyone who does not have EIS instructions (MUL,

DIV,  etc.).   These  programs  do  not   test   FIS   or   CIS

instructions,  because  none  of  the  machines to which I have

access have these instructions.

In  the  main  body of the program there is space reserved

beginning at ISTRT:  this is initially filled with  No  OPerate

instructions,  and the program fills this space with one of the

instructions from the TABLE in order  to  test  it's  execution

time.   The  system  directive  GTIM$  (Get  Time) is used just

before the start of the loop and just after the loop to see how

long  it  took:   as  system  time  is  delivered in 0.1 second

intervals, it is necessary to make the test loop long enough to

use  more  time  than  this value.  The fastest 11/70 and 11/84

instructions  are  about  0.3  micro-seconds  long,  so  if   a

1,000,000  instruction  loop  is  used,  the measured times are

about  0.3  seconds,  only  a  little  above  the   measurement

granularity.    Though   this  is  enough  for  an  approximate

comparison, running a 10,000,000 instruction  test  brings  the

shortest  times  up  to  about  3.0 seconds, and gives one more

significant digit of data.   Running  tests  longer  than  this

should  not  be  necessary.   For  slower machines (such as the

PRO-350 and 11/23), 10,000,000 instruction loops are nice but a

little  slow,  and  1,000,000  instruction  loops  are probably

adequate if you don't have much test time available.

The  use of the system directive to measure time brings up

several important factors in running these tests, of which  the

first  is  the  amount  of  time it takes to execute the GETIM$

directive.  I measured this on our 11/70 by  having  a  program

issue  several GETIM$ directives sequentially and measuring the

time each directive took to execute with SPM-11:  this  package

from  DEC hooks into the executive and measures time in 0.00001

second intervals (100,000 per second).  The GETIM$  took  about

0.0005 seconds (500 micro-seconds) to execute, which is so much

less than the minimum resolution of 0.1 second that it  can  be

ignored  if  the  loop  is long enough to register at least 0.1

seconds.  A related point is that  when  the  test  program  is

running,  it  should be the only program running, and at a high

Programs for testing the speed of CPUs                   Page 4

B. Z. Lederman

enough priority that it  will  "grab"  the  CPU  more  or  less

exclusively  (the  CPU  will  still  have to service the system

clock if you want the test times).  I have  run  the  tests  on

otherwise  idle  systems, and at a priority of 151 or 152 which

is high enough to be above just about everything except MCR:  I

want  the  test  to  be  below  MCR in case I want to abort it.

While this might make the instructions test a little  slow,  it

is  still  a  valid  comparison between different machines, and

when I  have  compared  the  test  results  obtained  with  the

published  instruction times in the processor handbooks, I have

obtained quite good correlations, indicating that  the  program

is  testing  accurately.   Under these conditions, it should be

possible to run the test either on RSX-11M-Plus, or P/OS (as  I

did),  or  RSX-11M,  or  possibly  on  RSX-11S and obtain valid

results.  The basic program would probably also run on IAS, but

it might be necessary to change the calls to the system library

that format the output messages and the QIOs if  they  are  not

the  same  on IAS.  I've never tried the RSX run-time system on

RSTS/E.

I  have  only  been  able  to test 3 machines:  the 11/70,

11/84P and PRO-350.  One of the reasons I am  publishing  these

programs  and  the  results,  and  placed these programs on the

DECUS RSX SIG tape, is the hope that  other  users  with  other

machines  will run the programs and publish the results, so the

user community will have additional data with which comparisons

between  models  of  PDP-11s  may  be  made when choosing a new

system or contemplating an upgrade path.  Also, by testing more

machines,  it  will  be  possible to compare the times with the

published  speeds  in  the  processor  handbooks,  and   obtain

additional  verification  that the speed tests are accurate (or

not, though I have reasonable evidence now that the  tests  are

correct).   I would be very interested in seeing the results of

tests on other CPUs, or suggestions on other instructions which

might be added to the test set.

I have also started on a version which will run on the VAX

family of machines but so far the results are rather  variable,

due to my not having "stand-alone" time on the machine, and the

effects of paging, caching, crossing page boundaries, etc.

The  following  are  some of the results of timing tests I

have done on several PDP-11 family CPUs, and have been arranged

to  show  the effects of addressing modes.  The category "Basic

Instructions" includes the  TeST,  ROtate  Right,  SWAp  Bytes,

MOVe,  MOVe  Byte, CoMPare, ADD, and BIt Test instructions, all

of which can normally be expected to execute at the same  speed

when  operating  on  registers,  and  did  in fact do so in the

tests.  SWAp Bytes is also used again in one of the  addressing

mode  tests:   in  these  tests  "addr"  is  a symbolic address

reference:  when there are two references to "addr" in the same

PDP-11 CPU instruction timing tests                      Page 5

B. Z. Lederman

instruction they are to two different locations in memory.  The

Branch EQual and Branch Not Equal instructions are used to show

the difference in speed when the branch is taken and when it is

not taken (on some machines these two times differ,  on  others

they  don't):   all other branch instructions should execute at

the same speed under the same circumstances.   Another  special

case is the Jump to SubRoutine instruction:  this also includes

the time for the ReTurn from Subroutine instruction, and so  is

the   time   needed  to  execute  two  instructions.   This  is

reasonable,  as  most  subroutines  conclude   with   the   RTS

instruction.

The  reference  to  "Short" and "Long" is to the length of

the instruction sequence:  a short sequence should fit entirely

into  cache  and give the fastest possible CPU times.  The long

sequence does not fit into cache, and shows the effect of  main

memory speed.  All times are converted to micro-seconds.

11/70   11/70  11/84  11/84    PRO-

Instruction       Short   Long   Short   Long     350

Basic Instructions     0.31   0.91    0.3    0.7     2.38

TST     (R0)           0.76   1.36    0.7    1.3     4.20

CMP     (R0), (R3)     1.06   1.67    1.6    2.1     6.11

MOV    -(SP), (SP)+    1.70   2.55    2.1    2.7     6.50

MOVB   -(SP), (SP)+    2.00   2.86    2.1    2.7     6.51

MUL       R1, R0       3.19   3.81    5.7    6.5    26.64

DIV       R1, R0       1.98   2.59    9.3   10.2     8.96

BEQ     addr           0.31   0.90                   2.38

BNE     addr           0.61   1.21                   2.38

CCC, NOP               0.61   1.21    0.8    1.25    3.23

MOV       #1, R0       0.77   1.95    0.6    1.5     4.07

ASHC      #1, R0       1.38   2.56    2.1    3.1    11.92

MOV   @#addr, R1       1.67   2.42    1.1    2.0     5.49

TST   @#addr           1.67   2.42    1.1    2.0     5.49

SWAB  @#addr           2.03   3.66    1.9    2.9     6.78

MOV     addr, R1       1.43   2.27    1.4    2.3     6.07

TST    2(R0)           1.51   2.26    1.3    2.2     6.11

TST     addr           1.48   2.27    1.3    2.3     6.10

MOV    2(R0), (R3)     1.87   3.52    2.3    3.4     8.12

JSR       PC, (R2)     3.06   3.68                   9.03

JSR       PC, @#addr   3.80   4.63                  10.25

PDP-11 CPU instruction timing tests                      Page 6

B. Z. Lederman

MOV       #1, @#addr   2.04   4.08    1.9    3.2     7.34

MOV   @#addr, @#addr   2.76   4.54    2.4    3.7     8.96

MOV    4(R0), 2(R3)    2.62   4.44    3.0    4.3     9.68

ADD       #1, @#addr   3.02   4.34    2.1    3.5     8.39

ADD   @#addr, @#addr   2.81   4.84    2.7    4.0    10.10

ADD    4(R0), 2(R3)    2.68   4.75    3.2    4.5    10.73

SETF                   1.22   1.83    1.6    2.0     6.14

MULF     AC2, AC0      2.57   2.68   15.4   16.3    61.35

ADDF     AC0, AC1      2.72   2.81   14.3   15.3    72.78

NEGF     AC0           1.22   1.83    4.8    5.4    15.83

ABSF     AC0           1.22   1.83    5.2    5.9    15.99

LDF       R0, AC0      1.22   1.82                  11.47

LDF     (R3), AC0      1.97   2.60                  14.67

LDCIF     R0, AC0      2.27   2.37                  29.14

LDCIF   (R3), AC0      2.27   2.59                  20.47

STCFI    AC0,  R0      2.73   3.36                  36.00

STCFI    AC0, (R3)     3.36   4.17                  37.59

SETD                   1.22   1.82    1.6    2.1     6.13

MULD     AC2, AC0      3.94   4.01   44.4   46.4   215.44

ADDD     AC0, AC1      2.72   2.83   19.2   20.4    98.83

NEGD     AC0           1.22   1.82    5.9    6.5    19.32

ABSD     AC0           1.22   1.82    6.2    7.0    19.29

LDF       #1, AC0      1.81   2.88    3.4    4.4    14.29

LDF   @#addr, AC2      2.72   3.49                  16.06

STF      AC0, @#addr   4.22   5.26    4.0    5.1    11.80

CMPF  @#addr, AC0      2.89   3.49    5.7    6.9    30.40

LDCIF     #1, AC0      2.61   2.89                  42.01

LDCIF @#addr, AC0      2.44   3.18                  25.20

STCFI    AC0, @#addr   3.41   4.89    9.9   11.1    38.24

The   results   of  these  tests  need  to  be  considered

carefully.  First, I had a limited amount of time on the 11/84,

and  ran the shortest tests:  it would be desirable to run them

again to obtain the same precision as for the 11/70 and PRO-350

tests.   The  times  for  the  MULtiply and DIVide instructions

don't always match the  processor  handbook  values:   this  is

probably  because the data values used are special cases, and a

divide by one is faster on  some  machines  than  a  divide  by

another  number.   The  values  for  the  short loop, where the

entire loop is in cache,  do  match  very  well  the  published

values in the 11/70 processor handbook.

It  is  apparent,  however, that the 11/84 certainly comes

very close to the speed  of  an  11/70.   For  some  addressing

modes,  the  11/84  is  faster:  when this is combined with the

larger cache, the net effect is that some other  test  programs

actually  ran  significantly  faster  on  the 11/84 than on the

11/70.  The two cases where the 11/84 is obviously  slower  are

PDP-11 CPU instruction timing tests                      Page 7

B. Z. Lederman

arithmetic  instructions  (MUL,  DIV,  ASHC) where the 11/84 is

slightly slower, and with floating  point  instructions,  where

the  11/84  is  considerably  slower.  How much this affects an

application depends upon how many floating  point  instructions

are  executed:   even  a  program with floating point variables

generally has a relatively low percentage of the total  program

executing  actual  floating  point arithmetic.  A program which

did use floating point heavily (large  matrix  inversion,  fast

Fourier  transforms,  network  analysis, and similar scientific

and engineering applications) would see a reduction in speed on

an 11/84.

The  story  does not end here, however.  DEC has announced

an upgraded 11/84A, whose processor  is  supposed  to  run  20%

faster  than  the  machine  I tested, so all of the instruction

times given above can probably be reduced by 20%:   this  would

make  the  11/84 faster than the 11/70 in nearly all instances.

This  machine  will   also   accommodate   a   floating   point

co-processor,   which   is   said  to  execute  floating  point

instructions 5 to 8 times faster than the J-11 CPU:  this would

narrow  or  eliminate  the gap between 11/70 and 11/84 floating

point performance.

The times for the PRO-350 are close to but slightly slower

than the times published for an  11/23,  which  uses  the  same

processor.   The  PRO-350  CPU  has  to  do  extra work such as

updating the video screen and keeping the system clock working,

which may account for the difference.

The Effect of Cache Memory.

Both  the  11/70  and  the  11/84 have cache memory:  this

means that the speed at which instructions will execute depends

upon  the  CPU  having to fetch the instruction from fast cache

memory or relatively slower main memory.  To examine the effect

this  has, the length of the timing loop was varied from a very

small loop (which will fit  entirely  in  cache  and  therefore

execute  at  the maximum CPU speed), to a very large loop which

will not fit into cache, so that the instructions  will  mostly

be  fetched  from  main memory.  By using loops of intermediate

size,  one  can  see  the  effect  of  a  larger  cache.    The

instructions   are   divided  into  three  sets:   single  word

instructions, instructions followed by one word of address, and

instructions  followed by two words of address.  The numbers at

the top of the table give the number of  instructions  in  each

loop.   Though  the tests were run for all instructions, only a

few need be given here to show the effect of cache.

PDP-11 CPU instruction timing tests                      Page 8

B. Z. Lederman

PDP-11/84

Length of test loop

Instruction      100    250    500   1000    2500   5000 10000

ROR R0             .3    .3     .3     .3      .3     .4    .8

TST (R0)           .7    .8     .8     .8      .8    1.0   1.3

CMP (R0), (R3)    1.6   1.6    1.6    1.6     1.6    1.8   2.1

MOV -(SP), (SP)+  2.1   2.2    2.2    2.2     2.1    2.3   2.7

MULF AC2, AC0    15.4  15.6   15.7   15.8    15.8   16.1  16.3

MULD AC2, AC0    44.4  45.2   45.5   45.7    45.9   46.1  46.4

Instruction       100   320   400   500   800  1000  2000 10000

ASHC #1, R0       2.1   2.1   2.1   2.1   2.2   2.2   2.2   3.1

MOV @#A, R1       1.1   1.1   1.0   1.0   1.1   1.0   1.1   2.0

STCFI AC0, @#A    9.9  10.0  10.1  10.1  10.2  10.2  10.3  11.1

Instruction    100   125   200   320   400  500  800 1250  2500

MOV #1, @#A    1.9   1.9   1.9   1.9   1.9  1.9  1.9  1.9   3.2

ADD #1, @#B    2.1   2.1   2.1   2.2   2.2  2.2  2.6  2.2   3.5

PDP-11/70, Core Memory

Instruction       100    250    500   1000    2500   5000 10000

ROR R0             .3     .3     .3     .4      .8     .9    .9

TST (R0)           .7     .8     .8     .7     1.2    1.3   1.3

CMP (R0), (R3)    1.0    1.1    1.1    1.4     1.6    1.7   1.7

MOV -(SP), (SP)+  1.7    1.7    1.7    2.0     2.5    2.5   2.5

MULF AC2, AC0     2.5    2.6    2.6    2.6     2.6    2.7   2.7

MULD AC2, AC0     3.8    3.9    4.0    4.0     3.9    3.9   3.9

Instruction       100   320   400   500   800  1000  2000 10000

ASHC #1, R0       1.3   1.4   1.4   1.4   2.2   2.2   2.4   2.6

MOV @#A, R1       1.2   1.4   1.5   1.7   2.1   2.2   2.3   2.4

STCFI AC0, @#A    3.3   3.4   3.4   3.4   3.6   3.6   3.6   3.7

Instruction    100   125   200   320   400  500  800 1250  2500

MOV #1, @#A    2.0   2.0   2.0   2.1   2.7  3.5  4.0  4.0   4.0

ADD #1, @#B    2.3   2.3   2.5   3.1   3.5  3.9  4.1  4.1   4.2

Though the tests showed the effects of the larger cache on

the 11/84, the transition point was shifted  more  twords  long

loops  than I had anticipated.  If I had the opportunity to run

the tests again I would use a longer loop (31,250  instructions

for  the  first  test, 15,625 for the second and 10,000 for the

third) to insure obtaining  the  maximum  execution  times.   I

PDP-11 CPU instruction timing tests                      Page 9

B. Z. Lederman

would   probably  also  run  the  tests  for  10,000,000  total

instructions rather than the 1,000,000 used for the above tests

to  obtain one more significant digit of precision, and I would

not have to run as many tests between the shortest and  longest

loop  as  the  point  at which the effect of cache begins to be

lost can be determined sufficiently well from the above  tests.

It   is   interesting   to   see   what  happens  with  certain

instructions,  especially  floating  point:   the  instructions

which  take the most time to execute suffer least when the loop

is long as the time needed to fetch the instruction from memory

is  only  a small part of the total instruction execution time.

The fastest instructions suffer most, as here the  time  needed

to  fetch  the instruction can be as much as or longer than the

time the CPU needs to actually do the instruction once  it  has

it.