Cabletron Systems Inc.

ESNMP2.DOC

This SNMP directory should contain the following file(s):

SNMP\OS2>

ESNMP2.OS2         01-08-93     VERSION 1.5

ESTAT2.EXE         01-08-93     VERSION 1.6

MCONFIG2.EXE       01-08-93     VERSION 2.20

The following files are needed to run the SNMP Management software

on a PC client under OS/2:

ESNMP2.OS2     Cabletron SNMP Management TSR

ESTAT2.EXE     Cabletron Local Management

MCONFIG2.EXE   Configuration utility for SNMP

** NOTE:  ESNMP2.OS2 WILL ONLY RUN ON DRIVERS THAT SUPPORT SNMP

(ie. E30ODIM.SYS, E31ODIM.SYS, E30NDM.OS2, E31NDM.OS2).

A) CONFIGURATION & INSTALLATION OF THE MANAGEMENT:

ESNMP2.OS2 and MCONFIG2.EXE are needed to install and configure

Cabletron SNMP Management. The following steps are needed to

install the management:

1) Make directory SNMP on the drive you want to copy the management

files to.

2) Copy esnmp2.os2 and mconfig2.exe to the directory.

3) Run mconfig2.exe to configure the IP Address and other management

parameters. The following values can be changed

COMMUNITY NAME=     (up to 16 ASCII characters)

Community name for the DNI Card SNMP Agent.

This defaults to 'public'.  Only SNMP Requests

which match the configured COMMUNITY name will

be processed by the DNI Card SNMP Agent.

NAME=               (up to 32 ASCII characters)

This should be a meaningful and unique

name used to identify the PC SNMP Agent

on the Network.

Default: 'PC Name'

OWNER=              (up to 32 ASCII characters)

User name.

LOCATION=           (up to 32 ASCII characters)

Location of the PC SNMP Agent.

APPLICATION=        (up to 32 ASCII characters)

Name of the application.

COMMENT=            (up to 32 ASCII characters)

Miscellaneous comment field.

CONTACT=            (up to 32 ASCII characters)

Name of the contact person for this node.

MMAC ADDRESS=       (in the form xx-xx-xx-xx-xx-xx)

Ethernet address of the MMAC the PC is

connected to.(specified in HEX)

MMAC BOARD=         (1 decimal digit, range 0 - 8)

Board number on the MMAC to which the PC

is connected to.

MMAC PORT=          (2 decimal digits, range 0 - 24)

Port number on the specified MMAC Board

to which the PC is connected to.

IP ADDRESS=         (in standard dot notation, i.e. 134.141.2.94)

IP address of the PC.  This must be specified

for the DNI Card SNMP Mgt agent to respond to

SNMP requests.

REMOTE IP ADDR1      (should be configured only to enable redundancy)

IP address of the remote host, path 1.

REMOTE IP ADDR2      (should be configured only to enable redundancy)

IP address of the remote host, path 2.

NOTE: Be sure to enter the proper COMMUNITY NAME and IP ADDRESS of your system.

4) Edit the config.sys file to add device=<drive>:\SNMP\esnmp2.os2 where

<drive> is the alphabet representing the drive having the files.

NOTE: The directive should be added after the "device" directive

for the network driver in config.sys file.

5) Reboot the system.

6) The management configuration can be changed anytime by invoking

the mconfig2.exe utility.

7) Redundancy is activated only for drivers supporting it. It can

be disabled by entering any of the remote addresses as 0.0.0.0 .

NOTES:

a) The system should be rebooted everytime the management configuration

is changed.

b) The user can install the software in any directory they want. The

config.sys file should have the correct path in the device directive

in that case.

B) USING LOCAL MANAGEMENT (ESTAT2.EXE):

1)  OPTIONS:

estat2 /c                    Continues display of statistics

estat2 /s                    Displays SNMP statistics only

estat2                       Displays ethernet statistics

2) INSTALLATION:

The local management can be invoked by typing ESTAT2 <OPTION> at

the prompt. It can also be used to find out

a) Whether the packet driver is loaded ?

b) Whether management driver is loaded ?

c) To check the management configuration.

*** END OF ESNMP2.DOC FILE ***