TO GET A SYSTEM DUMP

0) Make sure the current rk pack has a swap area on it.  If not you will

destroy the file system on it.

1) sync if you can

2) halt

3) la 044

4) start

5) wait a few seconds -- till you hear the effects of a reset

6) reboot the system

7) do a checks if a bad crash

TO ANALYIZE A DUMP

!) chdir /usr/lpd/dumps

2) crashola -- this will copy the dump to a file named after the current date

and time and link sysdump to it.  It will also write a synopsis

of the dump to the line printer.

3) xps axlk -- to get a ps at death.  This is an old version of ps which works

on sysdumps.

4) crash -u <u addr> -- to get furthur info on a process

5) crash -c /dev/mem -s /unix -u <u addr> to look at a living process