*******************************************************************************

*****                                                                     *****

*****                              QSEND.DOC                              *****

*****                              Version 10                             *****

*****        Program Notes for the QSEND Message Sending Program          *****

*****                                                                     *****

*****      Copyright (c) 1987                                             *****

*****      David M. Krowitz                                               *****

*****      Massachusetts Institute of Technology                          *****

*****      Department of Earth, Atmosheric, and Planetary Sciences        *****

*******************************************************************************

Description of the QSEND Message Sending Package

------------------------------------------------

The QSEND package consists of two programs a message-sending

program (QSEND) which can send a message to any logged in user or

node on the Apollo Domain network and a message-receiving program

(QSEND_SERVER) which waits for incoming messages and displays

them in a window which it pop up on the node. The programs are

based on a set of network information routines which look up

the current list of catalogged nodes and users logged into the

display manager. These routines are, in turn, based upon invoking

the LCNODE and LUSR commands to get the necessary network

status information. As a result, QSEND is very flexible. It does

not rely on any central node in the network to act as a server,

it does not require a predetermined list of available nodes

and/or users on the network, and it does not require any specific

disk volume in the network to be mounted. Each node relies only

upon its own MBX_HELPER, its own QSEND_SERVER, and a mailbox in

its own NODE_DATA directory to receive messages. The node is

totally independent of any other node in the network. Nodes can

be shut down and restarted, diskless nodes can change partners,

users can logout and login again on another node and QSEND will

still be able to get the message to the desired recipient.

The one drawback to this method is that on large Apollo

networks the LCNODE and LUSR commands can take a long time to

return the network status information which QSEND needs in order

to determine where to find a particular user and where a node's

entry directory is located (for diskless nodes). On very large

networks this could limit the usefulness of the program. The

network info routines have been kept seperate from the main

body of the QSEND and QSEND_SERVER programs so that if a better

means of gathering up to date network status information becomes

available the QSEND package can be modified to take advantage

of it.

Using QSEND

-----------

To send a message to up to 64 users and/or nodes, simply type

the command:

QSEND recipient-1 recipient-2 ... recipient-64

(eg. qsend john mary //node1 george@node2 )

QSEND will then ask you for a one line subject of the message

after which you can enter as many lines of text as you wish,

ending the message with a control-Z (the end-of-file character.

Note that if you have logged into an Apollo node from another

machine with TELNET that the end-of-file character may be

something other than a control-Z. In particular, if you have

logged into an Apollo node with TELNET from a DSP9000 (an

Alliant FX/1 or FX/8) you will need to type a control-D

instead of a control-Z). QSEND will then determine where

each of the recipients is logged in (if anywhere) and

determine where to find the mailboxes of the QSEND_SERVER's

running on those nodes. If you do not list any recipients

on the command line, QSEND will prompt you for a list before

asking for the subject line. QSEND recognizes six different

formats of recipients names which are listed below along

with four different switches. The switches may be entered

either on the command line along with the recipient names

or when the program prompts you for a list of recipients.

The name formats which are recognized are:

USER            (eg. 'john')        Sends a copy of the message

to every node on which

john.%.%.% is logged into

the display manager.

USER@NODE       (eg. 'john@bert')   Sends one copy of the message

to the node //BERT only if

john.%.%.% is logged in there.

USER@NODE_ID    (eg. 'john@3115')   Sends one copy of the message

to node 3115 only if john.%.%.%

is logged in there.

@NODE           (eg. '@bert')       Sends one copy of the message

to the node //BERT regardless

of whether or not anyone is

logged in on the node.

@NODE_ID        (eg. '@3115')       Sends one copy of the message

to node 3115 regardless of

whether or not anyone is

logged in on the node.

//NODE          (eg. '//bert')      Sends one copy of the message

to the node //BERT regardless

of whether or not anyone is

logged in on the node.

The switches which are recognized are:

-ME         Sends a copy of the message to each node on which

the owner of the process is logged into. Useful for

background jobs notifying their owner that they

have completed. Note that it sends to the owner of

the process which is running the QSEND program, which

may be different from the person who is logged into

the display manager of the node (they could have

CRP'd onto the node from another node).

-NODE       Sends one copy of the message to the node on which

the QSEND program is running. Useful for background

jobs notifying the user of the node of some

condition (eg. 'the print server on this node has

died, please start a new one with the CPS command').

Also good for displaying the message of the day

when someone is logging into the node. (eg. in

the node's STARTUP_LOGIN file have a line like

IF EXISTF MESSAGE_FILE THEN QSEND -NODE ...).

-ALLUSERS   Sends a copy of the message to every node in the

network which has someone logged into the display

manager. Useful for system-wide notices (eg. 'Everyone

please logout -- diskless node server going down

in 5 minutes -- your node will crash unless it is

shutdown').

-ALLNODES   Sends a copy of the message to every node in the

network regardless of whether or not anyone is logged

in on that node. Useful for system wide messages

which you want people to see before they try to

login (eg. 'the 500 Mbyte disk is dead, it will not

be up again until tomorrow ...').

-FILE       Uses the contents of the specified file as the main

body of the message to be sent. Useful for sending

precanned messages (eg. a message of the day).

-SUBJECT    Uses the specified text string for the text of the

SUBJECT line of the message. If the string contains

spaces, it must be enclosed in single or double quotes.

Also useful for sending precanned messages (eg. a

message of the day).

-DEBUG      Prints out debugging info while the program is processing

the message and trying to find the mailbox of the server.

Note that the -ME, -NODE, -ALLUSERS, and -ALLNODES switches are

recognized if they appear either on the command line (eg. qsend -me) or

if they are given in response to the TO: prompt (if there were no

recipients given on the command line). The -FILE, -SUBJECT, and -DEBUG

switches are only recognized if they occur on the command line.

Running the QSEND_SERVER

------------------------

The QSEND_SERVER program must be running on a node in order for

it to receive and display messages. QSEND will give a warning to the

user if they try to send a message to a node which does not have a

server running. You can run the QSEND_SERVER program in a regular

window by typing the command QSEND_SERVER, and it will print out

some diagnostic messages as it receives and displays messages.

Alternatively, you can run the program as a server process (so

that it does not go away when you logout) by using the CPS command

to create a process with server status to run the QSEND_SERVER

program (ie. CPS /COM/QSEND_SERVER -N QSEND_SERVER). Once the

server is running it will create a mailbox in the node's `NODE_DATA

directory after which it is ready to receive messages. The

server can process up to four incoming messages simultaneously.

It can display much more than four messages on the screen at one

time, the limit is on the number of messages it can be in the

process of receiving at the same time.

The QSEND_SERVER will place each message that it receives

on the screen in a slightly different position so that the

message windows do not completely obscure each other. Normally,

the window is made large enough to hold the complete message,

but in some cases the message may be too long to fit on the

screen. In this case the window will be made as large as is

possible and the pad-scrolling keys can be used to look at

the rest of the message. The message windows can be deleted

by simply placing the cursor in the window and typing a

control-N or hitting the 'exit' key on the keyboard.

Differences Between Program Versions of QSEND

---------------------------------------------

Version 7

---------

First released version. Handles up to 64 different user names, node

name/ID's, and switches in the TO: line. Does not read names/nodes/switches

on the command line yet.

Version 8

---------

Same as version 7 except now reads names/nodes/switches from the

command line. If no recipients given on the command line, then QSEND

will give the TO: prompt for recipients before asking for the subject

of the message.

Version 9

---------

Added the -FILE and -SUBJECT switches to make it easier to use QSEND

for sending a message of the day and for doing bulletin board messages.

Also added time and date that the message was sent to the message header.

Version 10

----------

Added the -DEBUG switches to make it easier to find problems with

QSEND's method of finding the mailbox of the server program. Also

beefed up the network info routines so that they can recognize diskless

nodes which have been catalogged on the network.

(eg. //kanga  *** DISKLESS ***  partner node: 4892)

Differences Between Program Versions of QSEND_SERVER

----------------------------------------------------

Version 7

---------

First released version. Handles up to 4 incoming messages simultaneously.

Can display an unlimited number of messages on the screen, limit of 4 is

only on the number of messages that are in the process of being received

at once.

Version 8

---------

Made minor change in way the number of lines in a message are counted

to allow the -FILE option of QSEND to work correctly. Also added a line

in the message header giving the date and time the message was sent.

Files Needed for the QSEND Package

----------------------------------

The files which are provided for the QSEND package are:

QSEND.DOC                   - This file.

QSEND.DOC.INSTALL           - Documentation on how to install the

QSEND package on your network.

QSEND.HLP                   - Help file for using QSEND.

QSEND.PAS                   - The source code for the QSEND

message sending program.

QSEND_SERVER.PAS            - The source code for the QSEND message

message receiving and display program.

QSEND.INS.PAS               - Insert file defining common data

structures for QSEND and QSEND_SERVER.

NETWORK.PAS                 - Library of subroutines to get info

about the nodes and users on the

Domain network.

NETWORK.INS.PAS             - Insert file defining data type and

procedure calls for NETWORK library.

QSEND.BLD                   - Command file to compile and bind the

QSEND program.

QSEND_SERVER.BLD            - Command file to compile and bind the

QSEND_SERVER program.

QSEND                       - A ready to run copy of the QSEND

program (in case you don't have a

Pascal compiler).

QSEND_SERVER                - A ready to run copy of the QSEND_SERVER

program (in case you don't have a

Pascal compiler).

You will also need the following standard Apollo-supplied files:

/SYS/INS/BASE.INS.PAS       - These are all standard insert files which

/SYS/INS/CAL.INS.PAS          are used by QSEND, QSEND_SERVER, or the

/SYS/INS/GPR.INS.PAS          NETWORK library.

/SYS/INS/MBX.INS.PAS

/SYS/INS/NAME.INS.PAS

/SYS/INS/PAD.INS.PAS

/SYS/INS/PFM.INS.PAS

/SYS/INS/PGM.INS.PAS

/SYS/INS/PM.INS.PAS

/SYS/INS/STREAMS.INS.PAS

/SYS/INS/TIME.INS.PAS

/SYS/INS/TONE.INS.PAS

/SYS/INS/VFMT.INS.PAS

Bugs, Questions, and Improvements

---------------------------------

If you find a bugs in the QSEND package, have questions on how to

install or use it, or have a good idea for improving the program please

feel free to contact me at the address below.

David M. Krowitz

MIT dept. of Earth, Atmospheric, and Planetary Sciences

Room 54-527

Cambridge, MA 02139

(617) 253-6180

network mailing address:

mit-erl!mit-kermit!krowitz@eddie.mit.edu

mit-erl!mit-kermit!krowitz@mit-eddie.arpa

david@mit-mc.arpa

(in order of decreasing preference)