ETHER(4)            Xinu Programmer's Manual             ETHER(4)

NAME

ether - standard 10Mbps Ethernet network interface device

driver (type eth)

SYNOPSIS

#include <deqna.h>

#include <ether.h>

read(device buffer, length)

write(device, buffer, length)

init(device)

DESCRIPTION

The _#e_#t_#h device driver provides input and output for a 10

Mbps Ethernet local area network.  The standard Xinu device

name for Ethernet devices is _#E_#T_#H_#E_#R.

Implemented on a Digital Equipment Corporation DEQNA (Digi-

tal Equipment Q-Bus Network Adapter), the _#e_#t_#h driver handles

DMA input and output at the physical network level,

transmitting complete frames between user processes and the

device.

The driver honors the following operations:

read(device buffer, length)

Reads one frame into _#b_#u_#f_#f_#e_#r, copying no more than

_#l_#e_#n_#g_#t_#h characters.  It returns the length of the frame

read; the count includes the 14-octet header.

write(device, buffer, length)

Writes a frame of _#l_#e_#n_#g_#t_#h characters found in _#b_#u_#f_#f_#e_#r.

It returns _#O_#K if the frame was acceptable, _#S_#Y_#S_#E_#R_#R oth-

erwise.  It it important to note that _#b_#u_#f_#f_#e_#r must

specify the address of a valid buffer allocated from

GETBUF(2), and that the driver does not copy the con-

tents of the buffer.  The call to _#w_#r_#i_#t_#e will return

once output has been started, but the buffer will be in

use until the device finishes.  The driver calls

FREEBUF(2) to dispose of the buffer once the device

finishes using it.

init(device)

Initializes the device and driver, including DEQNA

setup packet processing.  The driver sets the device to

accept only those packets addressed directly to the

devices' physical address and broadcast packets (all

1's address).

NOTES

Although the DEQNA supports chaining back-to-back operations

on both input and output, the driver forces a context switch

Version 6b               Printed 1/12/87                        1

ETHER(4)            Xinu Programmer's Manual             ETHER(4)

and a new call from READ(2) or WRITE(2) to restart I/O, and

depends on the hardware to buffer packets from the Ethernet.

Version 6b               Printed 1/12/87                        2