Release Notes - SMCPWR NDIS 4.0 and 3.x drivers

Version v4.15

06-Jan-1997

This kit includes Release v4.15 of the SMCPWR NDIS 4.0 and 3.x drivers.

Changes from previous release:

==============================

- Fix link interrupt masking on 100Mbps force mode after software reset.

- Fix an Autonegotiation timing problem between 21143/QSI based adapter in one

machine and 21x4/National in the other, after software reset.

- Fix SROM V4.0 detection.

- Disable the HEARTBEAT counter in 21143/QSI based adapters.

History:

========

Release V4.14:

- This release kit contains two drivers that use Microsoft's NDIS3

wrapper for Windows NT 4.0 and the OEM Service Release 2 (OSR2) of

Windows 95.  These drivers are located in subdirectories named

\WNT40\NDIS31 and \W95_osr2\NDIS31.  Although these drivers are fully

functional, they should not be used for Microsoft certification.  We

recommend that certification be performed using the drivers that use

Microsoft's NDIS3 wrapper.  For Windows 95, use the driver in \WIN95

or W95_osr2\NDIS31 and for Windows NT 4.0 use the driver in \WNT40\NDIS31.

- Fix a Nway problem on the old 21142/ML6692 cardubs design

- Fix Full duplex issue which caused failures on test suite

and performance drop.

- Mask interrupts on different Link statuses to improve performance.

- Change the WFW driver to work with one string for 21140 and 21142/3

adapters in the inf file.

Release V4.13:

- Final release (full NWAY support) DC21143-ML6692 board.

- Support Unlimited Burst Length option.

- First release of ALPHA drivers for SMC.

Release V4.12:

- Fix a DC21143-BroadcomT4 problem.

Release V4.11:

- Support SROM format v4.00.

- Fix the full_mac sources directory.

Release V4.1:

- Final release for the 21143+QSI 6611 boards.

Release X4.1-004:

- Support SMC's FDX LED

- Fixed a problem in force AUI.

- Fixed a problem in force 100-FDX with 21143+QSI

Note: The 21143+Microlinear MII Phy, works OK in the modes

10BaseT Force, 100BaseTx Force, and in NWAY mode succeeds

to do parallel detection to 10BaseT and 100BaseTx media.

The NWAY protocol doesn't work.

The 21143+QSI Phy works well in all the mediums, although

a number of LinkChange interrupts were seen in some revisions

of the QSI Phy.

Release X4.1-003:

- Add support for the DC21143 controller.

- Add support for PCI Memory_Read_Multiple command

- Fix the Adapter's deallocation resource routine

- Fix the No_Carrier and Loss_of_Carrier bogus statuses

for MII + Full_Duplex mode

- Add dynamic Txm Threshold algorithm

Release 4.03:

- Modified detection of MII phy to work on boards

where Phy Addr pins are not pulled up/down correctly.

Release 4.02:

- Includes the Overflow fix for DC21140 and DC21142

- Add the Registry key InterruptMitigation to enable

the interrupt mitigation algorithm

- Fix the oemsetup.inf file for DC21142

Release 4.01:

- Add support for the Microlinear ML6692 PHY adapter.

- Add speed sensing workaround for National's

DP8340 PHY adapter.

- Modify the handling of Mii register's read failure

in the MiiPhyRegister routine.

- Add the dynamic setting of StoreAndForward transmit

mode if the number of underruns reachs the

Underrun threshold (default=10, programmable through the

Registry's key "UnderrunThreshold").

- Modify the support for Nway media autonegotiation

for the DC21041 Rev2.0 and DC21142 Rev 1.x.

- Add support for the DC21142 controller.

- Store the driver's revision information into

Windows95' Registry and WfW's protocol.ini file:

WNT's Registry:

HKEY_LOCAL_MACHINE\

Software\DigitalEquipmentCorporation\DC21X4\CurrentVersion

MajorVersion

MinorVersion

Win95's Registry:

HKEY_LOCAL_MACHINE\

System\CurrentControlSet\Services\Class\Net\"num"\NDIS

MajorDriverVersion

MinorDriverVersion

WfW's protocol.ini :

[DEC$DC21xxx]

MajorDriverVersion

MinorDriverVersion

Release 3.1:

- Single media adapter with no link indication supported

by the Autosense option

- Remove the pageable_function attribute of the 'DC21X4MediaDetect' and

'DC2114Sense100BaseTxLink' routines to allow their run_time invocations

by the Reset handler.

- Add MII management support for MII's 100baseTx and

100baseT4 PHY adapters

Release 3.0:

- Add Nway support for DC21041

- Separate transmit threshold values for 10Base and 100Base media

Both transmit threshold  values are programmable through the Registry:

- "TxmThreshold" for 10base media

- "TxmThreshold100" for 100base media (meaningless in

StoreAndForward mode)

- Fix unaligned pointers

- Updated Registry ConnectionType:

+-----+------------------+------------------------------------------+

|(dec)| Connection Type  |                                          |

+-----+------------------+------------------------------------------+

|  0  | AutoSense        |  AutoSense                               |

+-----+------------------+------------------------------------------+

|  1  | 10Base2          |  BNC,Thinwire,Coax                       |

+-----+------------------+------------------------------------------+

|  2  | 10BaseT          |  Twisted_Pair Half_Duplex                |

+-----+------------------+------------------------------------------+

|  3  | 10BaseT,FD       |  Twisted_Pair Full_Duplex                |

+-----+------------------+------------------------------------------+

|  4  | 10BaseT,NLT      |  Twisted_Pair Half_Duplex,No_Link_Test   |

+-----+------------------+------------------------------------------+

|  5  | 10Base5          |  AUI,Thickwire                           |

+-----+------------------+------------------------------------------+

|  6  | AutoSense,No_Nway|  AutoSense,Nway Disabled                 |

+-----+------------------+------------------------------------------+

|  7  |     -            |  Reserved                                |

+-----+------------------+------------------------------------------+

|  8  | 100BaseTx        |  100BaseTx(Symbol_Scrambler),Half_Duplex |

+-----+------------------+------------------------------------------+

|  9  | 100BaseTx,FD     |  100BaseTx(Symbol_Scrambler),Full_Duplex |

+-----+------------------+------------------------------------------+

| 10  | 100BaseT4        |  100BaseT4                               |

+-----+------------------+------------------------------------------+

Miniport driver only:

- Add Data copy to preallocated buffer instead of

Physical Mapping for Transmit Packets smaller or equal

to 256 bytes.

- Add Interrupt mitigation algorithm:

Interrupt/sec and Frames/sec threshold are programmable

through the Registry's keys "InterruptThreshold" & "FrameThreshold"

- Underrun retransmission. In case of underrun packets are

automaticaly retransmitted by the driver. The maximun number of

retransmission is programmable through the Registry's key

"UnderrunRetry".

Release 2.44:

- Remove BurstLength=32 option for DCc1040 & Dc21041

Release 2.43:

- Add PowerPC miniport

Release 2.42:

- Fix the PCI Mechanism 1 detection for (OPTI platforms) in the

WfW driver

- Fix the SROM parsing for 10BaseT Full Duplex mode

- Fix the DC21040's autodetect hang when the adapter is

unconnected

- Fix the DC2141's Autosense routine

Release 2.41:

- Enhance the DC21140 AutoDetection and AutoSense

routines.

- Add support for DC21140's 100BaseT4 and

100BaseFx media.

- Add support for PCI to PCI bridge in Windows for Workgroups

driver

Release 2.31:

- Number of map registers allocated by the adapter

configurable through the Registry

- Add missing NdisMCancelTimer in the Halt handler

- Add support for SMC 8432e adapter

Release 2.3:

- Fix DC21040 and DC21041's Full Duplex mode

Release 2.2:

- Fix the hash filter generation

- Add optionnal Txm FIFO Threshold values

for 100Mbps in the Registry

- Miniport driver fixes for WIN95

- Add Snooze mode option for Digital Semiconductor's21041

Release 2.1:

- Modify the 10/100 link autosensing algorithm to avoid

switching media while autosensing for link pass:

With the new algorithm the media is switched only if:

- the selected media is 10 Mbps and the 100Mbps link passes:

media is switched from 10Mbps to 100Mbps.

- the selected media is 100Mbps. Its link fails but

the 10 Mbps link passes: the media is switched from

100 Mbps to 10Mbps.

- Modify the DataRate values in the driver's installation file

to be backward compatible with the DC21040 V1.x values:

Release 2.0:

- Miniport driver for WNT

- Add support for Serial ROM format V1

- Add support for Digital Semiconductor's 21041

- Add dynamic autosense for Digital Semiconductor's 210140 & 21041

- Increase the minimal wait time for BNC

transceiver's setup to 300 milliseconds

- Fix Promiscuous filter setting

- Add support for SMC 8434

Release 1.2:

- Added workaround for Intel Host to PCI bridge

in WfW driver

- TransceiverDelay in 100Millisecond units

Release 1.1:

- Enhanced interrupt service routine.

- Support Digital Semiconductor's 21140 requirements for size of Multicast address table.

- Fix loopback routine

- Replace Fast Ethernet adapter name to 9332.

Windows 95:

- First release

Release 1.0:

- Added support for SMC 8932 Fast Ethernet Adapter.

- Added static keyword to OEMSETUP.INF: AdapterCFID