STANDARD MICROSYSTEMS CORPORATION

Microsoft Windows NT v3.5 and v3.51 NDIS v3.0 Driver

For Ether 10/100 Fast Ethernet EISA Adapters

This software is licensed by SMC for use by its customers only.

Copyright (c) 1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Preinstallation hardware and software requirements

o  Supplied files

o  How to use EZStart to copy the Microsoft Windows NT version 3.51

NDIS spec 3.0 driver from the SuperDisk to your PC.

o  How to install the driver.

NOTE: Ensure that the line speed (10 Mbps vs. 100 Mbps) configured

for the adapter matches the speed of the hub to which it is

attached. It is important to set the adapter at the correct

line speed (10/100 Mbps) using the EISA Configuration Utility.

Refer to pages 3-3 and A-9 in the user guide for further details.

PREINSTALLATION HARDWARE AND SOFTWARE REQUIREMENTS

Hardware Requirements:

- An IBM-compatible personal computer with EISA bus

- An Ether 10/100 Fast Ethernet EISA adapter for that bus.

Software Requirements:

- Windows NT Version 3.5 or later

SUPPLIED FILES

General files

o  INSTALL.DOC      This file

o  RELEASE.DOC      Release notes for this version of the driver

Files to install the driver on Windows NT

o  SMC9232N.SYS      The driver

o  OEMSETUP.INF      The driver's setup file

COPYING THE DRIVER FILES USING EZSTART

The EZStart default directory is a:\.  Please make a backup copy of

your SuperDisk before EZStart copies de-compressed driver files to the

SuperDisk to create an "option" diskette.

1. Boot the computer to the DOS prompt.

2. Insert the SuperDisk into a floppy disk drive.

3. Change to the floppy drive where the SuperDisk is installed.

For example, if the disk is in drive A, type "A:" and press Enter.

4. From the DOS prompt, type "EZStart" (or "EZStart /t" for text

mode) and press Enter.

5. Select "Custom" from the EZStart main screen.

6. Select "Drivers/Documents" from the Custom Installation screen.

7. At the Network Drivers and Documentation Selection screen,

select the appropriate driver you want to install (make sure the

selection you want is highlighted).  Then press the Enter key or

select "OK".

8. At the Network Drivers and Documentation screen, the default

destination directory and a list of files to be copied to the

default directory are displayed.  To accept the default

destination directory select "Copy Files".  Otherwise, enter a

destination directory at the prompt and select "Copy Files".

9. Follow the prompts to continue the installation.

10. Select "Exit" to exit EZStart.

Note:  If you change the Destination Directory, make sure that the

DISK.ID file is in the root directory of the disk that you

are using, and that the OEMSETUP.INF and SMC9232N.SYS files

reside in the root directory or a subdirectory on the disk

you are using.

INSTALLING THE DRIVER

1. Use EZStart to test and configure your Ethernet adapter. Make

sure you write down your Ethernet adapter IRQ, I/O Base, and

RAM Base settings.

2. Boot your computer from Windows NT.

3. From the Program Manager's "Main" window, double-click on the

"Control Panel" icon.

4. Double-click on "Network". The Network Settings dialog box

appears.

5. Select "Add Adapter".

6. Select "<Other> Requires disk from manufacturer".

7. Select "Continue".

8. When prompted, insert the SuperDisk.

9. Type the Destination Directory where EZStart copied the

driver files (for example, A:\WIN_NT). If you are not

sure where the Destination Directory resides, look for

the SMC9232N.SYS file. This file was one of the files

copied by EZStart to the Destination Directory. When you

find the directory where this file resides, you will have

found the Destination Directory.

10. Select "OK".

11. When the next dialog box appears, select the SMC Ethernet

adapter followed by "OK".

12. Choose the IRQ level, memory base address, and I/O port

address. Refer to the information you wrote down in step 1.

13. When the next two dialog boxes appear, select "OK" for

each one.

14. Shutdown and reboot your computer.

In the unlikely event you encounter any device driver errors

after Windows NT boots, check the event log in the

Administrative Tools icon to see whether the error is related

to your network adapter.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and

Ether 10/100, EZStart, and SuperDisk are trademarks of Standard

Microsystems Corporation. Other products and company names are

registered trademarks or trademarks of their respective companies.